3GPP TSG-RAN WG2 Meeting #107	                                                  R2-1911618

Prague, Czech Republic, 26th - 30th August 2019

Agenda item:		11.11.2

Source:			Asia Pacific Telecom co. Ltd

Title:		UE behavior on WUS occasion in DRX Active Time

Document for:	 	Discussion and decision

# Introduction

In last meeting, RAN2 agreed that WUS is used to indicate the UE to monitor PDCCH on the next occurrence of the DRX onDuration and without other impacts in DRX procedure apart from DRX onDuration. On the other hand, RAN2 briefly discussed an issue that WUS monitoring occasion may collide with legacy DRX Active Time in time domain [1]. The detailed UE behaviours of both issues are deferred to discuss in WI phase.

RAN2 #106 Agreement

1.	From RAN2 perspective, the WUS signalling is used to indicate to the UE to wake up to monitor the onDuration.  Apart from the onDuration, there are no DRX procedure impacts.

2.	FFS on UE behaviour when WUS collides with any event part of legacy active time (e.g. DRX Inactivity timer).  This will be discussed in the WI phase.

3.	Details of WUS design is left to RAN1

In this contribution, section 2.1 is to introduce the procedure of WUS in DRX and the corresponding UE behaviours. Section 2.2 is to address the issue that WUS monitoring occasion collides with legacy DRX Active Time, and some solutions are proposed for discussion.

# Discussion

## Procedure of WUS in DRX

According to RAN1 agreements, the WUS monitoring occasion is configured/indicated to the UE with an offset before the DRX onDuration. If WUS mechanism is configured for a UE, the UE should ramp up the power to monitor WUS on WUS monitoring occasion periodically, e.g., before each DRX onDuration, to receive the possible WUS indication. The simplest way to interpret the WUS indication is to indicate wake up or not wake up could be based on whether the UE detects the WUS on WUS occasion or not. No further field for WUS is needed to indicate wake up/not wake up. Also, the NW does not need to send a WUS to the UE while the NW is not willing to wake up the UE, which could significantly reduce signaling overhead.  That is, when the UE detects a WUS on WUS occasion, the UE should wake up to monitor the PDCCH on the subsequent DRX onDuration. On the contrary, when the UE does not detect the WUS, the UE may not be required to monitor the PDCCH on the subsequent DRX onDuration. From RAN2 perspective, the WUS scheme is considered jointly with DRX, which is handled by MAC entity. Thus, the MAC entity should get a “wake up” indication/information from lower layer if the WUS is detected. In DRX procedure, when the MAC entity gets the “wake up” indication from lower layer, the MAC entity will start the drx-onDurationTimer at the beginning of the next DRX cycle, otherwise, the MAC entity will not start the drx-onDurationTimer at the beginning of the next DRX cycle. The procedure of WUS in DRX is shown in Figure 1.

Proposal 1: When the UE monitors a WUS occasion and the WUS is detected, the UE will start the drx-onDurationTimer at the beginning of the subsequent DRX cycle.

Proposal 2: When the UE monitors a WUS occasion but the WUS is not detected, the UE will not start the drx-onDurationTimer at the beginning of the subsequent DRX cycle.

Figure 1: Procedure of WUS in DRX

## 2.2	WUS occasion collides with DRX Active Time

In current DRX operation, it is well known that the UE should monitor the PDCCH while it is in DRX Active Time. There are several timers (e.g., drx-InactivityTimer, etc.) and events (e.g., when SR is sent and is pending, etc.) depicted in MAC specification, which would trigger the UE to wake up to monitor the corresponding scheduling. In some situations, DRX Active Time may cover WUS monitoring occasion(s) in time domain. For instance, as shown in Example 1 of the Figure 2, UL data may arrive when the UE is not in DRX Active Time. In this time, the UE will trigger SR to request for the UL resource, and the UE will remain in DRX Active Time when the SR is sent and is pending. It is likely that the DRX Active Time overlaps with the WUS monitoring occasion. For another example as shown in Example 2 of the Figure 2, the drx-InactivityTimer would be started or restarted when receiving a PDCCH indicates a new transmission. Consequently, it is also possible that the drx-InactivityTimer is still running on the next WUS monitoring occasion(s).

Observation 1:  WUS monitoring occasion may collide with DRX Active Time in time domain

Figure 2: Examples of WUS monitoring occasion collides with DRX Active Time

However, it may be redundant that the UE monitors the WUS during DRX Active Time, since the UE already ramps up the power to monitor the PDCCH. It is noted that UE-specific configuration of the search space sets(s) dedicated to the WUS monitoring is supported. Consequently, monitoring the WUS in DRX Active Time would waste UE processing power due to additional blind decoding overhead, and increase unnecessary constraints on detection performance.

Observation 2:  Monitoring WUS in DRX Active Time is redundant since UE already keeps awake to monitor PDCCH

Observation 3:  Monitoring WUS in DRX Active Time introduces additional blind decoding overhand and increases unnecessary constraints on detection performance

Based on the above reasons, we think the UE is not required to monitor WUS occasion while the UE is in DRX Active Time, so we propose two options for the detailed UE behaviours in this situation (as illustrated in Figure 3).

- Option 1: UE does not monitor WUS occasion while the UE is in DRX Active Time and without further behaviour (i.e., not start the drx-onDurationTimer at the beginning of the subsequent DRX cycle) for this WUS occasion

- Option 2: UE does not monitor WUS occasion while the UE is in DRX Active Time and always considers as wake up (i.e., start the drx-onDurationTimer at the beginning of the subsequent DRX cycle) for this WUS occasion

For option 1, it means the UE will ignore the WUS during DRX Active Time, the UE only performs the legacy DRX operation except for the new behaviours of drx-onDurationTimer (i.e., the UE still applies the other Active Time activities, such as SR, RA, inactivity timer, retransmission timer, etc.)

For option 2, as long as WUS occasion(s) overlaps with DRX Active Time, the UE always considers as wake up (i.e. start the drx-onDurationTimer) on the next occurrence of the DRX onDuration. Thus, the UE applies the same behaviour as rel-15, i.e. the UE will always start the drx-onDurationTimer at the beginning of the subsequent DRX cycle, in this situation.

Figure 3: Illustration of option 1 and option 2

Proposal 3: UE does not monitor WUS occasion while the UE is in DRX Active Time

Proposal 4: RAN2 considers down-selection of the following two options for the corresponding UE behaviours on top of the proposal 3:

- Option 1: UE does not start the drx-onDurationTimer at the beginning of the subsequent DRX cycle if the UE does not monitor WUS on WUS occasion in DRX Active Time

- Option 2: UE always starts the drx-onDurationTimer at the beginning of the subsequent DRX cycle if the UE does not monitor WUS on WUS occasion in DRX Active Time

# Conclusion

To sum up, we propose the UE behaviours for WUS procedure in DRX mechanism and provide the solutions when the WUS occasion collides with DRX Active Time in time domain.

Proposal 1: When the UE monitors a WUS occasion and the WUS is detected, the UE will start the drx-onDurationTimer at the beginning of the subsequent DRX cycle.

Proposal 2: When the UE monitors a WUS occasion but the WUS is not detected, the UE will not start the drx-onDurationTimer at the beginning of the subsequent DRX cycle.

Observation 1:  WUS monitoring occasion may collide with DRX Active Time in time domain

Observation 2:  Monitoring WUS in DRX Active Time is redundant since UE already keeps awake to monitor PDCCH

Observation 3:  Monitoring WUS in DRX Active Time introduces additional blind decoding overhand and increases unnecessary constraints on detection performance

Proposal 3: UE does not monitor WUS occasion while the UE is in DRX Active Time

Proposal 4: RAN2 considers down-selection of the following two options for the corresponding UE behaviours on top of the proposal 3:

- Option 1: UE does not start the drx-onDurationTimer at the beginning of the subsequent DRX cycle if the UE does not monitor WUS on WUS occasion in DRX Active Time

- Option 2: UE always starts the drx-onDurationTimer at the beginning of the subsequent DRX cycle if the UE does not monitor WUS on WUS occasion in DRX Active Time

# Reference

[1] R2-1906425; RAN2 impacts when introducing the PDCCH-based WUS; Intel Corporation