3GPP TSG-RAN WG2 Meeting #107                                                              R2-1911720

Prague, Tcheque Republic, August 26th – 30th

Agenda item:	11.1.3

Source: 		Qualcomm Incorporated (Rapporteur)

Title: 	Offline 106: BAP configuration based on F1 to F7

Document for:	Discussion

# 1	Introduction

This is CB discussion:

Offline 106 (QC) on F1-F7 functions and the needed config information in light of the possible packet flows (Access IAB node, intermediate IAB node, Donor IAb node).

Here are F1-F7 for reference:

- The below lists the functions of BAP (initial, might not be complete) F1: Retrieve packets from ingress RLC layer F2: Deliver packets to egress RLC layer F3: Retrieve packets from upper layer F4: Deliver packets to upper layer F5: Differentiate traffic to be delivered to upper layers from traffic to be delivered to egress RLC layer F6: Perform bearer mapping and routing for packets delivered to egress RLC layer F7: Selection/addition of BAP identifiers for packets received from upper layer

Since the time for discussion is very short, a few observations are made. Companies are welcome to provide comments/feedback, provide alternative observations, etc.

# 2	Discussion

There are only three distinct manners a packet is processed by BAP:

1. The packet is:

- retrieved from ingress RLC layer (F1),

- differentiated to be delivered to upper layers (F5), and

- delivered to upper layers (F4).

This applies to upstream packets at the IAB-donor and to downstream packets at the IAB-node (“access IAB-node).

Observation 1: To differentiate traffic to be delivered to upper layers from traffic to be delivered to egress RLC layer, the IAB-node and IAB-donor need to be configured with a BAP-address.

2. The packet is:

- retrieved from ingress RLC layer (F1),

- differentiated to be delivered to egress RLC layer (F5),

- routed and mapped to an egress RLC channel (F6),

- delivered to RLC layer (F2).

This applies to upstream and downstream packets at the IAB-node (“intermediate IAB-node”).

Observation 2: For routing and bearer mapping of a packet retrieved from RLC layer, the IAB-node needs to be configured with the following mapping:

Ingress RLC channel, BAP routing ID  Egress link, egress RLC channel.

3. The packet is:

- retrieved from upper layer (F3),

- BAP identifiers are selected/added (F7),

- routed and mapped to an egress RLC channel (F6),

- delivered to RLC layer (F2).

This applies to downstream packets at the IAB-donor and to upstream packets at the IAB-node (“access IAB-node).

Observation 3: For the selection/addition of a BAP routing ID as well as routing and bearer mapping for a packet retrieved from upper layers, the IAB-node needs to be configured with the following mapping:

Upper layer information (FFS)  BAP Routing ID, egress link, egress RLC channel.

Questions:

- Can we agree on these observations?

- Are there other manners of packet processing not covered in these observations?

- Are there other BAP configurations not captured above?

# 3	Comments/feedback

| Company | Comment/feedback |
| --- | --- |
| Huawei | For observation 1: We agree with the intention. But we want to clarify that one essential function is missing, i.e. removing BAP header, for the packets to be delivered to upper layers. For observation 2 and 3: We have general comments on how we configure the BAP. We should have 3 separate configurations: 1 BAP header to be added; 2 routing table; 3 bearer mapping. Therefore, we should clarify the configuration of observation 2 and 3 as followings: Observation 2: For routing and bearer mapping of a packet retrieved from RLC layer, the IAB-node needs to be configured with the following mapping: BAP routing ID in BAP header Egress link (routing table); Ingress RLC channel egress RLC channel (bearer mapping) Observation 3: For the selection/addition of a BAP routing ID as well as routing and bearer mapping for a packet retrieved from upper layers, the IAB-node needs to be configured with the following mapping: Upper layer information (FFS) BAP Routing ID to be added in BAP header (BAP header to be added); BAP routing ID in BAP header Egress link (routing table); Upper layer information (FFS) egress RLC channel (bearer mapping); That means for any flow, it is sufficient once the BAP is configured with the above 3 configurations. As to the question part, “Are there other BAP configurations not captured above?”, We don’t need to exclude anything explicitly for now. |
| CATT | A few quick comments for clarification. 1. For observation 1, do you mean the bap address configured for the IAB node matching with the bap address contained in bap header? If so, the routing ID in the bap header needs to be configured in the first place? 2. In observation 2 it was mentioned in email discussions that this might require some previous hop IAB node identifier as well, so that the IAB node knows which ingress-egress channel mapping configuration to use. I guess this can be added to discussions as well. |
| LG | 1st question: we agree with the proposed 3 observations and maybe more clarification by Huawei above is helpful. 2nd question: there could be a BAP control PDU depending on other ongoing discussion. We may need other manner of packet processing not covered in these observations. 3rd question: considering that upper layer information is FFS, above BAP configurations are sufficient for now. |