3GPP TSG-RAN WG2 #107	R2-1911752

# Prague, Czech Republic, 26-30 August 2019

Agenda Item:	10.2.2

Source:	Nokia, Nokia Shanghai Bell (Rapporteur)

Title:	Report of Offline discussion #004

Document for:	Discussion

# Introduction

R2-1909961	Clarification on selectedBandCombinationNR	Nokia, Nokia Shanghai Bell	CR	Rel-15	37.340	15.6.0	0142	-	F	NR_newRAT-Core

-	Huawiei think this is network implementation and does not need to be captured

-	Ericsson also think it doesn't need to be explicitly captured in stage 2.

-	Nokia think the previous text was not clear and we need to differentiate the MN involved and not cases.

-	ZTE think the text is not wrong but it is also not essential. We could end up with an inifinite list of thinks to cover.

=>	Offline discussion to establish whether what is described in the CR uis a valid behaviour. (Offline discussion 04).

=>	We will not capture anything in the stage 2 spec.

# Discussion

In the above CR [1], the following behaviour was proposed to be captured as the interaction between MN and SN for coordination of band combination and feature set.

- For the SN initiated modification procedure requiring MN involvement, the SN will need to involve the MN when it chooses another MR-DC band combination or feature set within the allowed list of MR-DC band combinations. - For the SN initiated modification procedure not requiring MN involvement, the SN does not need to involve the MN when it chooses to add/remove SCell(s) or perform PSCell change when it retains the same selected MR-DC band combination and feature set.

Q1: Is the understanding described in the CR a valid behaviour? Please answer YES/NO and reason if NO.

| Company | Comments (YES/NO) |
| --- | --- |
| Nokia | Yes |
| Ericsson | Yes for 2 and no for 1 – our understanding is that once the MN and SN have coordinated a given band combination there would be no need to allow the SN to be able to dynamically change the MR-DC band combination or Feature Set. At least for the MN, we think it is not expected to have a frequent change on the selected band combination, otherwise, if this same SN principle is followed by the MN, it would result in unnecessary signalling/ coordination to keep changing its band combination and thus renegotiating the allowedBC-ListMRDC. |
| Huawei | Yes |
| DOCOMO | Apart from the necessity of renegotiation, according to the current specification, both 1 and 2 are possible in the network side. SgNB Modification Request conveys CG-Config in which SN can indicate its preferred band combination as selectedBandCombination, which is selected amongst allowedBC-ListMRDC. Of course, this requires SN to remember allowedBC-ListMRDC received when MR-DC is setup. Then, MN responds SgNB Modification Request including CG-ConfigInfo. The allowedBC-ListMRDC in CG-ConfigInfo could be the same as before or different. SN decides the SN part of the band combination amongst allowedBC-ListMRDC, no matter if the list includes SN preferred band combination or not. |

Based on the discussion in [2], the following text was discussed.

When attempting to add or modify an SgNB, the MN must include the configRestrictInfo with the allowedBC-ListMRDC. The SN must select an SCG configuration which is compatible with at least one of those allowed BC+FSs. It returns the SCG configuration (scg-CellGroupConfig, scg-RB-Config) as well as one selected BC+FS pair (selectedBandCombination). The field description of the BC+FS entries are as follows: allowedBC-ListMRDC A list of indices referring to band combinations in MR-DC capabilities from which SN is allowed to select the SCG band combination. Each entry refers to a band combination numbered according to supportedBandCombinationList in the UE-MRDC-Capability (in case of (NG)EN-DC or NE-DC) or UE-NR-Capability (in case of NR-DC) and the Feature Sets allowed for each band entry. All MR-DC band combinations indicated by this field comprise the MCG band combination, which is a superset of the MCG band(s) selected by MN.. selectedBandCombination Indicates the band combination selected by SN in (NG)EN-DC, NE-DC, and NR-DC.. What is not clear from those descriptions is which restrictions apply once the SN returned the selectedBandCombination. There could be two possible interpretations: Interpretation 1) selected BC+FS matters: Once the SN sends the CG-Config with the selectedBandCombination... - The SN may update its SCG configuration if the update is compatible with the “selected BC+FS” - The MN may choose any MCG configuration that is compatible with the “selected BC+FS” Interpretation 2) allowed BC+FSs matter: Once the SN sends the CG-Config with the selectedBandCombination... - The SN may update its SCG configuration if the update is compatible with at least one of the “allowed BC+FS”:s The MN may choose any MCG configuration that is compatible with all “allowed BC+FS” that it provided originally to the SN.

Q2: What is your company’s understanding of the above interpretations? Note that Interpretation 2 implies the understanding described by Bullet point 1 in Q1. Also note that Interpretation 1 assumes that the “allowedBC-ListMRDC” is only valid for the current transaction, i.e. the SN does not memorize it for further use.

| Company | Comments (Interpretation 1 or Interpretation 2) |
| --- | --- |
| Nokia | Interpretation 2 (the “allowedBC-ListMRDC” is stored by the SN and can move within or across band combinations and feature sets as discussed in [1]) |
| Ericsson | Interpretation 1, see comments for Q1. |
| Huawei | Neither. In our understanding, the SN can select only one “BC+FS” of the “allowed BC+FS” and returns it to the MN. The configuration of SN and MN should be compatible with this selected “BC+FS”. If the SN intends to have an update, the SN re-selects a new “BC+FS” from the “allowed BC+FS” the MN provides before. |
| DOCOMO | Interpretation 2 could be possible by network implementation as explained in Q1 |

# Conclusion

3 out of 4 companies agree that the following is valid network behaviour:

- For the SN initiated modification procedure requiring MN involvement, the SN will need to involve the MN when it chooses another MR-DC band combination or feature set within the allowed list of MR-DC band combinations.

- For the SN initiated modification procedure not requiring MN involvement, the SN does not need to involve the MN when it chooses to add/remove SCell(s) or perform PSCell change when it retains the same selected MR-DC band combination and feature set.

About the validity of allowedBC-ListMRDC, 3 out of 4 companies agree that Interpretation 2 is a valid network behaviour. However, only 1 company thinks that the SN can only freely move within the selected BC+FS and not across other BC’s+FS’s in the list.

Proposal 1:   RAN2 confirms that the following is a valid network behaviour.

- For the SN initiated modification procedure requiring MN involvement, the SN will need to involve the MN when it chooses another MR-DC band combination or feature set within the allowed list of MR-DC band combinations.

- For the SN initiated modification procedure not requiring MN involvement, the SN does not need to involve the MN when it chooses to add/remove SCell(s) or perform PSCell change when it retains the same selected MR-DC band combination and feature set.

Proposal 2: The choices in the allowedBC-ListMRDC remain valid for selection by the MN and SN  till the MN decides to update it.

# References

[1]	R2-1909961	Clarification on selectedBandCombinationNR	Nokia, Nokia Shanghai Bell	CR	Rel-15	37.340	15.6.0	0142	-	F	NR_newRAT-Core

[2] 	R2-1910054	Returning multiple BC+FS in CG-Config	Ericsson	discussion