3GPP TSG-RAN WG2 Meeting #107	R2-1911805

Prague, Czech Republic, 26th - 30th August 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4099 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NAS handling error of nas-Container for security key derivation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SA3 in TS 33.501 subclause 6.9.2.3.4 specifies that the UE shall verify the NAS COUNT and NAS MAC when receiving NASC (NAS Container) in the HO Command and process it only if the verification is successful. If the verification is not successful, NAS continues to use the old security information and hence the AS security keys derived by UE AS will be incorrect/not possible. RAN2 sent an LS to CT1 in R2-1905460 requesting feedback on NAS actions. CT1 in their LS response in R2-1908604 informed RAN2 that an indication was added from the NAS to the lower layers to inform about the invalid NAS container. CT1 CR is included in the LS response. CT1 also point out that the NAS will not trigger NAS signalling connection recovery in this case and CT1 expects RAN2 to attempt re-establishment to see if recovery is possible. The CR is proposed only for Rel-16 in line with the CT1 agreement that the NAS will not trigger NAS signalling connection recovery in this case. Also, since security failure of the NAS container in a handover command should be a rare event and Rel-15 is frozen, the CR was agreed only for Rel-16 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Indication from uppler layers of invalid NAS container is treated as RRC Reconfiguration failure with re-establishment as a possible recovery mechanism. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the UE receives an invalid nas-Container, the UE may apply wrong keys and be unable to continue the communication resulting in a failure of RRC connection. Impact analysis: Impacted architecture: SA Impact is isolated to RRCReconfiguration involving NAScontainer for security derivation. If the network is implemented in line with this CR and UE is not, there is no inter-operability issue since the CR only impacts UE. UE may apply wrong keys and be unable to continue the communication resulting in a failure of RRC connection. If the UE is implemented in line with this CR and network is not, there is no inter-operability issue. HO will fail and UE and network will fall back to previous (security) configuration after the re-establishment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.5, 5.4.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 5.3.5.5	Reconfiguration failure

The UE shall:

1>	if the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration message or if the upper layers indicates that the nas-Container is invalid:

2>	continue using the configuration used prior to the reception of RRCConnectionReconfiguration message;

2>	if security has not been activated:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause other;

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the connection reconfiguration procedure ends;

NOTE 1:	The UE may apply above failure handling also in case the RRCConnectionReconfiguration message causes a protocol error for which the generic error handling as defined in 5.7 specifies that the UE shall ignore the message.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/ failure.

NOTE 3:	The compliance also covers the NR configuration carried within octet strings e.g. field nr-SecondaryCellGroupConfig. I.e. the failure behaviour defined also applies in case the UE cannot comply with the NR configuration or with the combination of (parts of) the LTE and NR configurations.

**** Next Change ****

#### 5.4.2.4	Reconfiguration failure

The UE shall:

1>	if the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration message or if the upper layers indicates that the nas-Container is invalid:

2>	if the source RAT is E-UTRA:

3>	perform the actions as specified in 5.3.5.5;

2>	else:

3>	perform the actions defined for this failure case as defined in the specifications applicable for the other RAT;

NOTE 1:	The UE may apply above failure handling also in case the RRCConnectionReconfiguration message causes a protocol error for which the generic error handling as defined in 5.7 specifies that the UE shall ignore the message.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/ failure.