3GPP TSG RAN WG2#107	R2-1908620

Prague, Czech Republic, 26 – 30 August, 2019

3GPP TSG RAN WG1 #97	R1-1907939

Reno, USA, May 13th – 17th, 2019

Title:	LS on UE power saving

Response to:

Release:	Rel-16

Work Item:	NR_UE_pow_sav-Core

Source:	RAN1

To:	RAN2

Cc:	RAN4

# Contact Person:

# Name:	                Fang-Chen Cheng

E-mail Address:	 fcc@catt.cn

# 1. Overall Description:

RAN1 has discussed the design of the PDCCH-based power saving signal/channel for the triggering of UE adaptation to the power saving techniques.   RAN1 has reached the following agreements for the PDCCH-based power saving signal/channel outside Active Time,

Agreements:

A new RNTI (e.g., PS-RNTI) is introduced for the PDCCH-based power saving signal/channel decoding at least outside Active Time, UE-specifically configured

- FFS how to use the PS-RNTI for scrambling of the PDCCH-based power saving signal/channel

Agreements:

Support UE-specific configuration of the search space set(s) dedicated to the PDCCH-based power saving signal/channel for UE to monitor outside Active Time

- Following the principle of Rel-15 search space configuration

- FFS: the corresponding UE behaviour in monitoring the power saving signal/channel outside Active Time

- FFS whether UE can further monitor the search space set(s) inside Active Time

Agreements:

- The CORESET for the PDCCH-based power saving signal/channel outside Active Time can be configured to index to at least one of the CORESET(S) configured for other PDCCH monitoring

  - FFS whether the indexed CORESET can be exclusively used by the PDCCH-based power saving signal/channel (i.e., not be used for other PDCCH monitoring)

  - FFS whether or not to increase the number of CORESETs relative to that in Rel-15

  - FFS whether or not a BWP is dedicated for PDCCH-based power saving signal/channel

Agreements:

For power saving signal/channel configured outside Active Time, introduce a new DCI format for a UE, where the UE is configured to monitor the DCI format with the power saving information for the UE in the DCI configurable by RRC

- FFS whether the DCI is in UESS or CSS or both

- FFS detailed configuration of the power saving information

- FFS the new DCI format

- Note: the same DCI may carry power saving information for one or more UEs

# 2. Actions:

# To RAN WG2

# RAN1 kindly requests RAN2 to take the above information into account during in the UE power saving works.

# 3. Date of Next RAN1 Meetings:

TSG RAN WG1 Meeting #98	26 - 30 August 2019	Prague, Czech Republic

TSG RAN WG1 Meeting #98bis	14 - 18 October 2019	China