3GPP TSG RAN WG2#107	R2-1908641

Prague, Czech Republic, 26 – 30 August, 2019

3GPP TSG-RAN WG4 Meeting #91 	R4-1907730

Reno, USA, 13 – 17 May, 2019

Title:	LS on NR mobility enhancement

Response to:

Release:	Release 16

Work Item:	NR_Mob_enh-Core

Source:	TSG RAN WG4

To:	TSG RAN WG2, TSG RAN WG1

Contact Person:

#### Name:	Qiming Li

E-mail Address:	Qiming.li@intel.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 further discussed the feasibility of enhanced handover in RAN4 #91. After discussion the following agreements were made:

- Handover with simultaneous Tx/Rx with both source and target cells

  - From RAN4 perspective, handover from one frequency range to another is feasible.

  - Handover involving different SCS in source and target cells is feasible. However, all the existing UE capability regarding Rx/Tx with different SCS cannot be extended to cover this case. New UE capability needs to be introduced.

NOTE: it is up to RAN2 to decide if to support this.

- RACH-less handover

  - From RAN4 perspective, RACH-less handover with 0 or equal TA is feasible in FR2 if the source and target cells are well synchronized. But it is up to RAN2 to decide if the handover can be supported in this work item.

  - RAN4 is still discussing the feasibility of RACH-less handover with non-zero and non-equal TA.

  - Furthermore, it is preliminarily observed in RAN4 that if the source cell can configure multiple UL grant resources associated with different DL RS (SSB or CSI-RS) in target cell, handover successful rate can be increased. Note this also increase resource overhead in the target cell. However, the gain in handover successful rate over the resource overhead may need further study. RAN2 can decide if to support this.

2. Actions:

To RAN WG2 and RAN WG1:

ACTION: 	RAN WG4 respectfully asks RAN WG2 to consider above agreements in RAN4 in future discussion.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN4#92	26 - 30 Aug 2019	Ljubljana, Slovenia

3GPP RAN4#92bis	14 – 18 Oct 2019	China