3GPP TSG RAN WG2#107bis	R2-1912013

Chongqing, China, 14 – 18 October, 2019

3GPP TSG-RAN WG1 Meeting #98		R1-1909833

Prague, Czech Republic, 26 – 30 August 2019

Title:	Reply LS on MAC CE design for SCell BFR

Response to:	R2-1911795 (R1-1909813)

Release:	Release 16

Work Item:	NR_eMIMO-Core

Source:	TSG RAN WG1

To:	TSG RAN WG2

Cc:

Contact Person:

#### Name:	Yushu Zhang

E-mail Address:	yushu_zhang@apple.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Response/information on SCell BFR:

Response

RAN1 would like to thank RAN2 for the questions on SCell BFR and RAN1 responses are provided as follows.

Q1: Can the UE transmit BFR MAC CE using UL grant of any serving cell or should there be a restriction not to send it on failed serving cell(s)?

R1: At least from RAN1 perspective, there is no need for introducing such restrictions on MAC CE transmission for BFR in Rel-16.

Q2: If the UE already has the UL grant on serving cell(s) on which BFR MAC CE can be transmitted based on the answer to question 1, is the UE still required to transmit SR-like indication for BFR?

R2: In this case, UE is not required to transmit SR-like indication for SCell BFR.

Q3: Is there a case where the SR-like dedicated PUCCH resource for SCell BFR is not configured? If the SR-like dedicated PUCCH resource is not configured, one possible option being considered by RAN2 is that the UE follows the existing framework for requesting uplink resources when no uplink resources are available (i.e. performs CBRA on SpCell).

R3: RAN1 did not discuss this case. RAN1 plans to conclude on this by RAN1#98bis.

Q4: Is the SR-like dedicated PUCCH resource for SCell BFR configured for each SCell separately or is it common for all SCell(s) of the same cell group (i.e. MCG/SCG)?

R4: The SR-like dedicated PUCCH resource for SCell BFR is not configured separately for each SCell.

Q5: What conditions are used for the (successful) completion of the SCell BFR?

R5: When UE receives beam failure recovery response (BFRR) to step 2, UE can consider BFR procedure is finished, where the BFRR to step 2 is a normal uplink grant to schedule a new transmission for the same HARQ process as PUSCH carrying the step 2 MAC CE, which is the same as normal “ACK” for PUSCH.

Additional information

RAN1 would like to provide the following additional information on SCell BFR to RAN2.

- RAN1 suggests RAN2 to give higher priority for SCell BFR MAC CE than at least UL data, and also higher priority for SCell BFR PUCCH than normal SR

- The details on MAC CE for BFR, and whether to transmit a MAC CE to carry BFRQ information for 1 SCell or multiple SCells is up to RAN2

- RAN1 identified that beam failure on multiple SCells can occur simultaneously but have not reached consensus on how often this occurs

2. Actions:

To RAN2 group.

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account for future work.

3. Date of Next TSG-RAN WG1 Meetings:

3GPP RAN1#98-Bis		14 – 18 October 2019				Chongqing, China

3GPP RAN1#99		18 – 22 November 2019			Reno, USA