3GPP TSG RAN WG2#107bis	R2-1912041

Chongqing, China, 14 – 18 October, 2019

3GPP TSG-RAN WG4 #92	R4-1909992

Ljubljana, Slovenia, 26th – 30th August 2019

Title:	Reply LS on simultaneous RX/TX for NR

Response to:	R2-1902745

Release:	Release 16

Work Item:	NR_Mob_enh-Core

Source:	RAN WG4

To:	RAN WG1, RAN WG2

CC: 	-

Contact Person:

#### Name:	Chris Callender

E-mail Address:	christopher.callender@ericsson.com

1. Overall Description:

Further to the liaison response [1], RAN4 provides further information on the issues marked as FFS in the first response.

For handover/SCG change with simultaneous transmission/reception with source and target cells

- If the SCS is different among SSB and data in serving and target cell the following was agreed by RAN4

  - Interband HO with  different SCS configured for source and target cell is feasible.

  - Interfrequency intraband HO with different SCS configured for source and target cell is FFS

  - intrafrequency HO with with different SCS configured for source and target cell is FFS

- If different waveforms are configured for serving and target cells

  - No further agreements were reached for this scenario

RAN4 also discussed requirements for interruptions with simultaneous RX/TX. RAN4 has not agreed numerical values for interruptions. Based on the CA framework, RAN4 expects that there may be interruptions when the simultaneous RX/TX operation is started (UE starts receiving/transmitting with the target cell) and when the simultaneous RX/TX operation is stopped (UE stops receiving/transmitting with the source cell). For interfrequency handovers, such interruptions could always be expected, and for intrafrequency handovers, such interruptions could  be expected whenever the UE needs to reconfigure reception/transmission bandwidth).

2. Actions

To RAN1 and RAN2 group:

ACTIONS:

RAN WG4 respectfully asks RAN WG2 and RAN WG1 to consider above agreement in RAN4 in future discussion.

3. Date of Next RAN4 Meetings:

RAN4#92bis	14 – 18 October 2019	Chongqing, China

RAN4#93	18 – 22 November 2019	Reno, USA

4.References

- R4-1904826, “Reply LS on NR mobility enhancement”, Intel