3GPP TSG-RAN WG2 Meeting #107bis		R2-1912107

Chongqing, P. R. China, 14th–18th October 2019

Source:	CATT (Rapporteur)

Title:	RAN2 work plan for UE Power Saving WI

Agenda Item:	6.11.1

Document for:	Discussion and Decision

# Introduction

In RAN#83 meeting, the RAN1-led WI of UE power saving in NR was agreed and further updated in RAN#84 with the following objectives, left unchanged in RAN#85 [1]:

- Specify power saving techniques with PDCCH-based power saving signal/channel triggering UE adaptation in RRC_CONNECTED mode [RAN1, RAN2, RAN4] - Specify procedures triggering a MAC entity to “wake up” to monitor PDCCH at reception of the PDCCH-based power saving signal/channel for the next occurrence(s) of the drx-onDurationTimer [RAN2, RAN1] NOTE: Any change of PDCCH channel coding and payload interleaver is not in the scope - Specify the procedure of cross-slot scheduling power saving techniques [RAN1, RAN4] NOTE: The procedure is in addition to Rel-15 cross-slot scheduling procedure - Specify the power saving techniques of UE adaptation to the maximum number of MIMO layers [RAN1, RAN2, RAN4] - Specify configuration of a different MIMO layer configuration of the initial/default BWP compared with other BWPs of a Serving Cell. [RAN2, RAN4] - Discuss whether to also extend this to define per-BWP MIMO layer configuration [RAN1, RAN2] - Evaluate if switching and interruption times for UE dynamic adaptation to the maximum number of MIMO layers are needed and which case assuming a relationship between the number of RF ports and the MIMO layer configuration [RAN4] NOTE: Switching on/off the RF is part of the evaluation - Specify a mechanism for a UE to indicate its preference of transitioning out of RRC_CONNECTED state. [RAN2] - Specify, if agreed, the mechanism to provide UE assistance information [RAN2, RAN1]: - Study and select among the following UE assistance information for RAN2 by RAN#85: power preferred information (baseline LTE PPI in a well-defined manner) and UE's preference on C-DRX, BWP and SCell configuration [RAN2]. NOTE: additional UE assistance information for RAN1-specific power saving techniques to indicate what setting of parameters related to this Work Item will lead to power savings for the UE can be included if agreed in RAN1. - Specify network-configured mechanism to relax intra and inter-frequency RRM measurement for neighbour cells for RRC_IDLE/INACTIVE with minimal mobility performance impacts [RAN2, RAN4] - Specify RRM measurement relaxation by allowing measurements with longer intervals, and/or by reducing the number of cells/carriers to be measured [RAN4, RAN2] - Define triggering criteria for the UE to move between relaxed and normal RRM measurements, that considers at least if UE is not at cell edge, or if UE is stationary or with low mobility [RAN2, RAN4]. - Evaluate and specify requirements (if needed) depending on RAN2 criteria [ RAN4] NOTE: No new measurement quantities should be introduced Note: The following objective may be considered in RAN#85 pending progress on SCell dormant behaviour and will be re-evaluated in RAN#85 - Study and specify (if agreed) power saving techniques to reduce PDCCH monitoring on activated SCells for CA and DC (EN-DC in particular). - Solutions (e.g. power saving signal/channel, enhancing Rel-15 DRX procedure) are to be discussed - Enhancements to Rel-15 DRX procedure can be studied further if they do not violate the general principle of single DRX configuration per MAC entity

Furthermore, in RAN2#107, the following agreements were achieved:

PDCCH-WUS:

Agreements:

- The PDCCH-WUS triggers a MAC entity to “wake up” to monitor PDCCH at reception of the PDCCH-based power saving signal/channel for the next occurrence of the drx-onDurationTimer.

- The PDCCH-WUS is considered jointly with DRX i.e. it is only configured when DRX is configured.

- The PDCCH-WUS is monitored at occasions located at a configured offset before the start of the drx-onDurationTimer. The offset is part of physical layer design.

- On a PDCCH-WUS occasion that a UE is monitoring, if the UE is indicated to wake-up to monitor the PDCCH during the next occurrence of the drx-onDurationTimer, the UE starts the drx-onDurationTimer at its next occasion. Otherwise it does not.

- From RAN2 point of view the UE does not monitor WUS during active time.

- If UE is in DRX Active Time during a PDCCH-WUS occasion, it starts the drx-onDurationTimer at its next occasion as in legacy.

- The WUS is configured on the PCell with CA and SpCell with DC (i.e. PCell on MCG and PSCell on SCG)

- RLM and RRM measurements are not impacted by WUS design (i.e. the UE continues to measure the required reference signals as per RRM requirements)

Efficient transition from RRC_CONNECTED to RRC_IDLE/RRC_INACTIVE:

Agreements

-	UE indicates the preferred state using RRC signalling.   The information is provided using UE assistance.  The network uses legacy release to transition the UE. It is up to the network whether and which state the UE should transition to.  UE autonomous release is not supported.

-	The network can control the rate in which the UE transmits the release request.  At least a prohibit timer is introduced.

MIMO layer adaptation:

Agreements

- Maximum number of MIMO layers can be configured at least per DL BWP including initial/default and others.

- If maximum number of MIMO layers is configured for a BWP, the UE uses this value and ignores the cell-specific value provided in the PDSCH-ServingCellConfig IE, when operating in the BWP.

- If maximum number of MIMO layers is not configured for a BWP, the UE uses the cell-specific value provided in the PDSCH-ServingCellConfig IE, when operating in the BWP.

UE assistance:

Agreements:

-	UE assistance reporting follows Rel-15 UE assistance information procedure as a baseline.  The network can configure the UE to be able to report C-DRX UE assistance.  The UE reports only the configured UE assistance information that triggered the report.

-	C-DRX configuration will be provided in UE assistance.   Include the following DRX configuration parameters in UE assistance: long DRX cycle, short DRX cycle, DRX inactivity timer, short DRX cycle timer. FFS DRX on duration and DRX start offset

-	BWP and SCells UE assistance there is not enough consensus.

-	LTE PPI is excluded from UE assistance

RRM measurement relaxation:

Agreements - Measurement relaxation criteria can consider both low mobility and UE location in the cell (e.g. whether the UE is in cell-edge). - UE may activate relaxed measurement criteria if at least any of the following conditions are met: a) Serving Cell measurement does not change more than a relative threshold during a time period - LTE relaxed monitoring criteria in 36.304 is considered as a baseline. Additional enhancements to address aspects that are specific to NR can be considered. b) UE is not a cell edge, meaning that serving cell/beam RSRP/RSRQ/SINR is above a threshold FFS: Whether neighbour cell RSRP should also be considered.

This contribution takes the above into consideration to propose a work plan for RAN2 WG for the remaining RAN2 meeting #107bis – 109.

# Work Plan

RAN2#107bis (1 TU):

- PDCCH-WUS: procedures triggering a MAC entity to “wake up” to monitor PDCCH at reception of the PDCCH-WUS for the next occurrence(s) of the drx-onDurationTimer, specifically:

  - Remaining Stage 2 aspects (if any not already covered by RAN1)

  - Stage 3 aspects of PDCCH-WUS procedure

- Stage-3 aspects of the per-BWP configuration of the DL MIMO layer configuration of a Serving Cell.

- Stage-3 aspects of the configuration and procedure for a UE to indicate its preference of transitioning out of RRC_CONNECTED state with preferred RRC state.

- Mechanism to provide UE assistance information, including:

  - Remaining stage 2 aspects (if any)

  - Stage 3 aspects of UE’s preference on C-DRX

- Network-configured mechanism to relax intra and inter-frequency RRM measurement for neighbor cells for RRC_IDLE/INACTIVE with minimal mobility performance impacts

  - Stage 2 aspects of the RRM measurement relaxation method(s) (RAN4?)

  - Further Stage 2/3 aspects of the triggering criteria selected in RAN2#107 to use for the UE to move between normal and relaxed RRM measurements (and vice-versa).

RAN2#108 (1 TU):

- PDCCH-WUS: procedures triggering a MAC entity to “wake up” to monitor PDCCH at reception of the PDCCH-WUS for the next occurrence(s) of the drx-onDurationTimer

  - Remaining stage 3 details (if any)

  - If further study conducted in RAN1/RAN2 MR-DC/CA WI identifies that any additional UE behavior is needed in support of dormancy behavior when UE is also configured for receiving PDCCH based power saving signal/channel outside active time: define stage 2 aspects of the additional UE behavior.

- Remaining stage 3 details (if any) of per-BWP configuration of the DL MIMO layer configuration of a Serving Cell.

- Remaining stage 3 details (if any) of the configuration and procedure for a UE to indicate its preference of transitioning out of RRC_CONNECTED state with preferred RRC state.

- Remaining stage 3 details (if any) of the mechanism to provide UE assistance information

- Network-configured mechanism to relax intra and inter-frequency RRM measurement for neighbor cells for RRC_IDLE/INACTIVE with minimal mobility performance impacts:

  - Stage 3 aspects of RRM measurement relaxation method(s) (RAN4?)

  - Remaining stage 3 aspects of Triggering criteria selected for the UE to move between normal and relaxed RRM measurements (and vice-versa).

RAN2#109 (1 TU): remaining stage 3 details of:

- PDCCH-WUS: procedures triggering a MAC entity to “wake up” to monitor PDCCH at reception of the PDCCH-WUS for the next occurrence(s) of the drx-onDurationTimer

- Per-BWP configuration of the DL MIMO layer configuration of a Serving Cell

- Mechanism for a UE to indicate its preference of transitioning out of RRC_CONNECTED state with preferred RRC state

- Mechanism to provide UE assistance information

- Network-configured mechanism to relax intra and inter-frequency RRM measurement for neighbour cells for RRC_IDLE/INACTIVE with minimal mobility performance impacts

# Conclusion

This contribution discussed the RAN2 Work Plan for the WI on UE power saving in NR and proposes:

Proposal: RAN2 endorses and applies the proposed work plan.

# Reference

- RAN#84 meeting, RP-191607, New WID: UE Power Saving in NR, Newport Beach, USA, 3rd  – 6th   June 2019;