3GPP TSG-RAN WG2 Meeting #107bis                                                            	R2-1912155

Chongqing, P.R.China, 14th - 18th Oct 2019

Source:	CATT

Title:	Mobility Issue for LEO in NTN System

Agenda Item:	6.6.4.1

Document for:	Discussion and Decision

- Introduction

Handover is a main issue addressed for mobility in NTN system, the objective of this issue is given below:

- Handover: Study and identify mobility requirements and necessary measurements that may be needed for handovers between some non-terrestrial space-borne vehicles (such as Non Geo stationary satellites) that move at much higher speed but over predictable paths [RAN2, RAN1]

In RAN2#105bis meeting, some basic agreements are made for NTN mobility [1]:

Agreements:

- NTN should support both, UEs supporting GNSS based positioning methods and UE not supporting GNSS based positioning methods.

- The use of satellite ephemeris, time and UE location can be used in the RAN for mobility purposes in NTN

In this document, we’d like to discuss solutions for UE mobility in connected mode.

- Discussion

In TN system, cell is fixed once deployed by operator, but the UE is moving in an unpredictable way.  In order to control the mobility for UE in connected mode efficiently, the network needs UE measurement report to assist the serving cell to choose a feasible target cell for the UE.

Observation 1: In TN system, the mobility for UE in connected mode is mainly based on UE measurement report.

As for NTN system, the situation is quite different. All the satellites apart from GEO have speed relative to ground and the satellite cells are moving by time but in a predictable way.

Observation 2: In NTN system, all the satellites apart from GEO are moving by time relative to ground but in a predictable way.

According to the latest TP in TR38.821 [2], six basic scenarios are considered as depicted in Table 1:

Table 1: Reference scenarios

|  | Transparent satellite | Regenerative satellite |
| --- | --- | --- |
| GEO based non-terrestrial access network | Scenario A | Scenario B |
| LEO based non-terrestrial access network: steerable beams | Scenario C1 | Scenario D1 |
| LEO based non-terrestrial access network: the beams move with the satellite | Scenario C2 | Scenario D2 |

To make the analysis for UE mobility easy, we list all the possible handover cases in Table 2:

Table 2: Mobility cases

|  | Handover Cases | Serving cell | Target cell |
| --- | --- | --- | --- |
| Inter-satellite | Case1 | GEO | GEO |
|  | Case2 | GEO | LEO |
|  | Case3 | LEO | GEO |
|  | Case4 | LEO | LEO |
| Intra-satellite | Case5 | GEO | GEO |
|  | Case6 | LEO | LEO |

We don’t think there is necessary to discuss the cases above one by one. We can talk about some common baseline to cover all the cases. In RAN2#104 meeting, we agree to capture the details of the satellite ephemeris into TR38.821, so it’s natural to consider satellite ephemeris based handover mechanism in NTN system. But in TR 38.821, we also assume not all UEs are capable of positioning, so we can’t assume that the network can always get UE location info to assist the satellite ephemeris based handover mechanism. If some UEs are not capable of positioning, we think NR R15 handover mechanism can be as a baseline.

Proposal 1: For UEs not capable of positioning, the legacy handover mechanism can work as a baseline.

In RAN2#105bis meeting, we agreed the use of satellite ephemeris, time and UE location can be used in the RAN for mobility purposes in NTN, which implied UEs should report its valid location info to the network. But even UE capable of positioning, the RAN still can’t guarantee it has valid UE location info. In this case, RAN can’t trigger the location based mobility mechanism and NR R15 handover mechanism can reuse as a baseline.

Proposal 2: For UEs capable of positioning, if the RAN has valid UE location info, ephemeris and UE location based HO method is used for HO procedure, while if the RAN doesn’t have any valid UE location info, the legacy handover mechanism can work as a baseline. It’s up to network implementation to decide whether the UE location info is valid or not.

As for intra-satellite handover case, some adjustments are needed. If the satellite can get the UE location information, the satellite can know which neighbor cell is more feasible for intra-satellite handover based on UE location info and the cell distribution info within the satellite. But how the satellite knows its cell distribution info within the satellite is more like an implementation issue.

Proposal 3: For intra-satellite handover case, how the satellite knows its cell distribution info within the satellite is up to implementation.

Currently, most of our discussion only considers network triggered mobility for connected mode UEs. But the network triggered mobility needs some assistant info from UE, e.g. location info. If the UE can’t report its latest location info to the network in time, the network has to trigger the mobility based on the legacy way. So the performance of network triggered mobility method is restricted by UE location info. If the UE can use the location info itself to control the mobility, it’s more straightforward and UE doesn’t have to worry about when to report its location info to the network.

Proposal 4: RAN2 is kindly asked to consider the UE triggered mobility method for handover purpose, e.g. UE chooses a target cell based on its location info and configured ephemeris info.

- Conclusion

In conclusion, we propose the followings:

Observation 1: In TN system, the mobility for UE in connected mode is mainly based on UE measurement report.

Observation 2: In NTN system, all the satellites apart from GEO are moving by time relative to ground but in a predictable way.

Proposal 1: For UEs not capable of positioning, NR R15 handover mechanism can work as a baseline.

Proposal 2: For UEs capable of positioning, if the RAN has valid UE location info, ephemeris and UE location based HO method is used for HO procedure, while if the RAN doesn’t have any valid UE location info, the legacy handover mechanism can work as a baseline. It’s up to network implementation to decide whether the UE location info is valid or not.

Proposal 3: For intra-satellite handover case, how the satellite knows its cell distribution info within the satellite is up to implementation.

Proposal 4: RAN2 is kindly asked to consider the UE triggered mobility method for handover purpose, e.g. UE chooses a target cell based on its location info and configured ephemeris info.

- References

[1] RAN2#105bis Chairman Notes

[2] 3GPP TR 38.821