3GPP TSG-RAN WG2 Meeting #107bis                                                   R2-1912160

Chongqing, P.R. China, 14th - 18th Oct, 2019

Source:                CATT

Title:	Consideration on the SPS and CG for NTN

Agenda Item:	6.3.3.1

Document for:	Discussion and Decision

Introduction

Compared with the TN system, the characteristic of NTN system is that it has larger RTD. According to [1], the RTDs for different satellites are shown as below:

Table-1 RTD for different satellites

| NTN scenarios | A | B | C1 | C2 | D1 | D2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | GEO transparent payload | GEO regenerative payload | LEO transparent payload |  | LEO regenerative payload |  |
| Maximum Round Trip Delay on the radio interface between the gNB and the UE | 541.46 ms (Worst case) | 270.73 ms | 25.77 ms |  | 12.89 ms |  |
| Minimum Round Trip Delay on the radio interface between the gNB and the UE | 477.48 ms | 238.74 ms | 8 ms |  | 4 ms |  |

For NR Uu, SPS/CG is introduced for reducing the PDCCH overhead and the service latency. For NTN, due to the same reasons, SPS/CG should also be supported. Till now, how to support the SPS/CG for NTN has not been discussed. Hence in this contribution, this issue will be discussed from the following aspects:

- Issue 1: Whether the periodicities of SPS/CG should be modified?

- Issue 2: Whether the maximum number of HARQ processes for SPS/CG should be modified?

- Issue 3: Whether the configuredGrantTimer should be modified?

Discussion

Necessity for modifying the periodicities of SPS/CG

In Rel-15 NR Uu, for TN system, the allowed SPS/CG periodicities for SPS/CG are listed below:

- For DL SPS:

periodicity                 ENUMERATED {ms10, ms20, ms32, ms40, ms64, ms80, ms128, ms160, ms320, ms640, spare6, spare5, spare4, spare3, spare2, spare1}

- For UL CG:

periodicity                      ENUMERATED {sym2, sym7, sym1x14, sym2x14, sym4x14, sym5x14, sym8x14, sym10x14, sym16x14, sym20x14, sym32x14, sym40x14, sym64x14, sym80x14, sym128x14, sym160x14, sym256x14, sym320x14, sym512x14, sym640x14, sym1024x14, sym1280x14, sym2560x14, sym5120x14, sym6, sym1x12, sym2x12, sym4x12, sym5x12, sym8x12, sym10x12, sym16x12, sym20x12, sym32x12, sym40x12, sym64x12, sym80x12, sym128x12, sym160x12, sym256x12, sym320x12, sym512x12, sym640x12, sym1280x12, sym2560x12 }

It is obvious that for TN system, the smallest DL SPS periodicity is 10ms in Rel-15. In Rel-16 URLLC/IIOT item, in order to support the time critical URLLC service which requires the minimum 0.5ms end-to-end latency, the DL SPS periodicity can be further reduced, e.g. reduced to 1 slot or even smaller such as 2 symbols, which is still under discussion.

For NTN system, considering the large RTD, it is obvious that it cannot support the time-sensitive URLLC services. Hence, RAN2 should consider which SPS/CG periodicities should be supported for NTN services. However, the SPS/CG periodicities have impacts on RAN1, hence we had better send LS to RAN1 to check whether the SPS/CG periodicities should be modified for NTN from RAN1 perspective.

Proposal 1: Send LS to RAN1 to check whether the SPS/CG periodicities should be modified for NTN.

Necessity for modifying the maximum number of HARQ processes for SPS/CG

In Rel-15 NR Uu, the maximum HARQ processes for SPS/CG are listed below:

- For DL SPS:

nrofHARQ-Processes    INTEGER (1..8)

- For UL CG:

periodicity                      ENUMERATED {sym2, sym7, sym1x14, sym2x14, sym4x14, sym5x14, nrofHARQ-Processes                  INTEGER(1..16)

For NTN, in order to avoid the misunderstanding of HARQ process ID, it should ensure the HARQ process ID will not be reused during its lifetime. On the last RAN2 meeting, it was agreed that:

- It should be possible to semi-statically enable / disable HARQ feedback by RRC signalling. - The enabling / disabling of HARQ feedback can be configurable on a per UE and per HARQ process basis via RRC signalling.

Based on the above agreements, it means that HARQ feedback enable/disable is configurable. Hence, the needed HARQ process number will be analyzed in case of HARQ feedback is enabled and disabled individually. Assuming the DL SPS periodicity and UL CG periodicity are both use 10ms, the analysis can be as below:

Table-2 Number of HARQ process needed for different types of satellites

|  | A | B | C1 | C2 | D1 | D2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | GEO transparent payload | GEO regenerative payload | LEO transparent payload |  | LEO regenerative payload |  |
| Maximum RTD | 541.46 ms | 270.73 ms | 25.77 ms |  | 12.89 ms |  |
| HARQ process number for DL SPS (HARQ feedback is enabled and maxRetxThreshold is assumed to be 4) | Floor(541.46*5/10) =271 | Floor(270.73*5/10) =136 | Floor(25.77*5/10) =13 |  | Floor(12.89 *5/10) =7 |  |
| HARQ process number for DL SPS (HARQ feedback is disable and repetition is enabled) | Floor(541.46/10) =55 | Floor(5270.73/10) =28 | Floor(25.77/10) = 3 |  | Floor(12.89 /10) = 2 |  |
| HARQ process number for UL CG (HARQ feedback is enabled and maxRetxThreshold is assumed to be 4) | Floor(541.46*5/10) =271 | Floor(270.73*5/10) =136 | Floor(25.77*5/10) =13 |  | Floor(12.89 *5/10) =7 |  |
| HARQ process number for UL CG (HARQ feedback is disable and repetition is enabled) | Floor(541.46/10) =55 | Floor(270.73/10) = 28 | Floor(25.77/10) = 3 |  | Floor(12.89 /10) = 2 |  |

From the above table, it is obvious that for 10ms periodicity SPS/CG, the needed SPS/CG processes for NTN system is very large especially for GEO. Considering the number of SPS/CG process number is decided by RAN1, RAN2 had better send LS to RAN1 to check whether the maximum number HARQ processes supported for SPS/CG need to be extend for NTN.

Proposal 2: Send LS to RAN1 to check whether the maximum number of HARQ processes supported for SPS/CG need to be extended for NTN.

Necessity for modifying the value range of the configuredGrantTimer

In Rel-15 NR Uu, the configuredGrantTimer is introduced. While the timer is running, the UE does not use this HARQ process for new transmission and the UE assumes ACK if the timer is not running. It is obvious that the length of the configuredGrantTimer should at least cover the HARQ RTT.

The current configuredGrantTimer configuration in Rel-15 is as below:

configuredGrantTimer INTEGER (1..64) configuredGrantTimer Indicates the initial value of the configured grant timer (see TS 38.321 [3]) in multiples of periodicity.. periodicity ENUMERATED {sym2, sym7, sym1x14, sym2x14, sym4x14, sym5x14, sym8x14, sym10x14, sym16x14, sym20x14, sym32x14, sym40x14, sym64x14, sym80x14, sym128x14, sym160x14, sym256x14, sym320x14, sym512x14,sym640x14, sym1024x14, sym1280x14, sym2560x14, sym5120x14, sym6, sym1x12, sym2x12, sym4x12, sym5x12, sym8x12, sym10x12, sym16x12, sym20x12, sym32x12, sym40x12, sym64x12, sym80x12, sym128x12, sym160x12, sym256x12, sym320x12, sym512x12, sym640x12, sym1280x12, sym2560x12} periodicity Periodicity for UL transmission without UL grant for type 1 and type 2 (see TS 38.321 [3], clause 5.8.2). The following periodicities are supported depending on the configured subcarrier spacing [symbols]: 15 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 320, 640} 30 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 640, 1280} 60 kHz with normal CP: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} 60 kHz with ECP: 2, 6, n*12, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1280, 2560} 120 kHz: 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1024, 1280, 2560, 5120}.

According to the [2], the relationships amongst symbol, slot, subframe and frame are shown in the Table-3 and Table-4:

Table-3 OFDM symbols per slot, slots per frame, and slots per subframe (NCP)

| SCS |  |  |  |  |
| --- | --- | --- | --- | --- |
| 15 | 0 | 14 | 10 | 1 |
| 30 | 1 | 14 | 20 | 2 |
| 60 | 2 | 14 | 40 | 4 |
| 120 | 3 | 14 | 80 | 8 |

Table-4 OFDM symbols per slot, slots per frame, and slots per subframe (ECP)

| SCS |  |  |  |  |
| --- | --- | --- | --- | --- |
| 60 | 2 | 12 | 40 | 4 |

Based on the above tables, the maximum length of the configuredGrantTimer is 40960ms, which is same for different SCS and different CP length. It can cover the HARQ RTT for all kinds of satellites, which are shown in the Table-5:

Table-5 Maximum length of configuredGrantTimer

| SCS | Max length of the configuredGrantTimer | Can cover the HARQ RTT of satellite? |
| --- | --- | --- |
| 15 | (640*14/14/1)*64=40960ms | GEO transparent payload: Yes (40960>541.46) GEO regenerative payload: Yes (40960>270.73) LEO transparent payload: Yes (40960>25.77) LEO regenerative payload: Yes (40960>12.89) |
| 30 | (1280*14/14/2)*64=40960ms |  |
| 60(NCP) | (2560*14/14/4)*64=40960ms |  |
| 60(ECP) | (2560*12/12/4)*64=40960ms |  |
| 120 | (5120*14/14/8)*64=40960ms |  |

Based on the above table, it is obvious the current range of the configuredGrantTimer can cover the HARQ RTT of all kinds of satellites. Hence, it does not need to be extended for NTN.

Proposal 3: The modification of configuredGrantTimer is not needed to support NTN.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: Send LS to RAN1 to check whether the SPS/CG periodicities should be modified for NTN.

Proposal 2: Send LS to RAN1 to check whether the maximum number of HARQ processes supported for SPS/CG need to be extended for NTN.

Proposal 3: The modification of configuredGrantTimer is not needed to support NTN.

Reference

- TR38.821  Solutions for NR to support non-terrestrial networks (NTN)

- TS38.211 Technical Specification Group Radio Access Network; NR; Physical channels and modulation