# 3GPP TSG RAN WG2 Meeting #107bis      			                           R2-1912199
Chongqing, China, 14th – 18th Oct. 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1344 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.331 on configuration of rach-ConfigCommon in initial UL BWP for RRC triggered BWP switch |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-10-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to below field description of rach-ConfigCommon in BWP-UplinkCommon in latest 38.331, NW is expected to configure RACH-ConfigCommon for CBRA in initial UL BWP if NW configures CFRA for cases of reconfigWithSync or BFR (in firstActiveULBWP): “rach-ConfigCommon Configuration of cell specific random access parameters which the UE uses for contention based and contention free random access as well as for contention based beam failure recovery in this BWP. The NW configures SSB-based RA (and hence RACH-ConfigCommon) only for UL BWPs if the linked DL BWPs (same bwp-Id as UL-BWP) are the initial DL BWPs or DL BWPs containing the SSB associated to the initial DL BWP. The network configures rach-ConfigCommon, whenever it configures contention free random access (for reconfiguration with sync or for beam failure recovery)” The intention is in case that RACH resource is not available in firstActiveULBWP, the NW should ensure that UE have RACH resource to trigger RACH when UE triggers fallback to initial UL BWP according to 38.321: =====================TS 38.321============================== Upon initiation of the Random Access procedure on a Serving Cell, after the selection of carrier for performing Random Access procedure as specified in clause 5.1.1, the MAC entity shall for the selected carrier of this Serving Cell: 1> if PRACH occasions are not configured for the active UL BWP: 2> switch the active UL BWP to BWP indicated by initialUplinkBWP; ================================================================== However, the NW may also trigger BWP switch via RRC (i.e. Reconfiguration without sync). Similar to the case of reconfigWithSync, it is possible that RACH resource is also not available in firstActiveULBWP. Then, in this case if the UE has RACH trigger in firstActiveULBWP at least due to the following reasons: - SR failure - UL data arrival but there are no PUCCH resources for SR available - New Data Arrival when UL synchronisation status is "non-synchronised" UE has to switch to initialULBWP to trigger RACH according to above quoted TS 38.321. However, unlike above cases of ReconfigWithSync or BFR, there is no spec requirement to configure rach-ConfigCommon in initial UL BWP, which may cause ambiguity in UE behaviour if no rach-ConfigCommon available in initial UL BWP. Following the same logic for the cases in ReconfigWithSync or BFR, we think it is necessary to clarify that NW configures rach-ConfigCommon in initial UL BWP if it triggers BWP switch via Reconfiguration without sync. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In field description of rach-ConfigCommon in BWP-UplinkCommon, add below clarification for RRC triggered BWP switch via Reconfiguration without sync: rach-ConfigCommon Configuration of cell specific random access parameters which the UE uses for contention based and contention free random access as well as for contention based beam failure recovery in this BWP. The NW configures SSB-based RA (and hence RACH-ConfigCommon) only for UL BWPs if the linked DL BWPs (same bwp-Id as UL-BWP) are the initial DL BWPs or DL BWPs containing the SSB associated to the initial DL BWP. The network configures rach-ConfigCommon, whenever it configures contention free random access (for reconfiguration with sync, or for beam failure recovery) or it triggers BWP switch via RRCReconfiguration message without ReconfigurationWithSync. Impact Analysis: Impacted functionality: Section 6.3.2: - IE BWP-UplinkCommon Inter-operability: - If the UE is implemented according to the CR and the network is not, it is not clear how UE triggers fallback to initial UL BWP to trigger RACH, but no RACH resource configured. - If the network is implemented according to the CR and the UE is not, there is no impact. |  |  |  |  |  |  |  |  |
| E |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If NW triggers BWP switch via Reconfiguration without sync and doesn’t configures rach-ConfigCommon in initial UL BWP, UE may have RACH trigger due to High Data Arrival/SR failure and fallbacks to initialULBWP to trigger RACH. It will cause ambiguity in UE behaviour if no rach-ConfigCommon avialble in initial UL BWP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of 1st change

#### –	BWP-UplinkCommon

The IE BWP-UplinkCommon is used to configure the common parameters of an uplink BWP. They are "cell specific" and the network ensures the necessary alignment with corresponding parameters of other UEs. The common parameters of the initial bandwidth part of the PCell are also provided via system information. For all other serving cells, the network provides the common parameters via dedicated signalling.

BWP-UplinkCommon information element

-- ASN1START

-- TAG-BWP-UPLINKCOMMON-START

BWP-UplinkCommon ::=                SEQUENCE {

genericParameters                   BWP,

rach-ConfigCommon                   SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

pusch-ConfigCommon                  SetupRelease { PUSCH-ConfigCommon }                                     OPTIONAL,   -- Need M

pucch-ConfigCommon                  SetupRelease { PUCCH-ConfigCommon }                                     OPTIONAL,   -- Need M

...

}

-- TAG-BWP-UPLINKCOMMON-STOP

-- ASN1STOP

BWP-UplinkCommon field descriptions

pucch-ConfigCommon Cell specific parameters for the PUCCH of this BWP.

pusch-ConfigCommon Cell specific parameters for the PUSCH of this BWP.

rach-ConfigCommon Configuration of cell specific random access parameters which the UE uses for contention based and contention free random access as well as for contention based beam failure recovery in this BWP. The NW configures SSB-based RA (and hence RACH-ConfigCommon) only for UL BWPs if the linked DL BWPs (same bwp-Id as UL-BWP) are the initial DL BWPs or DL BWPs containing the SSB associated to the initial DL BWP. The network configures rach-ConfigCommon, whenever it configures contention free random access (for reconfiguration with sync or for beam failure recovery) or it triggers BWP switch via RRCReconfiguration message without ReconfigurationWithSync.

End of 1st change