3GPP TSG-RAN WG2 Meeting #107bis		R2-1912276

Chongqing, China, 14th – 18th Oct, 2019                            (revision of R2-1909202)

Source:	vivo

Title:	Minimize signaling overhead for cell setup of MR-DC

Agenda Item:	6.10.4.4

Document for:	Discussion and Decision

- Introduction

At RAN #81 one of the objectives for the MR-DC WI [1] was agreed to enhance multi-RAT dual connectivity and carrier aggregation for NR [1]:

- Efficient and low latency serving cell configuration/activation/setup: Minimizing signalling overhead and latency needed for initial cell setup, additional cell setup and additional cell activation for data transmission. [RAN2, RAN1, RAN4, RAN3] - This objective applies to MR-DC, NR-NR DC and CA - The objective should consider enhancements when starting from IDLE, INACTIVE mode and CONNECTED mode

In this contribution, we present our views and proposals on minimizing signaling overhead for cell setup for MR-DC.

- Discussion

The current SCell configuration can be roughly divided to common and dedicated configurations, as per the ServingCellConfigCommon and ServingCellConfig IEs that are used by sCellToAddModList. The details are highlighted in yellow.

*************************************From 38.331 ******************************************

CellGroupConfig information element

-- ASN1START

-- TAG-CELL-GROUP-CONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig ::=                         SEQUENCE {

cellGroupId                                 CellGroupId,

rlc-BearerToAddModList                      SEQUENCE (SIZE(1..maxLC-ID)) OF RLC-BearerConfig            OPTIONAL,   -- Need N

rlc-BearerToReleaseList                     SEQUENCE (SIZE(1..maxLC-ID)) OF LogicalChannelIdentity      OPTIONAL,   -- Need N

mac-CellGroupConfig                         MAC-CellGroupConfig                                         OPTIONAL,   -- Need M

physicalCellGroupConfig                     PhysicalCellGroupConfig                                     OPTIONAL,   -- Need M

spCellConfig                                SpCellConfig                                                OPTIONAL,   -- Need M

sCellToAddModList                           SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig           OPTIONAL,   -- Need N

sCellToReleaseList                          SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex            OPTIONAL,   -- Need N

...,

[[

reportUplinkTxDirectCurrent-v1530           ENUMERATED {true}                                           OPTIONAL    -- Cond BWP-Reconfig

]]

}

(omitted)

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

sCellConfigCommon                   ServingCellConfigCommon                                             OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated                ServingCellConfig                                                   OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                                                             OPTIONAL    -- Need S

]]

}

-- TAG-CELL-GROUP-CONFIG-STOP

-- ASN1STOP

*************************************From 38.331 ******************************************

Both ServingCellConfigCommon and ServingCellConfig IEs are configured per SCell.

Some of the parameters in both the ServingCellConfigCommon and ServingCellConfig could be the same for multiple SCells. Also, some of the parameters in both the ServingCellConfigCommon and ServingCellConfig for one cell could be the same for multiple UEs.

Observation 1: Some parameters in both the ServingCellConfigCommon and ServingCellConfig could be common to multiple SCells.

Observation 2: Some parameters in both the ServingCellConfigCommon and ServingCellConfig for one cell could be the same for multiple UEs.

Because the configurations of different SCells may include the same contents, the signalling overhead for SCell addition and modification procedure would basically increase with the increasing number of SCell. Then if the common parts are extracted and broadcasted by request-response method (for example on-demand SI), the signaling overhead can be significantly reduced for SCell(s) configuration. Further, if the UE has stored the SCell(s) configurations, the UE can indicate it to the network. Then the delta signaling mode can be used for the dedicated SCell configuration by RRC signaling.

Proposal 1: The restructuring common parts of the SCell configuration by SI is used to minimize the signaling overhead for SCell setup.

Proposal 2: The delta signaling mode can be used for the restructuring dedicated parts of the SCell configuration by RRC dedicated signaling.

- Conclusion

In this contribution, we discuss the minimizing signaling overhead for cell setup with the following observation and proposals:

Observation 1: Some parameters in both the ServingCellConfigCommon and ServingCellConfig could be common to multiple SCells.

Observation 2: Some parameters in both the ServingCellConfigCommon and ServingCellConfig for one cell could be the same for multiple UEs.

Proposal 1: The restructuring common parts of the SCell configuration by SI is used to minimize the signaling overhead for SCell setup.

Proposal 2: The delta signaling mode can be used for the restructuring dedicated parts of the SCell configuration by RRC dedicated signaling.

- References

- RP-181469, New WID on DC and CA enhancements.

- 3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".