3GPP TSG RAN WG2 Meeting #107-Bis	R2-1912405

Chongqing CN, 14-18 Oct 2019

Title:	Revision to Non-Terrestrial Networks scenarios

Source: 	HNS, Avanti, Thales

Type:	pCR

Document for:	Agreement

Agenda Item:

Study Item: 	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Introduction

This document considers NGSO constellation with altitude greater than 600 km altitude and update the TR 38.821 accordingly.

# Discussion

The purpose of the adding sentence “The study results apply to GEO as well as NGSO with circular orbit at altitude greater than 600 km” is to ensure the NTN characteristics encompass MEO constellation. MEO scenarios feature characteristics are less challenging than GEO scenarios in terms of latency and than LEO scenarios for the doppler effects. Hence, if the normative work considers GEO and LEO scenarios, the 5G system will also be able to support MEO scenarios.

MEO characteristics are already as described in TR 38.811:

Table 5.3.2.1-1: Propagation delays for GEO satellite at 35786 km

|  | GEO at 35786 km |  |  |
| --- | --- | --- | --- |
| Elevation angle | Path | D (km) | Time (ms) |
| UE :10° | satellite - UE | 40586 | 135.286 |
| GW : 5° | satellite - gateway | 41126.6 | 137.088 |
| 90° | satellite - UE | 35786 | 119.286 |
| Bent Pipe satellite |  |  |  |
| One way delay | Gateway-satellite_UE | 81712.6 | 272.375 |
| Round trip Time | Twice | 163425.3 | 544.751 |
| Regenerative Satellite |  |  |  |
| One way delay | Satellite -UE | 40586 | 135.286 |
| Round Trip Time | Satellite-UE-Satellite | 81172 | 270.572 |

Table 5.3.4.1-1: Propagation delays for different NGSO satellites (altitude and payload types)

|  |  | LEO at 600 km |  | LEO at 1500 km |  | MEO at 10000 km |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Elevation angle | Path | Distance D (km) | Delay (ms) | Distance D (km) | Delay (ms) | Distance D (km) | Delay (ms) |
| UE: 10° | satellite - UE | 1932.24 | 6,440 | 3647.5 | 12,158 | 14018.16 | 46.727 |
| GW: 5° | satellite - gateway | 2329.01 | 7.763 | 4101.6 | 13.672 | 14539.4 | 48.464 |
| 90° | satellite - UE | 600 | 2 | 1500 | 5 | 10000 | 33.333 |
| Bent pipe satellite |  |  |  |  |  |  |  |
| One way delay | Gateway-satellite_UE | 4261.2 | 14.204 | 7749.2 | 25.83 | 28557.6 | 95.192 |
| Round Trip Delay | Twice | 8522.5 | 28.408 | 15498.4 | 51.661 | 57115.2 | 190.38 |
| Regenerative satellite |  |  |  |  |  |  |  |
| One way delay | Satellite -UE | 1932.24 | 6.44 | 3647.5 | 12.16 | 14018.16 | 46.73 |
| Round Trip Delay | Satellite-UE-Satellite | 3864.48 | 12.88 | 7295 | 24.32 | 28036.32 | 93.45 |

Observation 1: Propagation round trip delay (RTD) for bent pipe GEO satellite at 35786 km is about 545 ms, while a typical LEO system at 600 km has RTD of 28.4 ms. The RTD for MEO or any NGSO satellite with altitude above 600 km falls in the range from 28.4 ms to 545 ms.

Table 5.3.2.2-1: Differential Delay for GEO satellite

| GEO at 35786 km |  |  |
| --- | --- | --- |
|  | Delta D (km) | Delta Time (ms) |
| Differential One-way delay between nadir and EOC paths | 4800 | 16 |
| Percentage of the difference compared to maximum delay (bent pipe) |  | 5.9 % |
| Percentage of the difference compared to maximum delay (regenerative satellite) |  | 11.9 % |

Table 5.3.4.2-1: Differential Delay for LEO satellite

|  | LEO 600 km |  | LEO 1500 km |  | MEO 10000 km |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Delta Distance | Delta Delay | Delta Distance | Delta Delay | Delta Distance | Delta Delay |
| Differential One-way delay between nadir and EOC paths | 1332.2 km | 4.44 ms | 2147.5 km | 7.158 ms | 4018.16 km | 13.4 ms |
| Percentage of the maximum delay (bent pipe) |  | 31.26 % |  | 27.8 % |  | 14.1 % |
| Percentage of the maximum delay (regenerative satellite) |  | 67 % |  | 58.9 % |  | 28.7 % |

Observation 2: The Differential One-Way Delay between nadir and EOC paths for GEO satellite at 35786 km is 16 ms and for LEO satellite at 600 km is 4.44ms. For MEO or any NGSO satellite with altitude between LEO and GEO, the Differential One-Way Delay between nadir and EOC is in the range from 4.44ms to 16ms, therefore it is within the range of LEO and GEO.

Table 5.3.4.3.2-7: Summary of Doppler shift and shift variation for different altitudes

| Frequency (GHz) | Max Doppler | Relative Doppler | Max Doppler shift variation |  |
| --- | --- | --- | --- | --- |
| 2 | +/- 48 kHz | 0.0024 % | - 544 Hz/s | LEO at 600 km altitude |
| 20 | +/- 480 kHz | 0.0024 % | -5.44 kHz/s |  |
| 30 | +/- 720 kHz | 0.0024 % | -8.16 kHz/s |  |
| 2 | +/- 40 kHz | 0.002 % | -180 Hz/s | LEO at 1500 km altitude |
| 20 | +/- 400 kHz | 0.002 % | -1.8 kHZ/s |  |
| 30 | +/- 600 kHz | 0.002 % | -2.7 kHz/s |  |
| 2 | +/- 15 kHz | 0.00075 % | -6 Hz/s | MEO at 10000 km altitude |
| 20 | +/- 150 kHz | 0.00075 % | -60 Hz/s |  |
| 30 | +/- 225 kHz | 0.00075 % | -90 Hz/s |  |

Observation 3: The Doppler shift and variation of MEO constellation or any NGSO satellite with altitude between LEO and GEO is smaller than that of for LEO constellation.

Table 7.3.3.1.1-1 gives an overview of the number of HARQ processes, based on different values (including the RTT) for different satellite constellations, LEO, MEO, and GEO

| constellation | Max. THARQ | NHARQ, min processes for 1 ms slot operation | UE side feasibility |
| --- | --- | --- | --- |
| Terrestrial | 16ms | 16 | Feasible (Rel. 15) |
| LEO | 50ms | 50 | Feasible (with HARQ extension) |
| MEO | 180ms | 180 | FFS (impact on TBS/MCS) |
| GEO/HEO | 600ms | 600 | FFS (impact on TBS/MCS) |

Observation 4: The RTT for MEO is less than for GEO but greater than for LEO and hence it is FFS if HARQ can be used

# Conclusion

Studies in TR 38.811 and solutions developed for GEO and LEO scenarios in TR 38.821 demonstrate that the doppler effect and latency for NGSO constellation (with altitude greater than 600 km) are less challenging than GEO scenarios in terms of latency and less challenging than LEO at 600 km scenarios for the doppler effects.

In this paper we propose to include in TR 38.821 “The study results apply to GEO scenarios as well as all NGSO scenarios with circular orbit at altitude greater than or equal to 600 km”.

Proposal 1: Add a sentence in the TR 38.821:

“The study results apply to GEO scenarios as well as all NGSO scenarios with circular orbit at altitude greater than or equal to 600 km”.

# Proposed Changes for TR 38.821

*** Start of change***

## 4.2	Non-Terrestrial Networks reference scenarios

We shall consider in this document non-terrestrial networks providing access to user equipment in six reference scenarios including

- Circular orbiting and notional station keeping platforms.

- Highest RTD constraint

- Highest Doppler constraint

- A transparent and a regenerative payload

- One ISL case and one without ISL. Regenerative payload is mandatory in the case of inter-satellite links.

- Fixed or steerable beams resulting respectively in moving or fixed beam foot print on the ground

Six scenarios are considered as depicted in Table 4.2-1 and are detailed in Table 4.2-2.

Table 4.2-1: Reference scenarios

|  | Transparent satellite | Regenerative satellite |
| --- | --- | --- |
| GEO based non-terrestrial access network | Scenario A | Scenario B |
| LEO based non-terrestrial access network: steerable beams | Scenario C1 | Scenario D1 |
| LEO based non-terrestrial access network: the beams move with the satellite | Scenario C2 | Scenario D2 |

Table 4.2-2: Reference scenario parameters

| Scenarios | GEO based non-terrestrial access network (Scenario A and B) | LEO based non-terrestrial access network (Scenario C & D) |
| --- | --- | --- |
| Orbit type | notional station keeping position fixed in terms of elevation/azimuth with respect to a given earth point | circular orbiting around the earth |
| Altitude | 35,786 km | 600 km 1,200 km |
| Spectrum (service link) | <6 GHz (e.g. 2 GHz) >6 GHz (e.g. DL 20 GHz, UL 30 GHz) |  |
| Max channel bandwidth capability (service link) | 30 MHz for band < 6 GHz 1 GHz for band > 6 GHz |  |
| Payload | Scenario A : Transparent (including radio frequency function only) Scenario B: regenerative (including all or part of RAN functions) | Scenario C: Transparent (including radio frequency function only) Scenario D: Regenerative (including all or part of RAN functions) |
| Inter-Satellite link | No | Scenario C: No Scenario D: Yes/No (Both cases are possible.) |
| Earth-fixed beams | Yes | Scenario C1: Yes (steerable beams), see note 1 Scenario C2: No (the beams move with the satellite) Scenario D 1: Yes (steerable beams), see note 1 Scenario D 2: No (the beams move with the satellite) |
| Max beam foot print size (edge to edge) regardless of the elevation angle | 3500 km (Note 5) | 1000 km |
| Min Elevation angle for both sat-gateway and user equipment | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link |
| Max distance between satellite and user equipment at min elevation angle | 40,581 km | 1,932 km (600 km altitude) 3,131 km (1,200 km altitude) |
| Max Round Trip Delay (propagation delay only) | Scenario A: 541.46 ms (service and feeder links) Scenario B: 270.73 ms (service link only) | Scenario C: (transparent payload: service and feeder links) - 25.77 ms (600km) - 41.77 ms (1200km) Scenario D: (regenerative payload: service link only) - 12.89 ms (600km) - 20.89 ms (1200km) |
| Max delay variation within a beam (earth fixed user equipment) | 16ms | 4.44ms (600km) 6.44ms (1200km) |
| Max differential delay within a cell (Note 6) | 10.3 ms | 3.12 ms and 3.18 ms for respectively 600km and 1200km |
| Max Doppler shift (earth fixed user equipment) | 0.93 ppm | 24 ppm (600km) 21ppm(1200km) |
| Max Doppler shift variation (earth fixed user equipment) | 0.000 045 ppm/s | 0.27ppm/s (600km) 0.13ppm/s(1200km) |
| User equipment motion on the earth | 1200 km/h (e.g. aircraft) | 500 km/h (e.g. high speed train) Possibly 1200 km/h (e.g. aircraft) |
| User equipment antenna types | Omnidirectional antenna (linear polarisation), assuming 0 dBi Directive antenna (up to 60 cm equivalent aperture diameter in circular polarisation) |  |
| User equipment Tx power | Omnidirectional antenna: UE power class 3 with up to 200 mW Directive antenna: up to 20 W |  |
| User equipment Noise figure | Omnidirectional antenna: 7 dB Directive antenna: 1.2 dB |  |
| Service link | 3GPP defined New Radio |  |
| Feeder link | 3GPP or non-3GPP defined Radio interface | 3GPP or non-3GPP defined Radio interface |

NOTE 1:	Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite

NOTE 2:	Max delay variation within a beam (earth fixed user equipment) is calculated based on Min Elevation angle for both gateway and user equipment

NOTE 3:	Max differential delay within a beam is calculated based on Max beam foot print diameter at nadir

NOTE 4:	Speed of light used for delay calculation is 299792458 m/s.

NOTE 5: The Maximum beam foot print size for GEO is based on current state of the art GEO High Throughput systems, assuming either spot beams at the edge of coverage (low elevation).

NOTE 6: The maximum differential delay at cell level has been computed considering the one at beam level for largest beam size. It doesn’t preclude that cell may include more than one beam when beam size are small or medium size. However the cumulated differential delay of all beams within a cell will not exceed the maximum differential delay at cell level in the table above.

The NTN study results apply to GEO scenarios as well as all NGSO scenarios with circular orbit at altitude greater than or equal to 600 km

*** End of change***