3GPP TSG-RAN2 Meeting #107-Bis	R2-1912422

# Chongqing, China, 14 – 18 Oct 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.355 | CR | 0245 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Sensor Provide Location Information Elements Correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Polaris Wireless |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2019-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To allow a UE to report to the Location Server not only the uncompensated barometric pressure but also the barometric pressure sensor’s bias (adjustment). This allows for more accurate altitude determination based on the UE’s barometric pressure measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding an IE representing the UE’s barometric pressure sensor’s bias (adjustment) to Sensor-MeasurementInformation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The location server will only be able to calculate the UE’s altitude based on the reported uncompensated barometric pressure but not based on the actual “true” barometric pressure (“true” barometric pressure = reported uncompensated barometric pressure + sensor bias). Therefore the calculated altitude of the UE may be inaccurate (the larger the pressure sensor’s bias, the larger the altitude error). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 6.5.5.2	Sensor Location Information Elements

#### –	Sensor-MeasurementInformation

The IE Sensor-MeasurementInformation is used by the target device to provide UE sensor measurements to the location server.

-- ASN1START

Sensor-MeasurementInformation-r13 ::= SEQUENCE {

measurementReferenceTime-r13		UTCTime						OPTIONAL,

uncompensatedBarometricPressure-r13	INTEGER (30000..115000)		OPTIONAL, -- Cond Barometer

...,

[[

uncertainty-r14					SEQUENCE {

range-r14		INTEGER (0..1000),

confidence-r14	INTEGER (1..100)

}									OPTIONAL

]],

[[

adjustment-r16					INTEGER (-5000..5000) OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Barometer | The field is mandatory present if the Sensor-MeasurementInformation is provided for barometric pressure; otherwise it is not present. |

Sensor-MeasurementInformation field descriptions

measurementReferenceTime This field provides the UTC time when the sensor measurements are performed and should take the form of YYMMDDhhmmssZ.

uncompensatedBarometricPressure This field provides the uncompensated barometric pressure as measured by the UE sensor, in units of Pa.

uncertainty This field provides the expected range for the pressure measurement in units of Pa and the confidence as a percentage that the true pressure lies in a range of (measurement – range) to (measurement + range).

adjustment This field specifies any adjustment in units of Pa applied by a UE to the output of the barometric pressure sensor(s) to produce a more accurate atmospheric pressure measurement. The adjustment may be enabled by previous calibration by the UE of the sensor output using a known reference atmospheric pressure for a known location and altitude or by other means. The more accurate atmospheric pressure is obtained as follows and is not reported directly but only via the uncompensated barometric pressure measurement and adjustment component: accurate atmospheric pressure = uncompensatedBarometricPressure + adjustment The adjustment shall be provided whenever available and applied. If there is no adjustment, a UE may omit the adjustment field. When omitted, a server shall assume a value of zero for the adjustment.

UNCHANGED PARTS OMITED

END OF CHANGES