# 3GPP TSG-RAN WG2#107bis	Tdoc R2-1912561

# Chongqing, China, 14 – 18 Oct 2019

Title:	[Draft] Reply LS on potential improvements for delay critical QoS flows

Response to:	S2-1906832

Release:	Release 16

Work Item:

Source:	Ericsson (To be RAN WG2)

To:	SA2

Cc:	RAN3

Contact Person:

#### Name:	Zhenhua Zou

E-mail Address:	Zhenhua.Zou@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 would like to thank SA2 on the LS S2-1906832 related with potential improvements for delay critical QoS flows. RAN2 has discussed and would like to provide the following answers:

Q1): SA WG2 would like to ask RAN WG2 whether for QoS Flows of Delay critical GBR resource type a new QoS profile parameter (Delayed Packet Discarding) for controlling the handling of delayed packets at the RAN node is considered to be helpful to avoid wasting RAN resources.

Q2): SA WG2 would like to ask RAN WG2 whether for QoS Flows of Delay critical GBR resource type a recommendation to deliver packets that are delayed more than the delay budget for the radio interface is acceptable as long as the other QoS requirements of this QoS Flow can be fulfilled or other QoS Flows are not affected.

Answer to Q1 and Q2):

The potential RAN resource waste is to continuously deliver one out of 10^4 to 10^5 packets after it has not been delivered within the delay budget. RAN2 does not consider it impacts RAN performance.

Q3): SA WG2 would like to ask RAN WG2 whether for QoS Flows of Delay critical GBR resource type direction-specific values for the CN component of the PDB can be used by the NG-RAN to operate with different delay budgets for the uplink and the downlink direction and helpful to improve the resource scheduling for the NG-RAN.

Answer Q3): If the expected CN delay in the uplink and the downlink is different, a reasonable configuration is to set the CN PDB to be the larger of the two so that transmissions in both directions can fulfil the delay budget. This would result in a smaller Uu PDB on one transmission direction. Since resource scheduling is more spectral efficient with a relaxed PDB on Uu interface and the scheduling of uplink and downlink is independent at NG-RAN, providing direction-specific values (if possible) for the CN component of the PDB can be used by the NG-RAN and helpful.

ACTION:

To: SA2

ACTION: 	RAN2 respectfully asks SA2 to take into account the above information.

3. Date of Next RAN WG2 Meetings:

RAN-WG2 Meeting #108	Reno, Nevada, US		18-22 Nov, 2019

RAN-WG2 Meeting #109	Athens, Greece			24-28 Feb, 2020