3GPP TSG RAN WG2 #107bis	 R2-1912642

Chongqing, China, 14 – 18 October, 2019

Agenda Item:	11.14.3 Inter-RAT handover to UTRAN for SRVCC

Source: 	Ericsson

Title:  	Measurements for SRVCC from 5G to 3G

Document for:	Discussion and Decision

# Introduction

In RAN#82 the work item for Single Radio Voice Call Continuity from 5G to 3G was approved and it was further revised in RAN#83 in [1]. The purpose of the work item is to enable handover of voice calls from 5G to 3G, in order to avoid drop of voice calls when the 5G coverage ends.

# Discussion

## Background

A study of SRVCC from 5G to 3G was done in SA2 and the result was captured in a technical report [2]. The report contains different issues that need to be handled to enable SRVCC from 5G to 3G and solutions for the issues. The following issues impact RAN (also listed in [1]):

- Solution#1: Enable 5G SRVCC with existing mechanism, including 5GS Registration for SRVCC, Service Request procedure for SRVCC, Intra-5GS PS Handover procedure for SRVCC, and eSRVCC from 5G to 3G using ATCF enhancements. Normative work is expected to be reflected in TS 23.501, TS 23.502 and TS 23.216.

- Solution#2: Indirect SRVCC Procedure to support SRVCC HO procedure from NG-RAN to UTRAN. Normative work is expected to be reflected in TS 23.216.

- Solution#5: PDU Session Release after 5G-SRVCC procedure is adopted as conclusion. IMS service control is assumed to be transferred to the CS domain when UE moves to UTRAN cell due to 5G-SRVCC.

- Solution #9: Return 5GS after 5G SRVCC depend on UE implementation and no normative work is needed.

The SRVCC procedure from NG-RAN to UTRAN was concluded in [2].

Figure 1: SRVCC call flow

## Measurements for SRVCC

In RAN2#105, RAN2 sent an LS to RAN4 asking questions about measurements for SRVCC and now RAN4 have replied in [3]. RAN4 informs that:

- Existing per-UE or FR1 gap patterns can be reused for measuring on UTRA FDD cells.

- There are no extra restrictions for configuring the UE with both UTRA FDD and E-UTRAN measurements.

- There is no extra work to support NR-DC and NE-DC in case of SRVCC.

Based on this reply it can be concluded that nothing new needs to be defined in 38.331 for the configuration of gaps as existing configurations can be reused. No restrictions in the configuration need to be taken into account either.

Proposal 1: No new gap configuration is added in 38.331 for SRVCC.

# Summary

RAN2 is kindly asked to discuss the following proposal:

Proposal 1: No new gap configuration is added in 38.331 for SRVCC.

# References

- RP-190713, Work Item on Single Radio Voice Call Continuity from 5G to 3G, China Unicom, RAN#83, March 2019

- TR 23.756, Study for single radio voice continuity from 5GS to 3G

- R2-1912043, LS Reply on measurements for SRVCC from 5G to 3G

- R2-1911566. Running CR for the introduction of SRVCC from 5G to 3G