3GPP TSG-RAN WG2 Meeting#107bis	R2-1912677 (Revision of R2-1909236)

Chongqing, China, 14th Oct – 18th Oct 2019

Agenda Item:	6.2.3.5

Source:	Panasonic, MediaTek Inc., vivo, Charter Communications, OPPO

Title:	The issue of putting all SIBs in one SI message

Document for:	Discussion, Decision

# Introduction

In RAN2#106 meeting, the SI enhancements for NR-U were discussed [1], and the conclusion is that no optimization is needed for the case when all broadcast SI can be put into one SI message.

R2-1906740 The Increased SI Transmission Opportunities in NR-U PANASONIC R&D Center Germany. Option 1. One SI message maps to one unique SI window, overlapping of SI windows is not allowed Option 2. One SI message maps to one unique SI window, overlapping of SI windows is allowed Option 3. One SI message map to one SI window which can be shared by multiple SI messages Option 4. One SI message is allowed to map to more than one SI windows Option 5. The mapping between the SI messages and SI windows is configurable DISCUSSION - QC think that O4 is not needed and think that it would be beneficial to be able to transmit as much SI as possible once the channel is occupied. - Panasonic think O4 is beneficial. - Nokia think we don’t have that many SIBs, Ericsson agrees that no improvement is needed and think that today all SI can be sent in one message. LG agrees. Google think the number of SIBs will increase. - ZTE also think we don’t need anything. With small cells we can rely more on on-demand SI. - Charter think we need to address this. For the case when all broadcast SI can be put in one SI message, no optimization is needed.

This paper first observes that putting all SIBs into one SI message and transmitting it in NR-U is not always feasible, and then proposes possible remedies to overcome the issue caused by previous agreement.

# Discussion

## The issue of putting all SIBs into one SI message

In TS 38.214 [2], section 5.1.3.2 specifies the restriction on the size of the SI message as follows.

“The UE is not expected to receive a PDSCH assigned by a PDCCH with CRC scrambled by SI-RNTI with a TBS exceeding 2976 bits.”

If we only rely on the assumption that all SIBs are to be put together in single SI message to resolve the SI issues (e.g., the overlapping of extended SI windows) in NR-U, it may not always be practicable due to the SI size limitation specified in TS 38.214. Table 1 lists the size of each SIB under different situations. The minimal size of SIBx is calculated based on the situation where no optional fields are not presented and only mandatory fields are presented. The maximal size of SIBx is calculated based on the situation where all fields are presented and the WhateverList IE contains the maximal possible elements (e.g., frequencyBandList in SIB2 contains 8 NR-MultiBandInfo IEs). As both ‘minimal size’ and ‘maximal size’ situations are extreme cases and hence are not suitable to be used to argue whether it is practicable to put all SIBs into one SI message, we further assume another compromised and conservative case where although all optional fields are presented, the WhateverList IE contains moderate number of elements. Table 2 explains the assumptions we use to calculate the size of each SIB in the 4th column of Table 1.

TABLE 1. The calculated SIB sizes under different situations.

|  | Minimal size (bit) | Maximal size (bit) | Size for the assumption case (bit) |
| --- | --- | --- | --- |
| SIB2 | 39 | 1,583 | 639 |
| SIB3 | 0 | 616 | 304 |
| SIB4 | 65 | 17,512 | 2,820 |
| SIB5 | 3 | 11,519 | 2,087 |
| SIB6 | 48 | 56 | 56 |
| SIB7 | 47 | 63 | 63 |
| SIB8 | 47 | 71 | 71 |
| SIB9 | 39 | 64 | 64 |
| Total size | 288 | 31,484 | 6,104 |

TABLE 2. The details of the assumption for the ‘assumption case’ in Table 1.

|  | Description of the assumption case |
| --- | --- |
| SIB2 | All fields are presented; frequencyBandList contains 4 NR-MultiBandInfo; frequencyBandListSUL contains 4 NR-MultiBandInfo; nr-NS-PmaxList contains 4 NR-NS-PmaxValue |
| SIB3 | All fields are presented; intraFreqNeighCellList contains 8 IntraFreqNeighCellInfo; intraFreqBlackCellList contains 8 PCI-Range |
| SIB4 | All fields are presented; interFreqCarrierFreqList contains 3 InterFreqCarrierFreqInfo; frequencyBandList contains 4 NR-MultiBandInfo; frequencyBandListSUL contains 4 NR-MultiBandInfo; nr-NS-PmaxList contains 4 NR-NS-PmaxValue; intraFreqNeighCellList contains 8 IntraFreqNeighCellInfo; intraFreqBlackCellList contains 8 PCI-Range |
| SIB5 | All fields are presented; carrierFreqListEUTRA contains 4 CarrierFreqEUTRA; eutra-multiBandInfoList contains 4 EUTRA-MultiBandInfo; eutra-FreqNeighCellList contains 4 EUTRA-FreqNeighCellInfo; eutra-BlackCellList contains 8 EUTRA-PhysCellIdRange |
| SIB6 | All fields are presented |
| SIB7 | All fields are presented |
| SIB8 | All fields are presented |
| SIB9 | All fields are presented |

In the assumption case, putting SIB2, SIB3, and SIB4 together already exceeds the size limitation specified in TS 38.214, hence the chance of exceeding the size limitation is even higher if all SIBs are to be put together in one SI message.

Observation 1: Putting all SIBs into one SI message may exceed the size limitation of the SI message, and hence is not always possible.

Furthermore, the latency requirement for acquiring one SIB could be different from that of acquiring another SIB. Putting all SIBs into one SI message means the network has to schedule the SI message in a way that the most restricted latency requirement among all SIBs has to be satisfied. Inevitably, the SI messages need to be transmitted quite frequently, which may result in significant overheads.

Observation 2: Putting all SIBs into one SI messages may force the network to schedule the SI messages according to the most restricted latency requirement among the SIBs, which may produce unnecessary overheads.

Based on the observations, we suggest RAN2 to reopen the discussion on the possible SI enhancements [1][3][4][5][6][7] for NR-U, instead of relying on the assumption that all SIBs are to be transmitted together in one SI message.

Proposal 1: RAN2 reopen the discussion on the possible SI enhancements for NR-U, instead of relying on the assumption that all SIBs are to be transmitted together in one SI message.

If proposal 1 is not agreeable, we may need to ask RAN1 to relax the size limitation for the SI message, so that it can be easier for the NR-U network to transmit all SIBs together in one SI message.

Proposal 2: Check RAN1 if it is possible to relax the size limitation (2976 bits) for the SI message, given that RAN2 assume all SIBs are to be transmitted together in one SI message in NR-U.

# Conclusion

In this paper, we discuss the feasibility/practicability of putting all SIBs into one SI message in NR-U, and have the following observations.

Observation 1: Putting all SIBs into one SI message may exceed the size limitation of the SI message, and hence is not always possible.

Observation 2: Putting all SIBs into one SI messages may force the network to schedule the SI messages according to the most restricted latency requirement among the SIBs, which may produce unnecessary overheads.

Based on the observations, we propose that RAN2 should reopen the discussion on the SI enhancements in NR-U, instead of relying on the assumption that all SIBs are to be transmitted together in one SI message.

Proposal 1: RAN2 reopen the discussion on the possible SI enhancements for NR-U, instead of relying on the assumption that all SIBs are to be transmitted together in one SI message.

If proposal 1 is not agreeable, we would like RAN2 to check with RAN1 whether it is possible to relax the size limitation of the SI message.

Proposal 2: Check RAN1 if it is possible to relax the size limitation (2976 bits) for the SI message, given that RAN2 assume all SIBs are to be transmitted together in one SI message in NR-U.

# References

R2-1906740, The Increased SI Transmission Opportunities in NR-U, Panasonic R&D Center Germany.

TS 38.214, NR Physical layer procedures for data, V15.6.0, June 2019.

R2-1908695, Enhancements of System Information, vivo.

R2-1908790, System information enhancements for NR-U, OPPO.

R2-1908850, SI scheduling enhancements for NR-U, MediaTek Inc.

R2-1910893, Broadcasting of System Information in NR-U, Qualcomm Incorporated.

R2-1911104, System Information Transmission in NR-U, Charter Communications.