3GPP TSG-RAN WG2 Meeting #107bis 	R2-1912715

# Chongqing, China, 14 – 18 October 2019 										 Revision of R2-1910905

Agenda item:		6.7.2.2.1

Source:	Intel Corporation

Title:	Support of multiple SPS and CG configurations

Document for:		Discussion and Decision

# Introduction

In RAN2#105bis meeting, following was agreed regarding the support of multiple SPS and CG configurations:

R2 assumes that the maximum number of active SPS configurations for a given BWP of a serving cell in the specification is 8 or 16 (FFS). R2 assumes short SPS/CG periodicities and/or multiple SPS/CG configurations and/or combination thereof could be used to mitigate the periodicity misalignment between the TSN periodicity and CG/SPS periodicity. Other solutions not precluded, e.g. to address resource consumption. Will support “short” SPS periodicities, at least down to 0.5ms R2 assumes that activation/deactivation is done by DCI. When multiple UL CG or DL SPS configurations is configured, an offset for each configuration is needed for the calculation of the HARQ process ID

In this contribution, we discuss the support of multiple SPS and CG configurations.

# Discussion

## Number of SPS configuations

In RAN2#105bis meeting, RAN2 agreed that “the maximum number of active SPS configurations for a given BWP of a serving cell in the specification is 8 or 16 (FFS).” In RAN1#97 meeting, RAN1 concluded that “Although RAN1 has not completely analysed the potential impact of supporting up to 16 SPS configurations for a given BWP of a serving cell, RAN1 has the understanding that 8 SPS configurations for a given BWP of a serving cell is sufficient in Rel-16” [6]. Note that the conclusion of supporting 12 UL CG configurations per BWP does not imply that DL should support the same number. In DL, gNB has more flexibility since it performs both transmission and scheduling of DL traffic. Given that 8 SPS configurations can satisfy the TSC requirements, RAN2 can conclude according to RAN1’s suggestion.

Proposal 1: RAN2 confirms RAN1’s earlier understanding that in Rel-16, the maximum number of active SPS configurations for a given BWP of a serving cell is 8.

## Configuration index for SPS and CG

LTE Rel-14 V2X introduces multiple SPS configurations. For example, following is a snippet of related ASN.1 code.

SPS-Config-v1430 ::=	SEQUENCE {

ul-SPS-V-RNTI-r14					C-RNTI					OPTIONAL,			-- Need OR

sl-SPS-V-RNTI-r14					C-RNTI					OPTIONAL,			-- Need OR

sps-ConfigUL-ToAddModList-r14		SPS-ConfigUL-ToAddModList-r14	OPTIONAL,	-- Need ON

sps-ConfigUL-ToReleaseList-r14		SPS-ConfigUL-ToReleaseList-r14	OPTIONAL,	-- Need ON

sps-ConfigSL-ToAddModList-r14		SPS-ConfigSL-ToAddModList-r14	OPTIONAL,	-- Need ON

sps-ConfigSL-ToReleaseList-r14		SPS-ConfigSL-ToReleaseList-r14	OPTIONAL	-- Need ON

}

SPS-ConfigUL-ToAddModList-r14 ::= SEQUENCE (SIZE (1..maxConfigSPS-r14)) OF SPS-ConfigUL

SPS-ConfigUL-ToReleaseList-r14 ::= SEQUENCE (SIZE (1..maxConfigSPS-r14)) OF SPS-ConfigIndex-r14

SPS-ConfigSL-ToAddModList-r14 ::= SEQUENCE (SIZE (1..maxConfigSPS-r14)) OF SPS-ConfigSL-r14

SPS-ConfigSL-ToReleaseList-r14 ::= SEQUENCE (SIZE (1..maxConfigSPS-r14)) OF SPS-ConfigIndex-r14

Although there are differences between LTE and NR regarding ASN.1 signalling details on SPS and configured grant (e.g. in LTE, sps-Config is signaled in RadioResourceConfigDedicated, while in NR, sps-Config and configuredGrantConfig are signaled in BWP-DownlinkDedicated and BWP-UplinkDedicated, respectively), it is nevertheless reasonable to use LTE design principles for multiple SPS configurations as a starting point in NR. In LTE, SPS configuration index is introduced for SPS configuration management in RRC signalling, and is also transmitted in DCI. It is straightforward to introduce SPS configuration index and Configured grant configuration index in NR in a similar way. DCI related aspects can be left for RAN1.

Proposal 2: Similar to LTE, SPS configuration index and Configured grant configuration index are introduced in RRC signalling.

For release of Type 2 Configured Grants, following was agreed in RAN1#98 meeting:

- M<=4 bits indication in the Release DCI is used for indicating which CG configuration(s) is/are released, where the association between each state indicated by the indication and the CG configuration(s) is - Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple CG configurations to be released - In case of no higher layer configured state(s), separate release is used where the release corresponds to the CG configuration index indicated by the indication

RRC parameters for release DCI was discussed in RAN1 email discussion “[98-RRC-NR_L1enh_URLLC] Email discussion on the RRC parameters for NR eURLLC”, and the details for RRC signaling design were left to RAN2. There are basically 2 options for RRC signaling design:

Option A:

- List of up to 12 ConfiguredGrantConfig, and

- A list of DCI states associated with one or more configured grant configuration indices

Option B:

- List of up to 12 ConfiguredGrantConfig, and

    - Each configuration has one new parameter, which is a list of DCI states which can deactivate this configuration.

Option B leads to less re-configurations when the list of CG is updated. Furthermore, it is simpler as it does not require a standalone table. An example to implement Option B is shown below with new parameter type2ReleaseStateList.

ConfiguredGrantConfig ::=           SEQUENCE {

-- Omitted

rrc-ConfiguredUplinkGrant               SEQUENCE {

timeDomainOffset                        INTEGER (0..5119),

timeDomainAllocation                    INTEGER  (0..15),

frequencyDomainAllocation               BIT STRING (SIZE(18)),

antennaPort                             INTEGER (0..31),

dmrs-SeqInitialization                  INTEGER (0..1)                                                  OPTIONAL,   -- Need R

precodingAndNumberOfLayers              INTEGER (0..63),

srs-ResourceIndicator                   INTEGER (0..15)                                                 OPTIONAL,   -- Need R

mcsAndTBS                               INTEGER (0..31),

frequencyHoppingOffset                  INTEGER (1.. maxNrofPhysicalResourceBlocks-1)                   OPTIONAL,   -- Need R

pathlossReferenceIndex                  INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1),

...

}                     OPTIONAL,   -- Need R

...

[[

type2ReleaseStateList           SEQUENCE (SIZE (1..16)) OF INTEGER (0..15)     OPTIONAL  -- Need R

]]

}

Proposal 3: RRC signaling design for release of type 2 CG is implemented by adding each CG configuration a new parameter, which is a list of DCI states which can deactivate the configuration.

## Address TSC message periodicities with non-integer multiple of NR supported periodicities

Following potential solutions were captured in TR 38.825 [1] to address TSC message periodicities with non-integer multiple of NR supported periodicities:

- Adjustment of SPS/CG resource by RRC reconfiguration (as per current specification)

- Usage of short SPS/CG periodicities and/or multiple SPS/CG configurations and/or combination thereof (for SPS, support for shorter periodicities than those available in Rel-15 may be required)

- More efficient adjustment of SPS/CG resource timing in the UE as compared to RRC reconfiguration, e.g. based on network configuration or dynamic network signalling and which could be based on knowledge of TSN traffic pattern

- Applying de-jittering buffer at the edges of 5G system

It should be noted that the 2nd potential solution is already in the scope of current WI (with discussion in section 2.1), while the 4th potential solution is also captured in SA2 TS 23.501, as below:

-	Support for hold & forward buffering mechanism in DS-TT and NW-TT to de-jitter flows that have traversed the 5G System.

Listed below are the examples of TSC use cases in [5] with message periodicities with non-integer multiple of the currently supported CG/SPS periodicities.

| Use case | Frequency |  |
| --- | --- | --- |
| Smart grid | 1200 Hz | 0.833 ms |
| Video based sampling | 60 Hz | 16.667 ms |
| PTP synchronization | 8 Hz | 125 ms |

It should be noted that in TS 22.104 [4], the required latency for the smart grid use case (cf. Table 5.3-1) is < 50 ms. Therefore, there is no need to have a very tight latency requirement, and the jittering issue can be handled by various options (e.g. dejittering buffer at the edge of 5G system).

According to [5], multiple SPS/CG configurations can achieve the same allocation utilization ratio as the pre-determined shift approach, while having better latency drift. The only potential drawback of this approach is the requirement of multiple SPS/CG configurations for a single TSC flow. Given that each BWP can support 8 SPS and 12 CG configurations and the UE can support carrier aggregation, the current specification already supports sufficient number of TSC flows even if one TSC flow requires multiple SPS/CG configurations.

In addition, in NR-U, configured grant with bitmap (as in AUL) is under discussion. The solution can also help to address TSC message periodicities with non-integer multiple of NR supported periodicities.

Given that gNB is provided with TSC traffic patterns, there are several mechanisms gNB could utilize to address TSC message periodicities with non-integer multiple of NR supported periodicities. For example, gNB can configure UE with shorter periodicity (UE can skip UL transmission if no data is to be sent), multiple configurations, bitmap based configured grant, or even use dynamic scheduling if necessary. In addition, de-jittering feature agreed by SA2 can minimize the jittering problem.

In RAN2#105bis meeting, following was agreed “R2 assumes short SPS/CG periodicities and/or multiple SPS/CG configurations and/or combination thereof could be used to mitigate the periodicity misalignment between the TSN periodicity and CG/SPS periodicity. Other solutions not precluded, e.g. to address resource consumption.” Regarding the potential resource consumption issue, it should be noted that gNB knows the TSC traffic pattern. Therefore gNB can know in advance which configured grant resource will not be used by the UE, and can schedule other UEs to use the resource. Therefore there is no resource consumption issue.

Observation 1: There is no resource waste/consumption issue of using multiple CG/SPS configurations and/or short SPS/CG periodicities, since gNB is aware of the TSC traffic pattern.

In summary, additional mechanisms are not needed to address TSC message periodicities with non-integer multiple of NR supported periodicities.

Proposal 4: There is no need to introduce additional mechanisms to support TSC message periodicities with non-integer multiple of NR supported CG/SPS periodicities.

One related discussion is the grouping of multiple SPS/CG configurations of the same TSC flow, with the intention to support TSC message periodicities with non-integer multiple of CG/SPS periodicities. The potential benefit is the reduction of the RRC signalling overhead. From the above discussion, existing mechanisms can already address the periodicity mismatch issue well. Considering that the RRC signalling saving is not significant, it is proposed not to consider the grouping approach, i.e. the SPS/CG configurations is configured in RRC signalling independently.

Proposal 5: SPS/CG configurations are independently configured in RRC, i.e. grouping of multiple SPS/CG configurations of the same TSC flow is not used.

## Collision between SPS configurations

Collision between SPS configurations were discussed in [7][8]. First, it should be noted that intra-UE prioritization of DL transmissions is not in the scope of IIoT WID [2]. With such background in mind, we can investigate the potential impacts of collision between SPS configurations. Two gNB behaviors could be possible:

- gNB sends a dynamic assignment to eliminate the collision between SPS configurations since the priority of the dynamic assignment is higher than SPS if dynamic assignment overlaps with the colliding SPS configurations. However, the question is why gNB should configure the colliding SPS configurations at all in this case given that anyway at least one of the SPS configurations will not be used.

- gNB does not send dynamic assignment to eliminate the collision. The problem here is that UE does not know which SPS to use. There are three possible UE behaviors:

  - UE selects one of the colliding SPS configuration to use. The problem here is that UE may select a wrong SPS configuration as UE is not aware which SPS configuration gNB selects.

  - UE performs blind detection on both SPS configurations. This behavior significantly increases UE complexity, since UE needs to have additional processing power, more soft buffering capability, and higher number of HARQ process capability.

  - UE uses none of the colliding SPS configurations.

Above discussion shows that colliding SPS configurations either increases UE complexity dramatically or doesn’t bring any clear benefits. Given that it is not in the scope of IIoT WID, it is proposed to not consider such configuration.

Proposal 6: Collision between SPS resources in one BWP is not part of the WI discussion.

# Conclusion

In this contribution, we discuss the support of multiple SPS and CG configurations. We have the following observation:

Observation 1: There is no resource waste/consumption issue of using multiple CG/SPS configurations and/or short SPS/CG periodicities, since gNB is aware of the TSC traffic pattern.

We propose the following:

Proposal 1: RAN2 confirms RAN1’s earlier understanding that in Rel-16, the maximum number of active SPS configurations for a given BWP of a serving cell is 8.

Proposal 2: Similar to LTE, SPS configuration index and Configured grant configuration index are introduced in RRC signalling.

Proposal 3: RRC signaling design for release of type 2 CG is implemented by adding each CG configuration a new parameter, which is a list of DCI states which can deactivate the configuration.

Proposal 4: There is no need to introduce additional mechanisms to support TSC message periodicities with non-integer multiple of NR supported CG/SPS periodicities.

Proposal 5: SPS/CG configurations are independently configured in RRC, i.e. grouping of multiple SPS/CG configurations of the same TSC flow is not used.

Proposal 6: Collision between SPS resources in one BWP is not part of the WI discussion.

# References

[1] 3GPP TR 38.825, "Study on NR Industrial Internet of Things (IoT)"

[2]	RP-192324, Nokia et al: "Revised WID: Support of NR Industrial Internet of Things (IoT)"

[3]	S2-1902855, Nokia et al: "Introducing support TSC Deterministic QoS"

[4] 3GPP TS 22.104, "Service requirements for cyber-physical control applications in vertical domains"

[5]	R2-1907198, Nokia et al: "Support for TSC message periodicities of non-integer multiple of NR CG/SPS periodicities"

[6]	R2-1908626 (R1-1907961), RAN1 LS: "Reply LS on SPS/CG for IIoT"

[7]	R2-1909491, Nokia et al: "SPS collisions with multiple SPS configurations"

[8]	R2-1909648, NTT DOCOMO: "Support for multiple CG and SPS configurations"