3GPP TSG-RAN2 Meeting # 107b	R2-1912758

# Chongqing, China, 14th – 18th Oct 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 0772 | rev | - | Current version: | 15.4.0 |  |
| For HELP on using t4is form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the handling of stored AS context upon CN type change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_5GCN_connect-Core |  |  |  |  |  | Date: |  |  | 2019-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For RRC_INACTIVE it is specified in 38.304 subclause 5.3.4.8 that the UE performing inter-RAT cell reselection shall transit to RRC_IDLE and perform relevant actions. In 38.331 it is specified that the UE moving to RRC_IDLE discards stored AS context information related to RRC_INACTIVE. Similarly, for RRC_INACTIVE it is specified in 36.331 subclause 5.3.17.3 that the UE performing inter-RAT cell reselection or to another CN type while in RRC_INACTIVE shall perform the actions upon leaving RRC_INACTIVE as specified in 5.3.12, with release cause 'other'. Hence, for RRC_INACTIVE case, both inter-RAT cell reselection and CN Type change (i.e. from 5GC to EPC) results in discarding the stored AS context information. However, for the CN type change from EPC to 5GC there is currently no corresponding requirement. In this case the UE should also discard the stored AS context information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 5.2.4.11 describes UE behaviours for cell reselection when stroing UE AS context, so this section is extended to apply to CN type change. Impact analysis Impacted functionality: LTE connected to 5GC Inter-operability: If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. If the network is implemented according to the CR and the UE is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may not discard the stored AS context upon CN type change, resulting in potential RRC Resume failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.4.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.2.4.11	Cell reselection or CN type change when storing UE AS context

For UEs storing UE AS context and resumeIdentity as specified in TS 36.331 [3], upon cell reselection to another RAT or upon reselecting to another CN type, the UE shall discard the stored UE AS context and resumeIdentity.