3GPP TSG-RAN WG2 Meeting #107bis	R2-1913014

# Chongqing, P.R. China, 14th – 18th October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0665 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of indication for successful Random access procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS 38.331 (subclause 5.3.5.3) there is a condition that the UE must successfully complete a Random Access procedure triggered earlier in the subclause. 1> if reconfigurationWithSync was included in spCellConfig of an MCG or SCG, and when MAC of an NR cell group successfully completes a Random Access procedure triggered above; Currently there is no indication from MAC to RRC defined when a Random Access procedure is successfully completed. This should be compared with request for on demand system information (subclause 5.2.2.3.3 in TS 38.331) in which RRC may receive an acknowledgement for SI request from MAC. The successful completion of the Random Access procedure is necessary for the RRC Reconfiguration with sync to complete. Thus, if there is no indication from MAC, then RRC Reconfiguration with sync may fail. Impact analysis: Impacted functionality: RRC Reconfiguration with sync Interoperability analysis: If the network implements the CR and the UE does not the network will assume the UE properly supports RRC Reconfiguration with sync which the UE need not. If the UE implements the CR and the network does not there is no interoperability problem. Impacted 5G architectures: (NG)EN-DC and NR-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of an indication in MAC that indicates successful completion of Random Access procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network may avoid configuring the UE with RRC Reconfiguration with sync to avoid a potential failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.1.6	Completion of the Random Access procedure

Upon completion of the Random Access procedure, the MAC entity shall:

1>	discard explicitly signalled contention-free Random Access Resources except contention-free Random Access Resources for beam failure recovery request, if any;

1>	flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer.

1>	if the Random access procedure was successfully completed:

2>	indicate successful completion of Random Access procedure to upper layers;