3GPP TSG-RAN WG2 Meeting #107bis	R2-1913022

Chongqing,  China, 14 – 18 October 2019

Agenda item:	5.4.4

Source:	Nokia, Nokia Shanghai Bell

Title:	On FR2 CA fallbacks

WID/SID:	NR_newRAT-Core- Release 15

Document for:	Discussion and Decision

# 1	Introduction

For FR2 operation, RAN4 agreed the following specification change in TS 38.101-2 [3]: “A terminal which supports CA or DC configurations, which include FR2 intra-band CA combinations with multiple subblocks, where at least one of the subblocks consists of a contiguous CA combination, is not required to support all possible fallback combinations but can directly fall back to a single FR2 carrier. Deactivating carriers within the CA or DC combination is still possible.”

RAN4 has sent the LS to RAN2 in [2] asking RAN2 the following [6] (with some emphasis added with yellow highlighting):

RAN4 would like to inform RAN2 about a decision on mandatory fallbacks in FR2: A terminal which supports CA or DC configurations, which include FR2 intra-band CA combinations with multiple subblocks, where at least one of the subblocks consists of a contiguous CA combination, is not required to support all possible fallback combinations but can directly fall back to a single FR2 carrier. Deactivating carriers within the CA or DC combination is still possible. RAN4 would like to ask RAN2 to help enabling signalling of CA/DC configurations without mandating all possible fallback combinations.

In this contribution we discuss the RAN2 aspects related to the above LS request.

# 2	Background

While we have fallback groups for contiguous CA, fallback groups or other limitations of fallbacks are missing for combinations of contiguous and non-contiguous fallbacks. Currently the specification carries this sentence on fallbacks in 38.101-3 [4]:

“A terminal which supports an inter-band EN-DC configuration with a certain UL configuration shall support the all lower order DL configurations of the lower order EN-DC combinations, which have this certain UL configuration and the fallbacks of this UL configuration.”

Table 5.3A.4-1: CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ 400 MHz | 1 | 1,2,3,4 |
| B | 400 MHz < BWChannel_CA ≤ 800 MHz | 2 | 1 |
| C | 800 MHz < BWChannel_CA ≤ 1200 MHz | 3 |  |
| D | 200 MHz < BWChannel_CA ≤ 400 MHz | 2 | 2 |
| E | 400 MHz < BWChannel_CA ≤ 600 MHz | 3 |  |
| F | 600 MHz < BWChannel_CA ≤ 800 MHz | 4 |  |
| G | 100 MHz < BWChannel_CA ≤ 200 MHz | 2 | 3 |
| H | 200 MHz < BWChannel_CA ≤ 300 MHz | 3 |  |
| I | 300 MHz < BWChannel_CA ≤ 400 MHz | 4 |  |
| J | 400 MHz < BWChannel_CA ≤ 500 MHz | 5 |  |
| K | 500 MHz < BWChannel_CA ≤ 600 MHz | 6 |  |
| L | 600 MHz < BWChannel_CA ≤ 700 MHz | 7 |  |
| M | 700 MHz < BWChannel_CA ≤ 800 MHz | 8 |  |
| O | 100 MHz ≤ BWChannel_CA ≤ 200 MHz | 2 | 4 |
| P | 150 MHz ≤ BWChannel_CA ≤ 300 MHz | 3 |  |
| Q | 200 MHz ≤ BWChannel_CA ≤ 400 MHz | 4 |  |
| NOTE 1: Maximum supported component carrier bandwidths for fallback groups 1, 2, 3 and 4 are 400 MHz, 200 MHz, 100 MHz and 100 MHz respectively. NOTE 2: It is mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order CA bandwidth class configuration that belong to a different fallback group. |  |  |  |

Table 2-1: Terminology used for CA bandwidth classes TS 38.101-2 [3] (for FR2)

Figure 2-2: Example of fallback combinations in FR2 CA across sub-blocks (based on [2])

In Figure 2-2, a simple chart showing most of the fallbacks from a band combination comprising of two sub-blocks H and I respectively are shown. The intent of the RAN4 in only allowing direct fallback supported by the UE from P1,1 to   P2,6 or P2,7 is the following:

- Reducing the amount of testing combinations for the FR2 CA cases where fallback combinations may potentially result in several tens of combinations (here is simplistic example where there are less than 20 but, in some cases, there may be few hundred easily [2])

- Each of the fallback cases for the intra-band non-contiguous sub-blocks result in additional work in RAN4 to formalize requirements.

- The RAN4 requirements for each of the fallbacks will have to be specifically worked out and documented to the RAN4 specification, which already has very lengthy table and it is rather impossible to document all the possible combinations. See Annex for the FR2 list comprising of only 2 bands [4].

RAN4 has discussed the issue and have agreed the following in [3]. Now the UEs does not have to support all the fallback combinations.

## 4.2 Applicability of minimum requirements a) In this specification the Minimum Requirements are specified as general requirements and additional requirements. Where the Requirement is specified as a general requirement, the requirement is mandated to be met in all scenarios b) For specific scenarios for which an additional requirement is specified, in addition to meeting the general requirement, the UE is mandated to meet the additional requirements. c) The spurious emissions power requirements are for the long-term average of the power. For the purpose of reducing measurement uncertainty it is acceptable to average the measured power over a period of time sufficient to reduce the uncertainty due to the statistical nature of the signal d) All the requirements for intra-band contiguous and non-contiguous CA apply under the assumption of the same slot format indicated by TDD-UL-DL-Config in the PCell and SCells for NR SA. A terminal which supports CA or DC configurations, which include FR2 intra-band CA combinations with multiple subblocks, where at least one of the subblocks consists of a contiguous CA combination, is not required to support all possible fallback combinations but can directly fall back to a single FR2 carrier. Deactivating carriers within the CA or DC combination is still possible.

Observation 1: RAN4 decision affects the fallback support for (only) the FR2 intra-band CA cases.

# 3	RAN2 Impacts of RAN4 FR2 fallback decision

## 3.1	Current way handling of band combinations and feature sets

The handling of featureSets within UE capabilities is described in subclause 5.6.1.4 of TS38.331, as shown in excerpt below.

#### 5.6.1.4 Setting band combinations, feature set combinations and feature sets supported by the UE The UE invokes the procedures in this clause if the NR or E-UTRA network requests UE capabilities for nr, eutra-nr or eutra. This procedure is invoked once per requested rat-Type (see clause 5.6.1.3 for capability enquiry by the NR network; see TS 36.331 [10], clause 5.6.3.3 for capability enquiry by the E-UTRA network). The UE shall ensure that the feature set IDs are consistent across feature sets, feature set combinations and band combinations in all three UE capability containers that the network queries with the same fields with the same values, i.e. UE-CapabilityRequestFilterNR and fields in UECapabilityEnquiry message (i.e. requestedFreqBandsNR-MRDC, requestedCapabilityNR and eutra-nr-only flag) as defined in TS 36.331, where applicable. NOTE 1: Capability enquiry without frequencyBandListFilter is not supported. NOTE 2: In EN-DC, the gNB needs the capabilities for RAT types nr and eutra-nr and it uses the featureSets in the UE-NR-Capability together with the featureSetCombinations in the UE-MRDC-Capability to determine the NR UE capabilities for the supported MRDC band combinations. Similarly, the eNB needs the capabilities for RAT types eutra and eutra-nr and it uses the featureSetsEUTRA-r15 in the UE-EUTRA-Capability together with the featureSetCombinations in the UE-MRDC-Capability to determine the E-UTRA UE capabilities for the supported MRDC band combinations. Hence, the IDs used in the featureSets must match the IDs referred to in featureSetCombinations across all three containers. The requirement on consistency implies that there are no undefined feature sets and feature set combinations. NOTE 3: If the UE cannot include all feature sets and feature set combinations due to message size or list size constraints, it is up to UE implementation which feature sets and feature set combinations it prioritizes.

The UE shall: 1> compile a list of "candidate band combinations" only consisting of bands included in frequencyBandListFilter, and prioritized in the order of frequencyBandListFilter (i.e. first include band combinations containing the first-listed band, then include remaining band combinations containing the second-listed band, and so on), where for each band in the band combination, the parameters of the band do not exceed maxBandwidthRequestedDL, maxBandwidthRequestedUL, maxCarriersRequestedDL, maxCarriersRequestedUL, ca-BandwidthClassDL-EUTRA or ca-BandwidthClassUL-EUTRA, whichever are received; 1> for each band combination included in the list of "candidate band combinations": 2> if the network (E-UTRA) included the eutra-nr-only field, or 2> if the requested rat-Type is eutra: 3> remove the NR-only band combination from the list of "candidate band combinations"; NOTE 4: The (E-UTRA) network may request capabilities for nr but indicate with the eutra-nr-only flag that the UE shall not include any NR band combinations in the UE-NR-Capability. In this case the procedural text above removes all NR-only band combinations from the candidate list and thereby also avoids inclusion of corresponding feature set combinations and feature sets below. 2> if it is regarded as a fallback band combination with the same capabilities of another band combination included in the list of "candidate band combinations": 3> remove the band combination from the list of "candidate band combinations"; NOTE 5: Even if the network requests (only) capabilities for nr, it may include E-UTRA band numbers in the frequencyBandListFilter to ensure that the UE includes all necessary feature sets needed for subsequently requested eutra-nr capabilities. At this point of the procedure the list of "candidate band combinations" contains all NR- and/or E-UTRA-NR band combinations that match the filter (frequencyBandListFilter) provided by the NW and that match the eutra-nr-only flag (if RAT-Type nr is requested by E-UTRA). In the following, this candidate list is used to derive the band combinations, feature set combinations and feature sets to be reported in the requested capability container. 1> if the requested rat-Type is nr: 2> include into supportedBandCombinationList as many NR-only band combinations as possible from the list of "candidate band combinations", starting from the first entry; 3> if srs-SwitchingTimeRequest is received: 4> if SRS carrier switching is supported; 5> include srs-SwitchingTimesListNR for each band combination; 4> set srs-SwitchingTimeRequested to true; 2> include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList according to the previous; 2> compile a list of "candidate feature set combinations" referenced from the list of "candidate band combinations" excluding entries (rows in feature set combinations) for fallback band combinations with same or lower capabilities; NOTE 6: This list of "candidate feature set combinations" contains the feature set combinations used for NR-only as well as E-UTRA-NR band combinations. It is used to derive a list of NR feature sets referred to from the feature set combinations in the UE-NR-Capability and from the feature set combinations in a UE-MRDC-Capability container. 2> include into featureSets the feature sets referenced from the "candidate feature set combinations" excluding entries (feature sets per CC) for fallback band combinations with same or lower capabilities and may exclude the feature sets with the parameters that exceed any of maxBandwidthRequestedDL, maxBandwidthRequestedUL, maxCarriersRequestedDL or maxCarriersRequestedUL, whichever are received; 1> else, if the requested rat-Type is eutra-nr: 2> include into supportedBandCombinationList and/or supportedBandCombinationListNEDC-Only as many E-UTRA-NR band combinations as possible from the list of "candidate band combinations", starting from the first entry; 3> if srs-SwitchingTimeRequest is received: 4> if SRS carrier switching is supported; 5> include srs-SwitchingTimesListNR and srs-SwitchingTimesListEUTRA for each band combination; 4> set srs-SwitchingTimeRequested to true; 2> include, into featureSetCombinations, the feature set combinations referenced from the supported band combinations as included in supportedBandCombinationList according to the previous; 1> else (if the requested rat-Type is eutra): 2> compile a list of "candidate feature set combinations" referenced from the list of "candidate band combinations" excluding entries (rows in feature set combinations) for fallback band combinations with same or lower capabilities; NOTE 7: This list of "candidate feature set combinations" contains the feature set combinations used for E-UTRA-NR band combinations. It is used to derive a list of E-UTRA feature sets referred to from the feature set combinations in a UE-MRDC-Capability container. 2> include into featureSetsEUTRA the feature sets referenced from the "candidate feature set combinations" excluding entries (feature sets per CC) for fallback band combinations with same or lower capabilities and where the parameters do not exceed ca-BandwidthClassDL-EUTRA and ca-BandwidthClassUL-EUTRA, whichever are received; 1> if capabilityRequestFilterCommon is received: 2> include and/or remove band combinations from the list of "candidate band combinations" in accordance with the given filter criteria in capabilityRequestFilterCommon; 1> include the received frequencyBandListFilter in the field appliedFreqBandListFilter of the requested UE capability, except if the requested rat-Type is nr and the network included the eutra-nr-only field; 1> if the network included ue-CapabilityEnquiryExtension: 2> include the received ue-CapabilityEnquiryExtension in the field receivedFilters;

As can be seen from the procedural text, the falback band combinations are specifically omitted from the UE capabilities as per the RAN2 decisions on how to design the Rel-15 capabilities. This also includes reporting of the featureSets for the fallback BCs.

Observation 1: Fallback band combinations are not reported by the UE.

Observation 2:  Fallback feature sets are not reported by the UE.

Hence, there is a need to understand what the impacts to RAN2 are as the RAN4 decision is touching one of the cornerstones of the RAN2 capability signalling design.

## 3.2	List of possible impacts

There are several non-trivial impacts for implementing the RAN4 LS giving rise to multiple solution options:

| Solution direction | Pros | Cons |
| --- | --- | --- |
| Option 1: UE reports all the fallbacks that it does not support (i.e. incapability indication) | (+) Increase in capability container size is reduced provided that these indications can be limited (+) Approach is well known and simple to understand (i.e. similar reporting as in LTE) | (-) Introduces new UE behavior and requirement for network (-) Works only if the non supported cases are limited but explodes size if the UE only supports limited number of fallbacks (see Annex A) |
| Option 2: UE reports the fallbacks it does not support outside the current capability framework (e.g. new procedure or new UE “incapabilities”) | (+) Specification wise this does not cause non-backward compatible behavior i.e. existing specification text is the norm rather than the exception | (-) Introduces new UE behavior and requirement for network (-) Overhead of additional procedure |
| Option 3: Specific criteria sent with the problematic band combination allowing network to figure out the unsupported cases | (+) Low impact on capability container (+) Tied to certain band combinations only | (-) Introduces new UE behavior and requirement for network (-) Difficult to interpret, chances of misinterpretation will lead to radio link failures during reconfiguration |

As can be seen, no matter what there are impacts to RAN2 for both UEs and networks, and the impacts are not trivial. Therefore, RAN2 needs to start doing the work as requested by RAN4 by identifying which option(s) could be feasible and which are not seen possible.

Observation 3: The impacts of the RAN4 decision to RAN2 specifications are non-trivial.

Proposal 1: RAN2 to discuss how to resolve the issue of fallback signalling for FR2 when multiple sub-blocks in a band participate in intra-band non-contiguous CA.

Proposal 2: RAN2 to discuss interoperability aspects between network and UE in signalling of fallback band combinations when multiple sub-blocks in a band participate in intra-band non-contiguous CA in FR2.

We would also note that upon reading the RAN4 discussion [1][2], this issue seems aimed at only FR2 band combinations (i.e. not to FR1) with a mix of intra-band contiguous and intra-band non-contigous sub-blocks (i.e. carrier groups). While this seems clear from the RAN4 LS, it would be good to verify this from them when sending the initial LS reply.

Proposal 3: RAN2 to discuss how to limit the issue to only FR2 intra-band combinations with multiple sub-blocks and verify that this is the only case where something needs to be considered.

# 4	Conclusion

Observation 1: Fallback band combinations are not reported by the UE.

Observation 2:  Fallback feature sets are not reported by the UE.

Observation 3: The impact to RAN2 specifications are non-trivial.

Proposal 1: RAN2 to discuss signalling of fallback band combinations when multiple sub-blocks in a band participate in intra-band non-contiguous CA in FR2.

Proposal 2: RAN2 to discuss interoperability aspects between network and UE in signalling of fallback band combinations when multiple sub-blocks in a band participate in intra-band non-contiguous CA in FR2.

Proposal 3: RAN2 to discuss how to limit the issue to only FR2 intra-band combinations with multiple sub-blocks and verify that this is the only case where something needs to be considered.

# 5	References

[1] R4-1910238: RAN4 CR for handling of fallbacks for combined contiguous and non-contiguous CA in FR2, Apple

[2] R4-1908027: Handling of fallbacks for combined contiguous and non-contiguous CA in FR2, Apple

[3] TS 38.102-2: User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone, 3GPP

[4] TS 38.102-3: User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios, 3GPP

[5] TS 38.331: Technical Specification Group Radio Access Network; NR; Radio Resource Control (RRC) protocol specification, 3GPP

[6] R4-1910238: LS on Handling of Fallbacks for combined contiguous and non-contiguous CA or DC configurations in FR2

# 7	Annex

### 5.5B.5	Inter-band EN-DC including FR2

Supported channel bandwidths for E-UTRA operating bands and CA configurations are defined in TS 36.101 [4] and for NR operating bands and CA configurations in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3.

#### 5.5B.5.1	Inter-band EN-DC configurations including FR2 (two bands)

Table 5.5B.5.1-1: Inter-band EN-DC configurations including FR2 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n257A DC_1A_n257D DC_1A_n257E DC_1A_n257F | DC_1A_n257A |
| DC_1A_n257G | DC_1A_n257A DC_1A_n257G |
| DC_1A_n257H | DC_1A_n257A DC_1A_n257G DC_1A_n257H |
| DC_1A_n257I | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I |
| DC_1A_n257J | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J |
| DC_1A_n257K | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K |
| DC_1A_n257L | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L |
| DC_1A_n257M | DC_1A_n257A DC_1A_n257G DC_1A_n257H DC_1A_n257I DC_1A_n257J DC_1A_n257K DC_1A_n257L DC_1A_n257M |
| DC_1A_n258A DC_1A_n258D | DC_1A_n258A DC_1A_n258D |
| DC_2A_n257A DC_2C_n257A | DC_2A_n257A |
| DC_2A_n257(2A) | DC_2A_n257A |
| DC_2A-2A_n257A | DC_2A_n257A |
| DC_2A_n258A | DC_2A_n258A |
| DC_2A_n260A DC_2A_n260G DC_2A_n260H DC_2A_n260I DC_2A_n260J DC_2A_n260K DC_2A_n260L DC_2A_n260M DC_2C_n260A | DC_2A_n260A |
| DC_2A_n260(2A) DC_2A_n260(3A) DC_2A_n260(4A) DC_2A_n260(5A) DC_2A_n260(6A) DC_2A_n260(7A) DC_2A_n260(8A) DC_2A_n260(2D) DC_2A_n260(2G) DC_2A_n260(3G) DC_2A_n260(4G) DC_2A_n260(2H) DC_2A_n260(2O) DC_2A_n260(3O) DC_2A_n260(4O) | DC_2A_n260 |
| DC_2A_n260G | DC_2A_n260G |
| DC_2A_n260(A-G) | DC_2A_n260A DC_2A_n260G |
| DC_2A_n260(2A-2G) | DC_2A_n260A DC_2A_n260G |
| DC_2A_n260H | DC_2A_n260G DC_2A_n260H |
| DC_2A_n260(2H) | DC_2A_n260G DC_2A_n260H |
| DC_2A_n260(2A-2H) | DC_2A_n260A DC_2A_n260G DC_2A_n260H |
| DC_2A_n260O | DC_2A_n260A DC_2A_n260O |
| DC_2A_n260P | DC_2A_n260A DC_2A_n260O DC_2A_n260P |
| DC_2A_n260Q | DC_2A_n260A DC_2A_n260O DC_2A_n260P DC_2A_n260Q |
| DC_2A_n260(A-P-Q) | DC_2A_n260A DC_2A_n260O DC_2A_n260P DC_2A_n260Q |
| DC_2A_n260(3A-O-P) | DC_2A_n260A DC_2A_n260O DC_2A_n260P |
| DC_2A_n260(4A-2O) | DC_2A_n260A DC_2A_n260O |
| DC_2A_n260(2G) | DC_2A_n260G |
| DC_2A_n260(2A-G) | DC_2A_n260A DC_2A_n260G |
| DC_2A_n260(A-2G) | DC_2A_n260A DC_2A_n260G |
| DC_2A_n260(2A-H) | DC_2A_n260A DC_2A_n260G DC_2A_n260H |
| DC_2A_n260(A-2H) | DC_2A_n260A DC_2A_n260G DC_2A_n260H |
| DC_2A_n260(A-P) | DC_2A_n260A DC_2A_n260O DC_2A_n260P DC_2A_n260Q |
| DC_2A_n260(A-Q) | DC_2A_n260A DC_2A_n260O DC_2A_n260P DC_2A_n260Q |
| DC_2A_n260(P-Q) | DC_2A_n260A DC_2A_n260O DC_2A_n260P DC_2A_n260Q |
| DC_2A_n260(3A-O) | DC_2A_n260A DC_2A_n260O DC_2A_n260P |
| DC_2A_n260(3A-P) | DC_2A_n260A DC_2A_n260O DC_2A_n260P |
| DC_2A_n260(2A-O-P) | DC_2A_n260A DC_2A_n260O DC_2A_n260P |
| DC_2A_n260(4A-O) | DC_2A_n260A DC_2A_n260O |
| DC_2A_n260(3A-2O) | DC_2A_n260A DC_2A_n260O |
| DC_2A_n260(A-H) | DC_2A_n260A DC_2A_n260G DC_2A_n260H |
| DC_2A-2A_n260A DC_2A-2A_n260G DC_2A-2A_n260H DC_2A-2A_n260I DC_2A-2A_n260J DC_2A-2A_n260K DC_2A-2A_n260L DC_2A-2A_n260M | DC_2A_n260A |
| DC_2A_n261A DC_2A_n261(2A) DC_2A_n261(3A) DC_2A_n261(4A) | DC_2A_n261A |
| DC_2A_n261G | DC_2A_n261A DC_2A_n261G |
| DC_2A_n261H | DC_2A_n261A DC_2A_n261H DC_2A_n261G |
| DC_2A_n261I | DC_2A_n261A DC_2A_n261I DC_2A_n261H DC_2A_n261G |
| DC_2A_n261L | DC_2A_n261A DC_2A_n261I DC_2A_n261H DC_2A_n261G |
| DC_2A_n261M | DC_2A_n261A DC_2A_n261I DC_2A_n261H DC_2A_n261G |
| DC_2A_n261(A-H) | DC_2A_n261A DC_2A_n261H DC_2A_n261G |
| DC_2A_n261(G-H) | DC_2A_n261A DC_2A_n261H DC_2A_n261G |
| DC_2A_n261(2H) | DC_2A_n261H DC_2A_n261G |
| DC_2A_n261(A-G-H) | DC_2A_n261A DC_2A_n261H DC_2A_n261G |
| DC_2A_n261(A-G-I) | DC_2A_n261A DC_2A_n261I DC_2A_n261H DC_2A_n261G |
| DC_2A_n261(H-I) | DC_2A_n261A DC_2A_n261I DC_2A_n261H DC_2A_n261G |
| DC_2A_n261(A-G) | DC_2A_n261A DC_2A_n261G |
| DC_2A_n261(A-I) | DC_2A_n261A DC_2A_n261I DC_2A_n261H DC_2A_n261G |
| DC_2A_n261(G-I) | DC_2A_n261A DC_2A_n261I DC_2A_n261H |
| DC_3A_n257A DC_3A_n257B DC_3A_n257C DC_3A_n257D DC_3A_n257E DC_3A_n257F | DC_3A_n257A DC_3A_n257B |
| DC_3A_n257G | DC_3A_n257A DC_3A_n257G |
| DC_3A_n257H | DC_3A_n257A DC_3A_n257G DC_3A_n257H |
| DC_3A_n257I | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I |
| DC_3A_n257J | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J |
| DC_3A_n257K | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K |
| DC_3A_n257L | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L |
| DC_3A_n257M | DC_3A_n257A DC_3A_n257G DC_3A_n257H DC_3A_n257I DC_3A_n257J DC_3A_n257K DC_3A_n257L DC_3A_n257M |
| DC_3A_n258A DC_3A_n258B DC_3A_n258C DC_3A_n258D DC_3A_n258E DC_3A_n258F DC_3A_n258G DC_3A_n258H DC_3A_n258I DC_3A_n258J DC_3A_n258K DC_3A_n258L DC_3A_n258M DC_3C_n257A DC_3C_n257D DC_3C_n257E DC_3C_n257F DC_3C_n257G DC_3C_n257H DC_3C_n257I DC_3C_n257J DC_3C_n257K DC_3C_n257L DC_3C_n257M | DC_3A_n258A DC_3C_n257A |
| DC_3A-3A_n257A | DC_3A_n257A |
| DC_4A_n260(2A) DC_4A_n260(3A) DC_4A_n260(4A) DC_4A_n260(5A) DC_4A_n260(6A) DC_4A_n260(7A) DC_4A_n260(8A) DC_4A_n260(2D) DC_4A_n260(2G) DC_4A_n260(3G) DC_4A_n260(4G) DC_4A_n260(2H) DC_4A_n260(2O) DC_4A_n260(3O) DC_4A_n260(4O) DC_4A_n260(A-D) DC_4A_n260(2A-D) DC_4A_n260(A-O) DC_4A_n260(2A-O) DC_4A_n260(A-D-O) DC_4A_n260(2A-D-O) DC_4A_n260(A-2O) DC_4A_n260(D-2O) DC_4A_n260(A-D-2O) DC_4A_n260(2A-D-2O) DC_4A_n260(A-2D) DC_4A_n260(2A-2D) DC_4A_n260(A-P) DC_4A_n260(2A-P) DC_4A_n260(A-2P) DC_4A_n260(2A-2P) DC_4A_n260(A-G) DC_4A_n260(2A-G) DC_4A_n260(A-2G) DC_4A_n260(2A-2G) DC_4A_n260(G-O) DC_4A_n260(2G-O) DC_4A_n260(A-G-O) DC_4A_n260(2A-G-O) DC_4A_n260(A-2G-O) DC_4A_n260(2A-2G-O) DC_4A_n260(A-H) DC_4A_n260(A-2H) DC_4A_n260(2A-H) DC_4A_n260(2A-2H) DC_4A_n260(2A-2O) DC_4A_n260(A-3O) DC_4A_n260(2A-3O) DC_4A_n260(A-4O) DC_4A_n260(2A-4O) DC_4A_n260(3A-O) DC_4A_n260(3A-2O) DC_4A_n260(3A-3O) DC_4A_n260(3A-G) DC_4A_n260(3A-2G) DC_4A_n260(4A-G) DC_4A_n260(4A-2G) DC_4A_n260(4A-O) DC_4A_n260(4A-2O) DC_4A_n260(D-2G) DC_4A_n260(2D-O) DC_4A_n260(G-2O) DC_4A_n260(2G-2O) DC_4A_n260(G-3O) DC_4A_n260(2G-3O) DC_4A_n260(G-4O) DC_4A_n260(2G-4O) DC_4A_n260(3G-O) DC_4A_n260(4G-O) DC_4A_n260(H-O) DC_4A_n260(2H-O) | DC_4A_n260A |
| DC_4A_n260G | DC_4A_n260A |
| DC_4A_n260G | DC_4A_n260G |
| DC_4A_n260(2A-2G) | DC_4A_n260A |
| DC_4A_n260(2A-2G) | DC_4A_n260G |
| DC_4A_n260H | DC_4A_n260A |
| DC_4A_n260H | DC_4A_n260G |
| DC_4A_n260H | DC_4A_n260H |
| DC_4A_n260(2H) | DC_4A_n260G |
| DC_4A_n260(2H) | DC_4A_n260H |
| DC_4A_n260(2A-2H) | DC_4A_n260G |
| DC_4A_n260(2A-2H) | DC_4A_n260H |
| DC_4A_n260O | DC_4A_n260A |
| DC_4A_n260O | DC_4A_n260O |
| DC_4A_n260P | DC_4A_n260A |
| DC_4A_n260P | DC_4A_n260O |
| DC_4A_n260P | DC_4A_n260P |
| DC_4A_n260Q | DC_4A_n260A |
| DC_4A_n260Q | DC_4A_n260O |
| DC_4A_n260Q | DC_4A_n260P |
| DC_4A_n260Q | DC_4A_n260Q |
| DC_4A_n260(A-P-Q) | DC_4A_n260A |
| DC_4A_n260(A-P-Q) | DC_4A_n260O |
| DC_4A_n260(A-P-Q)_ | DC_4A_n260P |
| DC_4A_n260(A-P-Q) | DC_4A_n260Q |
| DC_4A_n260(3A-O-P) | DC_4A_n260A |
| DC_4A_n260(3A-O-P) | DC_4A_n260O |
| DC_4A_n260(3A-O-P) | DC_4A_n260P |
| DC_4A_n260(4A-2O) | DC_4A_n260O |
| DC_4A_n261(2H) | DC_4A_n261A |
| DC_4A_n261(2I) | DC_4A_n261A |
| DC_4A_n261(A-D) | DC_4A_n261A |
| DC_4A_n261(A-H) | DC_4A_n261A |
| DC_4A_n261(A-2H) | DC_4A_n261A |
| DC_4A_n261(A-D-H) | DC_4A_n261A |
| DC_4A_n261(A-G) | DC_4A_n261A |
| DC_4A_n261(A-G-H) | DC_4A_n261A |
| DC_4A_n261(A-I) | DC_4A_n261A |
| DC_4A_n261(A-2I) | DC_4A_n261A |
| DC_4A_n261(G-I) | DC_4A_n261A |
| DC_4A_n261(A-G-I) | DC_4A_n261A |
| DC_4A_n261(A-H-I) | DC_4A_n261A |
| DC_4A_n261(G-H) | DC_4A_n261A |
| DC_4A_n261(H-I) | DC_4A_n261A |
| DC_4A_n261(4A) | DC_4A_n261A |
| DC_4A_n261G | DC_4A_n261G |
| DC_4A_n261H | DC_4A_n261H |
| DC_4A_n261H | DC_4A_n261G |
| DC_4A_n261I | DC_4A_n261I |
| DC_4A_n261I | DC_4A_n261H |
| DC_4A_n261I | DC_4A_n261G |
| DC_4A_n261M | DC_4A_n261A |
| DC_4A_n261M | DC_4A_n261I |
| DC_4A_n261M | DC_4A_n261H |
| DC_4A_n261M | DC_4A_n261G |
| DC_4A_n261(A-H) | DC_4A_n261H |
| DC_4A_n261(A-H) | DC_4A_n261G |
| DC_4A_n261(G-H) | DC_4A_n261H |
| DC_4A_n261(G-H) | DC_4A_n261G |
| DC_4A_n261(2H) | DC_4A_n261H |
| DC_4A_n261(2H) | DC_4A_n261G |
| DC_4A_n261(A-G-H) | DC_4A_n261H |
| DC_4A_n261(A-G-H) | DC_4A_n261G |
| DC_4A_n261(A-G-I) | DC_4A_n261I |
| DC_4A_n261(A-G-I) | DC_4A_n261H |
| DC_4A_n261(A-G-I) | DC_4A_n261G |
| DC_4A_n261(H-I) | DC_4A_n261I |
| DC_4A_n261(H-I) | DC_4A_n261H |
| DC_4A_n261(H-I) | DC_4A_n261G |
| DC_4A_n260A | DC_4A_n260A |
| DC_4A_n260(2A-G) | DC_4A_n260G |
| DC_4A_n260(A-2G) | DC_4A_n260G |
| DC_4A_n260(2A-H) | DC_4A_n260G |
| DC_4A_n260(A-2H) | DC_4A_n260G |
| DC_4A_n260(2A-H) | DC_4A_n260H |
| DC_4A_n260(A-2H) | DC_4A_n260H |
| DC_4A_n260(A-Q) | DC_4A_n260A |
| DC_4A_n260(P-Q) | DC_4A_n260A |
| DC_4A_n260(A-P) | DC_4A_n260O |
| DC_4A_n260(A-Q) | DC_4A_n260O |
| DC_4A_n260(P-Q) | DC_4A_n260O |
| DC_4A_n260(A-P) | DC_4A_n260P |
| DC_4A_n260(A-Q) | DC_4A_n260P |
| DC_4A_n260(P-Q) | DC_4A_n260P |
| DC_4A_n260(A-P) | DC_4A_n260Q |
| DC_4A_n260(A-Q) | DC_4A_n260Q |
| DC_4A_n260(P-Q) | DC_4A_n260Q |
| DC_4A_n260(3A-P) | DC_4A_n260A |
| DC_4A_n260(2A-O-P) | DC_4A_n260A |
| DC_4A_n260(3A-O) | DC_4A_n260O |
| DC_4A_n260(3A-P) | DC_4A_n260O |
| DC_4A_n260(2A-O-P) | DC_4A_n260A |
| DC_4A_n260(3A-O) | DC_4A_n260O |
| DC_4A_n260(3A-P) | DC_4A_n260O |
| DC_4A_n260(2A-O-P) | DC_4A_n260O |
| DC_4A_n260(3A-P) | DC_4A_n260P |
| DC_4A_n260(2A-O-P) | DC_4A_n260P |
| DC_4A_n260(4A-O) | DC_4A_n260O |
| DC_4A_n260(A-2O) | DC_4A_n260O |
| DC_4A_n260(A-O-P) | DC_4A_n260A |
| DC_4A_n260(A-O-P) | DC_4A_n260O |
| DC_4A_n260(A-O-P) | DC_4A_n260P |
| DC_4A_n260(2A-O-P) | DC_4A_n260A |
| DC_4A_n260(4A-O) | DC_4A_n260O |
| DC_4A_n260(A-2O) | DC_4A_n260O |
| DC_4A_n260(A-O-P) | DC_4A_n260A |
| DC_4A_n260(A-O-P) | DC_4A_n260O |
| DC_4A_n260(A-O-P) | DC_4A_n260P |
| DC_4A_n261A | DC_4A_n261A |
| DC_4A_n261A | DC_4A_n261G |
| DC_4A_n261(2A) | DC_4A_n261A |
| DC_4A_n261(3A) | DC_4A_n261A |
| DC_4A_n261D | DC_4A_n261A |
| DC_4A_n261(D-H) | DC_4A_n261A |
| DC_4A_n261L | DC_4A_n261A |
| DC_4A_n261L | DC_4A_n261I |
| DC_4A_n261L | DC_4A_n261H |
| DC_4A_n261L | DC_4A_n261G |
| DC_4A_n261(A-G) | DC_4A_n261H |
| DC_4A_n261(A-G) | DC_4A_n261G |
| DC_4A_n261(A-I) | DC_4A_n261I |
| DC_4A_n261(G-I) | DC_4A_n261I |
| DC_4A_n261(A-I) | DC_4A_n261H |
| DC_4A_n261(G-I) | DC_4A_n261H |
| DC_4A_n261(A-I) | DC_4A_n261G |
| DC_4A_n261(G-I) | DC_4A_n261G |
| DC_5A_n257A DC_5A_n257D DC_5A_n257E DC_5A_n257F DC_5A_n257G DC_5A_n257H DC_5A_n257I DC_5A_n257J DC_5A_n257K DC_5A_n257L DC_5A_n257M DC_5B_n257A | DC_5A_n257A DC_5B_n257A |
| DC_5A-5A_n257A | DC_5A_n257A |
| DC_5A_n258A | DC_5A_n258A |
| DC_5A_n260A DC_5A_n260B DC_5A_n260C DC_5A_n260D DC_5A_n260E DC_5A_n260F DC_5A_n260G DC_5A_n260H DC_5A_n260I DC_5A_n260J DC_5A_n260K DC_5A_n260L DC_5A_n260M DC_5A_n260O DC_5A_n260P DC_5A_n260Q DC_5B_n260A | DC_5A_n260A DC_5B_n260A |
| DC_5A_n260(2A) DC_5A_n260(3A) DC_5A_n260(4A) DC_5A_260(5A) DC_5A_260(6A) DC_5A_260(7A) DC_5A_260(8A) DC_5A_260(9A) DC_5A_260(10A) DC_5A_n260(A-I) DC_5A_n260(A-P-Q) DC_5A_n260(3A-O-P) DC_5A_n260(D-G) DC_5A_n260(D-H) DC_5A_n260(D-I) DC_5A_n260(D-O) DC_5A_n260(D-P) DC_5A_n260(D-Q) DC_5A_n260(E-O) DC_5A_n260(E-P) DC_5A_n260(E-Q) DC_5A_n260(G-I) DC_5A_n260(2G) DC_5A_n260(2H) DC_5A_n260(2O) DC_5A_n260(3O) DC_5A_n260(4O) DC_5A_n260(2P) DC_5A_n260(3P) DC_5A_n260(4P) DC_5A_n260(2A-O) DC_5A_n260(A-2O) DC_5A_n260(2A-G) DC_5A_n260(A-2G) DC_5A_n260(2A-2G) DC_5A_n260(2G-O) DC_5A_n260(2A-2G-O) DC_5A_n260(A-2H) DC_5A_n260(2A-H) DC_5A_n260(2A-2H) DC_5A_n260(2A-2O) DC_5A_n260(2A-3O) DC_5A_n260(A-4O) DC_5A_n260(2A-4O) DC_5A_n260(3A-2O) DC_5A_n260(3A-2G) DC_5A_n260(4A-G) DC_5A_n260(4A-2G) DC_5A_n260(4A-O) DC_5A_n260(4A-2O) DC_5A_n260(A-O) DC_5A_n260(A-G) DC_5A_n260(G-O) DC_5A_n260(A-G-O) DC_5A_n260(2A-G-O) DC_5A_n260(A-2G-O) DC_5A_n260(A-H) DC_5A_n260(A-3O) DC_5A_n260(3A-O) DC_5A_n260(3A-G) DC_5A_n260(2D) DC_5A_n260(3G) DC_5A_n260(4G) DC_5A_n260(2H) DC_5A_n260(A-D) DC_5A_n260(2A-D) DC_5A_n260(A-D-O) DC_5A_n260(2A-D-O) DC_5A_n260(D-2O) DC_5A_n260(A-D-2O) DC_5A_n260(2A-D-2O) DC_5A_n260(A-2D) DC_5A_n260(2A-2D) DC_5A_n260(A-P) DC_5A_n260(2A-P) DC_5A_n260(A-2P) DC_5A_n260(2A-2P) DC_5A_n260(3A-3O) DC_5A_n260(D-2G) DC_5A_n260(2D-O) DC_5A_n260(G-2O) DC_5A_n260(2G-2O) DC_5A_n260(G-3O) DC_5A_n260(2G-3O) DC_5A_n260(G-4O) DC_5A_n260(2G-4O) DC_5A_n260(3G-O) DC_5A_n260(4G-O) DC_5A_n260(H-O) DC_5A_n260(2H-O) | DC_5A_n260A |
| DC_5A-5A_n260A | DC_5A_n260A |
| DC_5A-n260(2A-2G-2O) | DC_5A-n260A |
| DC_5A-n260(6A-2O) | DC_5A-n260A |
| DC_5A-n260(8A-2O) | DC_5A-n260A |
| DC_5A-n260(2A-2O-2P) | DC_5A-n260A |
| DC_5A-n260(6A-3O) | DC_5A-n260A |
| DC_5A-n260(2A-4O) | DC_5A-n260A |
| DC_5A-n260(4A-4O) | DC_5A-n260A |
| DC_5A-n260(6A-2P) | DC_5A-n260A |
| DC_5A-n260(2O-2P) | DC_5A-n260A |
| DC_5A-n260(4P) | DC_5A-n260A |
| DC_5A-n260(2A-4P) | DC_5A-n260A |
| DC_5A-n260(4A-2Q) | DC_5A-n260A |
| DC_5A-n260(2A-2O-2Q) | DC_5A-n260A |
| DC_5A_n260G | DC_5A_n260G |
| DC_5A_n260(A-G) | DC_5A_n260G |
| DC_5A_n260(2G) | DC_5A_n260G |
| DC_5A_n260(2A-2G) | DC_5A_n260G |
| DC_5A_n260H | DC_5A_n260G |
| DC_5A_n260H | DC_5A_n260H |
| DC_5A_n260(2H) | DC_5A_n260G |
| DC_5A_n260(2H) | DC_5A_n260H |
| DC_5A_n260(2A-2H) | DC_5A_n260G |
| DC_5A_n260(2A-2H) | DC_5A_n260H |
| DC_5A_n260O | DC_5A_n260O |
| DC_5A_n260P | DC_5A_n260O |
| DC_5A_n260P | DC_5A_n260P |
| DC_5A_n260Q | DC_5A_n260O |
| DC_5A_n260Q | DC_5A_n260P |
| DC_5A_n260Q | DC_5A_n260Q |
| DC_5A_n260(A-P-Q) | DC_5A_n260O |
| DC_5A_n260(A-P-Q) | DC_5A_n260P |
| DC_5A_n260(A-P-Q) | DC_5A_n260Q |
| DC_5A_n260(3A-O-P) | DC_5A_n260O |
| DC_5A_n260(3A-O-P) | DC_5A_n260P |
| DC_5A_n260(4A-2O) | DC_5A_n260O |
| DC_5A_n260G | DC_5A_n260G |
| DC_5A_n260(2A-G) | DC_5A_n260G |
| DC_5A_n260(A-2G) | DC_5A_n260G |
| DC_5A_n260H | DC_5A_n260G |
| DC_5A_n260H | DC_5A_n260H |
| DC_5A_n260(2A-H) | DC_5A_n260G |
| DC_5A_n260(A-2H) | DC_5A_n260G |
| DC_5A_n260(2A-H) | DC_5A_n260H |
| DC_5A_n260(A-2H) | DC_5A_n260H |
| DC_5A_n260O | DC_5A_n260O |
| DC_5A_n260P | DC_5A_n260A |
| DC_5A_n260P | DC_5A_n260O |
| DC_5A_n260P | DC_5A_n260P |
| DC_5A_n260(A-P) | DC_5A_n260O |
| DC_5A_n260(A-Q) | DC_5A_n260O |
| DC_5A_n260(P-Q) | DC_5A_n260O |
| DC_5A_n260(A-P) | DC_5A_n260P |
| DC_5A_n260(A-Q) | DC_5A_n260P |
| DC_5A_n260(P-Q) | DC_5A_n260P |
| DC_5A_n260(A-Q) | DC_5A_n260Q |
| DC_5A_n260(P-Q) | DC_5A_n260Q |
| DC_5A_n260(3A-P) | DC_5A_n260A |
| DC_5A_n260(2A-O-P) | DC_5A_n260A |
| DC_5A_n260(3A-O) | DC_5A_n260O |
| DC_5A_n260(3A-P) | DC_5A_n260O |
| DC_5A_n260(2A-O) | DC_5A_n260O |
| DC_5A_n260(3A-P) | DC_5A_n260P |
| DC_5A_n260(2A-O-P) | DC_5A_n260P |
| DC_5A_n260(4A-O) | DC_5A_n260O |
| DC_5A_n260(3A-2O) | DC_5A_n260O |
| DC_5A_n261A DC_5A_n261B DC_5A_n261C DC_5A_n261D DC_5A_n261E DC_5A_n261F DC_5A_n261G DC_5A_n261H DC_5A_n261I DC_5A_n261J DC_5A_n261K DC_5A_n261L DC_5A_n261M DC_5A_n261O DC_5A_n261P DC_5A_n261Q | DC_5A_n261A |
| DC_5A_n261(2A) DC_5A_n261(3A) DC_5A_n261(4A) DC_5A_n261(D-G) DC_5A_n261(D-H) DC_5A_n261(D-I) DC_5A_n261(D-O) DC_5A_n261(D-P) DC_5A_n261(D-Q) DC_5A_n261(E-O) DC_5A_n261(E-P) DC_5A_n261(E-Q) DC_5A_n261(2H) DC_5A_n261(2I) DC_5A_n261(A-H) DC_5A_n261(A-I) DC_5A_n261(A-D) DC_5A_n261(A-D-H) DC_5A_n261(A-G) DC_5A_n261(A-G-H) DC_5A_n261(G-I) DC_5A_n261(A-G-I) DC_5A_n261(A-H-I) DC_5A_n261(G-H) DC_5A_n261(H-I) | DC_5A_n261A |
| DC_5A-n261(A-2D) | DC_5A-n261A |
| DC_5A-n261(A-2G-2O) | DC_5A-n261A |
| DC_5A-n261(A-3G-O) | DC_5A-n261A |
| DC_5A-n261(A-4G) | DC_5A-n261A |
| DC_5A-n261(A-2H) | DC_5A-n261A |
| DC_5A-n261(A-2I) | DC_5A-n261A |
| DC_5A-n261(A-4O) | DC_5A-n261A |
| DC_5A-n261(A-7O) | DC_5A-n261A |
| DC_5A-n261(A-2P) | DC_5A-n261A |
| DC_5A-n261(A-2Q) | DC_5A-n261A |
| DC_5A_n261G | DC_5A_n261G |
| DC_5A_n261H | DC_5A_n261H |
| DC_5A_n261H | DC_5A_n261G |
| DC_5A_n261I | DC_5A_n261I |
| DC_5A_n261I | DC_5A_n261H |
| DC_5A_n261I | DC_5A_n261G |
| DC_5A_n261M | DC_5A_n261I |
| DC_5A_n261M | DC_5A_n261H |
| DC_5A_n261M | DC_5A_n261G |
| DC_5A_n261(A-H) | DC_5A_n261H |
| DC_5A_n261(A-H) | DC_5A_n261G |
| DC_5A_n261(G-H) | DC_5A_n261H |
| DC_5A_n261(G-H) | DC_5A_n261G |
| DC_5A_n261(2H) | DC_5A_n261H |
| DC_5A_n261(2H) | DC_5A_n261G |
| DC_5A_n261(A-G-H) | DC_5A_n261H |
| DC_5A_n261(A-G-H) | DC_5A_n261G |
| DC_5A_n261(A-G-I) | DC_5A_n261I |
| DC_5A_n261(A-G-I) | DC_5A_n261H |
| DC_5A_n261(A-G-I) | DC_5A_n261G |
| DC_5A_n261(H-I) | DC_5A_n261I |
| DC_5A_n261(H-I) | DC_5A_n261H |
| DC_5A_n261(H-I) | DC_5A_n261G |
| DC_5A_n261(3A) | DC_5A_n261A |
| DC_5A_n261(2A) | DC_5A_n261A |
| DC_5A_n261G | DC_5A_n261G |
| DC_5A_n261H | DC_5A_n261H |
| DC_5A_n261H | DC_5A_n261G |
| DC_5A_n261L | DC_5A_n261I |
| DC_5A_n261L | DC_5A_n261H |
| DC_5A_n261L | DC_5A_n261G |
| DC_5A_n261(A-G) | DC_5A_n261G |
| DC_5A_n261(A-H) | DC_5A_n261H |
| DC_5A_n261(G-H) | DC_5A_n261H |
| DC_5A_n261(A-H) | DC_5A_n261G |
| DC_5A_n261(G-H) | DC_5A_n261G |
| DC_5A_n261(A-I) | DC_5A_n261I |
| DC_5A_n261(G-I) | DC_5A_n261I |
| DC_5A_n261(A-I) | DC_5A_n261H |
| DC_5A_n261(G-I) | DC_5A_n261H |
| DC_5A_n261(A-I) | DC_5A_n261G |
| DC_5A_n261(G-I) | DC_5A_n261G |
| DC_5A_n261I | DC_5A_n261I |
| DC_5A_n261I | DC_5A_n261H |
| DC_5A_n261I | DC_5A_n261G |
| DC_7A_n257A DC_7A_n257D DC_7A_n257E DC_7A_n257F DC_7A_n257G DC_7A_n257H DC_7A_n257I DC_7A_n257J DC_7A_n257K DC_7A_n257L DC_7A_n257M | DC_7A_n257A |
| DC_7A-7A_n257A DC_7A-7A_n257D DC_7A-7A_n257E DC_7A-7A_n257F DC_7A-7A_n257G DC_7A-7A_n257H DC_7A-7A_n257I DC_7A-7A_n257J DC_7A-7A_n257K DC_7A-7A_n257L DC_7A-7A_n257M | DC_7A_n257A |
| DC_7A_n258A DC_7A_n258B DC_7A_n258C DC_7A_n258D DC_7A_n258E DC_7A_n258F DC_7A_n258G DC_7A_n258H DC_7A_n258I DC_7A_n258J DC_7A_n258K DC_7A_n258L DC_7A_n258M | DC_7A_n258A |
| DC_8A_n257A DC_8A_n257D DC_8A_n257E DC_8A_n257F DC_8A_n257G DC_8A_n257H DC_8A_n257I DC_8A_n257J DC_8A_n257K DC_8A_n257L DC_8A_n257M | DC_8A_n257A |
| DC_8A_n258A | DC_8A_n258A |
| DC_11A_n257A DC_11A_n257D | DC_11A_n257A |
| DC_12A_n257A | DC_12A_n257A |
| DC_12A_n258A | DC_12A_n258A |
| DC_12A_n260A DC_12A_n260G DC_12A_n260H DC_12A_n260I DC_12A_n260J DC_12A_n260K DC_12A_n260L DC_12A_n260M | DC_12A_n260A |
| DC_12A_n260(A-I) DC_12A_n260(G-I) | DC_12A_n260A |
| DC_12A_n261A | DC_12A_n261A |
| DC_13A_n257A | DC_13A_n257A |
| DC_13A_n260A DC_13A_n260(2A) DC_13A_n260(3A) DC_13A_n260(4A) DC_13A_n260(5A) DC_13A_n260(6A) DC_13A_n260(7A) DC_13A_n260(8A) DC_13A_n260(2D) DC_13A_n260(2G) DC_13A_n260(3G) DC_13A_n260(4G) DC_13A_n260(2H) DC_13A_n260(2O) DC_13A_n260(3O) DC_13A_n260(4O) | DC_13A_n260A |
| DC_13A_n260G | DC_13A_n260A |
| DC_13A_n260G | DC_13A_n260G |
| DC_13A_n260(A-G) | DC_13A_n260A |
| DC_13A_n260(A-G) | DC_13A_n260G |
| DC_13A_n260(2G) | DC_13A_n260G |
| DC_13A_n260(2A-2G) | DC_13A_n260A |
| DC_13A_n260(2A-2G) | DC_13A_n260G |
| DC_13A_n260H | DC_13A_n260A |
| DC_13A_n260H | DC_13A_n260G |
| DC_13A_n260H | DC_13A_n260H |
| DC_13A_n260(2H) | DC_13A_n260G |
| DC_13A_n260(2H) | DC_13A_n260H |
| DC_13A_n260(2A-2H) | DC_13A_n260A |
| DC_13A_n260(2A-2H) | DC_13A_n260G |
| DC_13A_n260(2A-2H) | DC_13A_n260H |
| DC_13A_n260O | DC_13A_n260A |
| DC_13A_n260O | DC_13A_n260O |
| DC_13A_n260P | DC_13A_n260A |
| DC_13A_n260P | DC_13A_n260O |
| DC_13A_n260P | DC_13A_n260P |
| DC_13A_n260Q | DC_13A_n260A |
| DC_13A_n260Q | DC_13A_n260O |
| DC_13A_n260Q | DC_13A_n260P |
| DC_13A_n260Q | DC_13A_n260Q |
| DC_13A_n260(A-P-Q) | DC_13A_n260A |
| DC_13A_n260(A-P-Q) | DC_13A_n260O |
| DC_13A_n260(A-P-Q) | DC_13A_n260P |
| DC_13A_n260(A-P-Q) | DC_13A_n260Q |
| DC_13A_n260(3A-O-P) | DC_13A_n260A |
| DC_13A_n260(3A-O-P) | DC_13A_n260O |
| DC_13A_n260(3A-O-P) | DC_13A_n260P |
| DC_13A_n260(4A-2O) | DC_13A_n260A |
| DC_13A_n260(4A-2O) | DC_13A_n260O |
| DC_13A_n260(2A-G) | DC_13A_n260A |
| DC_13A_n260(A-2G) | DC_13A_n260A |
| DC_13A_n260(2A-G) | DC_13A_n260G |
| DC_13A_n260(A-2G) | DC_13A_n260G |
| DC_13A_n260(2A-H) | DC_13A_n260A |
| DC_13A_n260(A-2H) | DC_13A_n260A |
| DC_13A_n260(2A-H) | DC_13A_n260G |
| DC_13A_n260(A-2H) | DC_13A_n260G |
| DC_13A_n260(2A-H) | DC_13A_n260H |
| DC_13A_n260(A-2H) | DC_13A_n260H |
| DC_13A_n260(A-P) | DC_13A_n260A |
| DC_13A_n260(A-Q) | DC_13A_n260A |
| DC_13A_n260(P-Q) | DC_13A_n260A |
| DC_13A_n260(A-P) | DC_13A_n260O |
| DC_13A_n260(A-Q) | DC_13A_n260O |
| DC_13A_n260(P-Q) | DC_13A_n260O |
| DC_13A_n260(A-P) | DC_13A_n260P |
| DC_13A_n260(A-Q) | DC_13A_n260P |
| DC_13A_n260(P-Q) | DC_13A_n260P |
| DC_13A_n260(A-Q) | DC_13A_n260Q |
| DC_13A_n260(P-Q) | DC_13A_n260Q |
| DC_13A_n260(3A-O) | DC_13A_n260A |
| DC_13A_n260(3A-P) | DC_13A_n260A |
| DC_13A_n260(2A-O-P) | DC_13A_n260A |
| DC_13A_n260(3A-O) | DC_13A_n260O |
| DC_13A_n260(3A-P) | DC_13A_n260O |
| DC_13A_n260(2A-O-P | DC_13A_n260O |
| DC_13A_n260(3A-P) | DC_13A_n260P |
| DC_13A_n260(2A-O-P) | DC_13A_n260P |
| DC_13A_n260(4A-O) | DC_13A_n260A |
| DC_13A_n260(3A-2O) | DC_13A_n260A |
| DC_13A_n260(4A-O) | DC_13A_n260O |
| DC_13A_n260(3A-2O) | DC_13A_n260O |
| DC_13A_n261(2H) DC_13A_n261(2I) | DC_13A_n261A |
| DC_13A_n261(4A) | DC_13A_n261A |
| DC_13A_n261G | DC_13A_n261G |
| DC_13A_n261H | DC_13A_n261H |
| DC_13A_n261H | DC_13A_n261G |
| DC_13A_n261I | DC_13A_n261I |
| DC_13A_n261M | DC_13A_n261A |
| DC_13A_n261M | DC_13A_n261I |
| DC_13A_n261(A-H) | DC_13A_n261A |
| DC_13A_n261(A-H) | DC_13A_n261H |
| DC_13A_n261(A-H) | DC_13A_n261G |
| DC_13A_n261(G-H) | DC_13A_n261A |
| DC_13A_n261(G-H) | DC_13A_n261H |
| DC_13A_n261(G-H) | DC_13A_n261G |
| DC_13A_n261(2H) | DC_13A_n261H |
| DC_13A_n261(2H) | DC_13A_n261G |
| DC_13A_n261(A-G-H) | DC_13A_n261A |
| DC_13A_n261(A-G-H) | DC_13A_n261H |
| DC_13A_n261(A-G-H) | DC_13A_n261G |
| DC_13A_n261(A-G-I) | DC_13A_n261A |
| DC_13A_n261(A-G-I) | DC_13A_n261I |
| DC_13A_n261(A-G-I) | DC_13A_n261H |
| DC_13A_n261(A-G-I) | DC_13A_n261G |
| DC_13A_n261(H-I) | DC_13A_n261A |
| DC_13A_n261(H-I) | DC_13A_n261I |
| DC_13A_n261(H-I) | DC_13A_n261H |
| DC_13A_n261(H-I) | DC_13A_n261G |
| DC_13A_n261(3A) | DC_13A_n261A |
| DC_13A_n261A | DC_13A_n261A |
| DC_13A_n261(2A) | DC_13A_n261A |
| DC_13A_n261L | DC_13A_n261A |
| DC_13A_n261L | DC_13A_n261I |
| DC_13A_n261H | DC_13A_n261A |
| DC_13A_n261G | DC_13A_n261A |
| DC_13A_n261G | DC_13A_n261G |
| DC_13A_n261(A-G) | DC_13A_n261A |
| DC_13A_n261(A-G) | DC_13A_n261G |
| DC_13A_n261(A-I) | DC_13A_n261A |
| DC_13A_n261(G-I) | DC_13A_n261A |
| DC_13A_n261(A-I) | DC_13A_n261I |
| DC_13A_n261(G-I) | DC_13A_n261I |
| DC_13A_n261(A-I) | DC_13A_n261H |
| DC_13A_n261(G-I) | DC_13A_n261H |
| DC_13A_n261(A-I) | DC_13A_n261G |
| DC_13A_n261(G-I) | DC_13A_n261G |
| DC_13A_n261I | DC_13A_n261A |
| DC_13A_n261I | DC_13A_n261H |
| DC_13A_n261I | DC_13A_n261G |
| DC_18A_n257A DC_18A_n257D DC_18A_n257E DC_18A_n257F DC_18A_n257G DC_18A_n257H DC_18A_n257I DC_18A_n257J DC_18A_n257K DC_18A_n257L DC_18A_n257M | DC_18A_n257A |
| DC_19A_n257A DC_19A_n257D DC_19A_n257E DC_19A_n257F | DC_19A_n257A |
| DC_19A_n257G | DC_19A_n257A DC_19A_n257G |
| DC_19A_n257H | DC_19A_n257A DC_19A_n257G DC_19A_n257H |
| DC_19A_n257I | DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I |
| DC_19A_n257J | DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_19A_n257J |
| DC_19A_n257K | DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_19A_n257J DC_19A_n257K |
| DC_19A_n257L | DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_19A_n257J DC_19A_n257K DC_19A_n257L |
| DC_19A_n257M | DC_19A_n257A DC_19A_n257G DC_19A_n257H DC_19A_n257I DC_19A_n257J DC_19A_n257K DC_19A_n257L DC_19A_n257M |
| DC_20A_n258A | DC_20A_n258A |
| DC_21A_n257A DC_21A_n257D DC_21A_n257E DC_21A_n257F | DC_21A_n257A |
| DC_21A_n257G | DC_21A_n257A DC_21A_n257G |
| DC_21A_n257H | DC_21A_n257A DC_21A_n257G DC_21A_n257H |
| DC_21A_n257I | DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I |
| DC_21A_n257J | DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J |
| DC_21A_n257K | DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K |
| DC_21A_n257L | DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L |
| DC_21A_n257M | DC_21A_n257A DC_21A_n257G DC_21A_n257H DC_21A_n257I DC_21A_n257J DC_21A_n257K DC_21A_n257L DC_21A_n257M |
| DC_26A_n257A | DC_26A_n257A |
| DC_28A_n257A DC_28A_n257D DC_28A_n257E DC_28A_n257F | DC_28A_n257A |
| DC_28A_n258A DC_28A_n258B DC_28A_n258C DC_28A_n258D DC_28A_n258E DC_28A_n258F DC_28A_n258G DC_28A_n258H DC_28A_n258I DC_28A_n258J DC_28A_n258K DC_28A_n258L DC_28A_n258M | DC_28A_n258A |
| DC_30A_n260A DC_30A_n260G DC_30A_n260H DC_30A_n260I DC_30A_n260J DC_30A_n260K DC_30A_n260L DC_30A_n260M | DC_30A_n260A |
| DC_30A_n260(A-I) DC_30A_n260(G-I) | DC_30A_n260A |
| DC_39A_n258A | DC_39A_n258A |
| DC_41A_n257A DC_41A_n257D DC_41A_n257E DC_41A_n257F DC_41A_n257G DC_41A_n257H DC_41A_n257I DC_41A_n257J DC_41A_n257K DC_41A_n257L DC_41A_n257M DC_41C_n257A DC_41C_n257D DC_41C_n257E DC_41C_n257F DC_41C_n257G DC_41C_n257H DC_41C_n257I DC_41C_n257J DC_41C_n257K DC_41C_n257L DC_41C_n257M | DC_41A_n257A |
| DC_41A_n258A | DC_41A_n258A |
| DC_41C_n257A | DC_41C_n257A |
| DC_42A_n257A DC_42C_n257A | DC_42A_n257A |
| DC_42A_n257D | DC_42A_n257A DC_42A_n257D |
| DC_42A_n257E | DC_42A_n257A DC_42A_n257D DC_42A_n257E |
| DC_42A_n257F | DC_42A_n257A DC_42A_n257D DC_42A_n257E DC_42A_n257F |
| DC_42A_n257G | DC_42A_n257A DC_42A_n257G |
| DC_42A_n257H | DC_42A_n257A DC_42A_n257G DC_42A_n257H |
| DC_42A_n257I | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_42A_n257J | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J |
| DC_42A_n257K | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K |
| DC_42A_n257L | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L |
| DC_42A_n257M | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L DC_42A_n257M |
| DC_42C_n257D | DC_42C_n257A DC_42C_n257D |
| DC_42C_n257E | DC_42C_n257A DC_42C_n257D DC_42C_n257E |
| DC_42C_n257F | DC_42C_n257A DC_42C_n257D DC_42C_n257E DC_42C_n257F |
| DC_42C_n257G | DC_42A_n257A DC_42A_n257G |
| DC_42C_n257H | DC_42A_n257A DC_42A_n257G DC_42A_n257H |
| DC_42C_n257I | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_42C_n257J | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J |
| DC_42C_n257K | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K |
| DC_42C_n257L | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L |
| DC_42C_n257M | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L DC_42A_n257M |
| DC_42D_n257A | DC_42C_n257A |
| DC_42D_n257D | DC_42D_n257A DC_42D_n257D |
| DC_42D_n257E | DC_42D_n257A DC_42D_n257D DC_42D_n257E |
| DC_42D_n257F | DC_42D_n257A DC_42D_n257D DC_42D_n257E DC_42D_n257F |
| DC_42D_n257G | DC_42A_n257A DC_42A_n257G |
| DC_42D_n257H | DC_42A_n257A DC_42A_n257G DC_42A_n257H |
| DC_42D_n257I | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_42D_n257J | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J |
| DC_42D_n257K | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K |
| DC_42D_n257L | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L |
| DC_42D_n257M | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L DC_42A_n257M |
| DC_42E_n257A | DC_42A_n257A |
| DC_42E_n257D | DC_42E_n257A DC_42E_n257D |
| DC_42E_n257E | DC_42E_n257A DC_42E_n257D DC_42E_n257E |
| DC_42E_n257F | DC_42E_n257A DC_42E_n257D DC_42E_n257E DC_42E_n257F |
| DC_42E_n257G | DC_42A_n257A DC_42A_n257G |
| DC_42E_n257H | DC_42A_n257A DC_42A_n257G DC_42A_n257H |
| DC_42E_n257I | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I |
| DC_42E_n257J | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J |
| DC_42E_n257K | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K |
| DC_42E_n257L | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L |
| DC_42E_n257M | DC_42A_n257A DC_42A_n257G DC_42A_n257H DC_42A_n257I DC_42A_n257J DC_42A_n257K DC_42A_n257L DC_42A_n257M |
| DC_48A_n257A DC_48C_n257A | DC_48A_n257A DC_48C_n257A |
| DC_48A-48A_n257A | DC_48A_n257A |
| DC_48A_n260A DC_48C_n260A | DC_48A_n260A DC_48C_n260A |
| DC_48A-48A_n260A | DC_48A_n260A |
| DC_66A_n257A DC_66A_n257(2A) DC_66A_n257G DC_66A_n257H DC_66A_n257I DC_66A_n257J DC_66A_n257K DC_66A_n257L DC_66A_n257M DC_66C_n257A | DC_66A_n257A |
| DC_66A_n257(2A) | DC_66A_n257A |
| DC_66A-66A_n257A | DC_66A_n257A |
| DC_66A_n258A | DC_66A_n258A |
| DC_66A_n260A DC_66A_n260D DC_66A_n260E DC_66A_n260F DC_66A_n260G DC_66A_n260H DC_66A_n260I DC_66A_n260J DC_66A_n260K DC_66A_n260L DC_66A_n260M DC_66A_n260O DC_66A_n260P DC_66A_n260Q | DC_66A_n260A |
| DC_66A_n260(2A) DC_66A_n260(3A) DC_66A_n260(4A) DC_66A_n260(5A) DC_66A_n260(6A) DC_66A_n260(7A) DC_66A_n260(8A) DC_66A_n260(9A) DC_66A_n260(10A) DC_66A_n260(A-I) DC_66A_n260(D-G) DC_66A_n260(D-H) DC_66A_n260(D-I) DC_66A_n260(D-O) DC_66A_n260(D-P) DC_66A_n260(D-Q) DC_66A_n260(E-O) DC_66A_n260(E-P) DC_66A_n260(E-Q) DC_66A_n260(G-I) DC_66A_n260(2G) DC_66A_n260(2H) DC_66A_n260(2O) DC_66A_n260(3O) DC_66A_n260(4O) DC_66A_n260(2P) DC_66A_n260(3P) DC_66A_n260(4P) DC_66A_n260(2A-O) DC_66A_n260(A-2O) DC_66A_n260(2A-G) DC_66A_n260(A-2G) DC_66A_n260(2A-2G) DC_66A_n260(2G-O) DC_66A_n260(2A-2G-O) DC_66A_n260(A-2H) DC_66A_n260(2A-H) DC_66A_n260(2A-2H) DC_66A_n260(2A-2O) DC_66A_n260(2A-3O) DC_66A_n260(A-4O) DC_66A_n260(2A-4O) DC_66A_n260(3A-2O) DC_66A_n260(3A-2G) DC_66A_n260(4A-G) DC_66A_n260(4A-2G) DC_66A_n260(4A-O) DC_66A_n260(4A-2O) DC_66A_n260(A-O) DC_66A_n260(A-G) DC_66A_n260(G-O) DC_66A_n260(A-G-O) DC_66A_n260(2A-G-O) DC_66A_n260(A-2G-O) DC_66A_n260(A-H) DC_66A_n260(A-3O) DC_66A_n260(3A-O) DC_66A_n260(3A-P) DC_66A_n260(3A-G) DC_66A_n260(2D) DC_66A_n260(3G) DC_66A_n260(4G) DC_66A_n260(2H) DC_66A_n260(A-D) DC_66A_n260(2A-D) DC_66A_n260(A-D-O) DC_66A_n260(2A-D-O) DC_66A_n260(D-2O) DC_66A_n260(A-D-2O) DC_66A_n260(2A-D-2O) DC_66A_n260(2A-O-P) DC_66A_n260(A-2D) DC_66A_n260(2A-2D) DC_66A_n260(A-P) DC_66A_n260(2A-P) DC_66A_n260(A-2P) DC_66A_n260(2A-2P) DC_66A_n260(3A-3O) DC_66A_n260(D-2G) DC_66A_n260(2D-O) DC_66A_n260(G-2O) DC_66A_n260(2G-2O) DC_66A_n260(G-3O) DC_66A_n260(2G-3O) DC_66A_n260(G-4O) DC_66A_n260(2G-4O) DC_66A_n260(3G-O) DC_66A_n260(4G-O) DC_66A_n260(H-O) DC_66A_n260(2H-O) DC_66A-n260(4A-2G) DC_66A-n260(2A-2G-2O) DC_66A-n260(2A-2H) DC_66A-n260(2A-2O) DC_66A-n260(4A-2O) DC_66A-n260(6A-2O) DC_66A-n260(8A-2O) DC_66A-n260(2A-2O-2P) DC_66A-n260(6A-3O) DC_66A-n260(2A-4O) DC_66A-n260(4A-4O) DC_66A-n260(6A-2P) DC_66A-n260(2O-2P) DC_66A-n260(4P) DC_66A-n260(2A-4P) DC_66A-n260(4A-2Q) DC_66A-n260(2A-2O-2Q) DC_66A_n260(A-Q) DC_66A_n260(P-Q) | DC_66A_n260A DC_66A_n260G DC_66A_n260H DC_66A_n260O DC_66A_n260P DC_66A_n260Q |
| DC_66A-66A_n260A DC_66A-66A_n260G DC_66A-66A_n260H DC_66A-66A_n260I DC_66A-66A_n260J DC_66A-66A_n260K DC_66A-66A_n260L DC_66A-66A_n260M | DC_66A_n260A |
| DC_66A_n261A DC_66A_n261D DC_66A_n261E DC_66A_n261F DC_66A_n261G DC_66A_n261H DC_66A_n261I DC_66A_n261J DC_66A_n261K DC_66A_n261L DC_66A_n261M DC_66A_n261O DC_66A_n261P DC_66A_n261Q | DC_66A_n261A |
| DC_66A_n261(2A) DC_66A_n261(3A) DC_66A_n261(4A) DC_66A_n261(D-G) DC_66A_n261(D-H) DC_66A_n261(D-I) DC_66A_n261(D-O) DC_66A_n261(D-P) DC_66A_n261(D-Q) DC_66A_n261(E-O) DC_66A_n261(E-P) DC_66A_n261(E-Q) DC_66A_n261(2H) DC_66A_n261(2I) DC_66A_n261(A-H) DC_66A_n261(A-I) DC_66A_n261(A-D) DC_66A_n261(A-D-H) DC_66A_n261(A-G) DC_66A_n261(A-G-H) DC_66A_n261(G-I) DC_66A_n261(A-G-I) DC_66A_n261(A-H-I) DC_66A_n261(G-H) DC_66A_n261(H-I) DC_66A-n261(A-D-2O) DC_66A-n261(A-2D) DC_66A-n261(A-2G-2O) DC_66A-n261(A-3G-O) DC_66A-n261(A-4G) DC_66A-n261(A-2H) DC_66A-n261(A-2I) DC_66A-n261(A-4O) DC_66A-n261(A-7O) DC_66A-n261(A-2P) DC_66A-n261(A-2Q) | DC_66A_n261A DC_66A_n261G DC_66A_n261H DC_66A_n261I |
| DC_71A_n257A | DC_71A_n257A |
| DC_71A_n258A | DC_71A_n258A |
| DC_71A_n260A | DC_71A_n260A |
| DC_71A_n261A | DC_71A_n261A |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability for all of the above combinations |  |