3GPP TSG-RAN WG2 Meeting #107Bis	R2-1913239

Chongqing, China, 14th - 18th October 2019                        Revision of R2-1911129

Agenda Item:	6.16.2

Source:	MediaTek Inc.

Title:	MAC CE format for single PDCCH multi-TRP

Document for:     Discussion and Decision

# 1 Introduction

Based on RAN1#96 LS [1], RAN1 will support the enhancemnet of multi-TRP/panel transmission enhancement in Rel-16.

For multi-TRP/panel transmission, both multiple PDCCH and single PDCCH designs are supported in Rel-16 - Applies for eMBB For multiple PDCCH based multi-TRP/panel transmission, - The total number of CWs in scheduled PDSCHs, each of which is scheduled by one PDCCH, is up to 2 and also the total number of MIMO layers of scheduled PDSCHs is up to reported UE MIMO capability, if resource allocation of PDSCHs are overlapped. - To support multiple-PDCCH based multi-TRP/panel transmission with intra-cell (same cell ID) and inter-cell (different Cell IDs), RRC configuration can be used to link multiple PDCCH/PDSCH pairs with multiple TRPs. One CORESET in a “PDCCH-config” corresponds to one TRP. - Separate ACK/NACK payload/feedback for received PDSCHs is supported. For separate ACK/NACK payload/feedback for received PDSCHs where multiple DCIs are used, PUCCH resources conveying ACK/NACK feedback can be TDM with separated HARQ-ACK codebook. For single PDCCH based multi-TRP/panel transmission, - TCI indication framework shall be enhanced in Rel-16 at least for eMBB. Each TCI code point in a DCI can correspond to 1 or 2 TCI states. When 2 TCI states are activated within a TCI code point, each TCI state corresponds to one CDM group, at least for DMRS type 1 and FFS design for DMRS type 2. MAC-CE can be enhanced to map one or two TCI states for a TCI code point where further detailed design is determined in RAN2.

In RAN2#107 meeting[2], RAN2 send LS to RAN1 asking for clarification[3]:

Question 1: Should the pairing of the TCI states be flexible such that each TCI state can be dynamically paired with each TCI state or is there an upper limit of pairs by configuration? <ignore context explanation> Question 2: What is the total number of activated TCI states in mTRP operation?

In this paper, we propose our preferred MAC CE fomat.

# 2 Discussion

In our view, pairing of the TCI states for different TRP would probably be quite flexible, i.e. all combinations of TCI state for two TRPs cannot be excluded. So, instead of associating DCI codepoint with TCI state pairs (e.g. configured by RRC [4]), a more straightforward way is to signal the association between DCI point and TCI states of multiple TRP explicitly in MAC CE.

There are two ways to implement the enhanced MAC CE:

- First, modify/extend the existing MAC CE in subclause 6.1.3.14 to support 1 codepoint mapped to 2 TCI state

- Second, apply a new MAC CE which supports each DCI state mapped to 1 or 2 TCI state

Since current reserved LCID is still sufficient, we prefer not to change the existing MAC CE format in subclause 6.1.3.14, which would be good for backward compatibility.

- Introduce a new MAC CE to support single PDCCH indicating TCI states of multi-TRP.

There are two ways to implement the new MAC CE:

- Option 1: The new MAC CE associate each DCI codepoint with the TCI state of the first and second TRP (1 or 2 TCI states per DCI codepoint). [5][6][7][8][9][10][11]

  - For example, for each TCI codepoint a flag is used to indicate whether the codepoint is associated with a TCI state of the second TRP.

- Option 2: The new MAC CE associate each DCI codepoint with the TCI state of the second TRP (0 or 1 TCI state per TCI codepoint).

  - That is, the existing MAC CE defined in subclause 6.1.3.14 associates codepoint and TCI state of the first TRP, if an association exists; in contrast, the new MAC CE associates codepoint and TCI state of the second TRP, if an association exists

We think both options would work. The difference is that for option 2, the TCI state of the first TRP and the second TRP can be separately activated and deactivated via separate MAC CE. In contrast, for option 1, if there is a TCI state change on TRP 2 only, network still needs to signal the TCI state of TRP 1 together in the new MAC CE even if there is no change to the TCI state of TRP 1 at all (otherwise, UE will consider all the TCI state on TRP 1 as deactivated). So, approach 2 seems to have slightly less signaling overhead.

Observation: Signaling overhead to update the change of TCI state can be reduced if the new MAC CE is only used for activating/deactivating the TCI state of the second TRP.

- The new MAC CE is applied to activate/deactivate the TCI states for the second TRP only. The TCI states of the fist TRP is indicated by the existing MAC CE specified in subclause 6.1.3.14.

An exemplary CR text is provided in the Annex, in which we assume that at most 16 different TCI states can be activated when two TRPs are supported.

# 3 Conclusion

Based on the observations:

Observation: Signaling overhead to update the change of TCI state can be reduced if the new MAC CE is only used for activating/deactivating the TCI state on the second TRP.

We propose:

- Introduce a new MAC CE to support single PDCCH indicating TCI states of multi-TRP.

- The new MAC CE is applied to activate/deactivate the TCI states for the second TRP only. The TCI states of the fist TRP is indicated by the existing MAC CE specified in subclause 6.1.3.14.

# 4 Reference

- R1-1903697, LS on support of Enhancements on multi-TRP/panel transmission, RAN WG1, RAN1#96, Athens, Greece, 25th February – 1st March, 2019

- RAN2#107 Chairman’s note

- R2-1911848, LS on single PDCCH-based multi-TRP operation

- R2-1910144, MAC CE for DCI referring to more than one TCI state,	Ericsson

- R2-1911359, Support of Multiple Beam Indication for Multiple TRPs, Samsung

- R2-1910940, MAC CE signalling enhancement for TCI indication of single-PDCCH based multi-TRP transmission, Huawei, HiSilicon

- R2-1911375, Enhanced TCI States Activation/Deactivation PDSCH MAC CE for Multi-TRP, Qualcomm Incorporated

- R2-1909773, Consideration on MAC CE design for single PDCCH based multi-TRP transmission, CMCC

- R2-1910401, Consideration on Enhancement of TCI state for multiple-TRP transmission, ZTE

- R2-1910630, MAC CE format to indicate up to 2 TCI state per indication, OPPO

- R2-1910966, MAC CE design on single PDCCH based multi-TRP/panel transmission, NTT DOCOMO

# 5 Annex

<Start of modification>

### 5.18.xx	Activation of the second UE-specific PDSCH TCI state

The network may activate the second TCI state for a DCI codepoint of UE-sepcific PDSCH of a Serving Cell by sending the Enhanced TCI States Activation for UE-specific PDSCH MAC CE described in subclause 6.1.3.14.

The MAC entity shall:

1>	if the MAC entity receives an Enhanced TCI States Activation for UE-specific PDSCH MAC CE on a Serving Cell:

2>	indicate to lower layers the information regarding the TCI States Activation/Deactivation for UE-specific PDSCH MAC CE.

### 5.18.4	Activation/Deactivation of UE-specific PDSCH TCI state

The network may activate and deactivate the configured TCI states for PDSCH of a Serving Cell by sending the TCI States Activation/Deactivation for UE-specific PDSCH MAC CE described in subclause 6.1.3.14, or by sending the Enhanced TCI States Activation for UE-specific PDSCH MAC CE in subclause 6.1.3.xx if multiple TRP/panel transmission is supported. The configured TCI states for PDSCH are initially deactivated upon configuration and after a handover.

The MAC entity shall:

1>	if the MAC entity receives an TCI States Activation/Deactivation for UE-specific PDSCH MAC CE or an Enhanced TCI States Activation/Deactivation for UE-specific PDSCH MAC CE on a Serving Cell:

2>	indicate to lower layers the information regarding the TCI States Activation/Deactivation for UE-specific PDSCH MAC CE or the Enhanced TCI States Activation/Deactivation for UE-specific PDSCH MAC CE.

<Next modification>

#### 6.1.3.yy Enhanced TCI States Activation/Deactivation for UE-specific PDSCH MAC CE

The Enhanced TCI States Activation for UE-specific PDSCH is identified by a MAC PDU subheader with LCID as specified in Table 6.2.1-1. It has a variable size consisting of following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a DL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	DCIi: This field indicates the presence of the TCI-StateId field for DCI codepoint value i. The DCIi field set to "1" indicates that the TCI-StateId field for the DCI codepoint value i is present, and the TCI state identified by TCI-StateId shall be activated as an active TCI state mapped to DCI codepoint value i for this TRP. The DCIi field set to "0" indicates that the TCI-StateId field for DCI codepoint value i is not presented, and any TCI state mapped to DCI codepoint value i for this TRP should be deactivated;

-	TCI-StateID: This field indicates the identifier of TCI state, i.e., TCI-StateId, as specified in TS 38.331 [5]. The length of the field is 7 bits;

-	R: Reserved bit, set to "0".

Figure 6.1.3.yy-1: Enhanced TCI States Activation/Deactivation for UE-specific PDSCH MAC CE

<Next modification>

## 6.2	Formats and parameters

### 6.2.1	MAC subheader for DL-SCH and UL-SCH

The MAC subheader consists of the following fields:

-	LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE or padding as described in Tables 6.2.1-1 and 6.2.1-2 for the DL-SCH and UL-SCH respectively. There is one LCID field per MAC subheader. The LCID field size is 6 bits;

-	L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC CE in bytes. There is one L field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the L field is indicated by the F field;

-	F: The Format field indicates the size of the Length field. There is one F field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the F field is 1 bit. The value 0 indicates 8 bits of the Length field. The value 1 indicates 16 bits of the Length field;

-	R: Reserved bit, set to "0".

The MAC subheader is octet aligned.

Table 6.2.1-1 Values of LCID for DL-SCH

| Index | LCID values |
| --- | --- |
| 0 | CCCH |
| 1–32 | Identity of the logical channel |
| 33-45 | Reserved |
| 46 | Enhanced TCI states Activation/Deactivation for UE-specific PDSCH |
| 47 | Recommended bit rate |
| 48 | SP ZP CSI-RS Resource Set Activation/Deactivation |
| 49 | PUCCH spatial relation Activation/Deactivation |
| 50 | SP SRS Activation/Deactivation |
| 51 | SP CSI reporting on PUCCH Activation/Deactivation |
| 52 | TCI State Indication for UE-specific PDCCH |
| 53 | TCI States Activation/Deactivation for UE-specific PDSCH |
| 54 | Aperiodic CSI Trigger State Subselection |
| 55 | SP CSI-RS / CSI-IM Resource Set Activation/Deactivation |
| 56 | Duplication Activation/Deactivation |
| 57 | SCell Activation/Deactivation (four octet) |
| 58 | SCell Activation/Deactivation (one octet) |
| 59 | Long DRX Command |
| 60 | DRX Command |
| 61 | Timing Advance Command |
| 62 | UE Contention Resolution Identity |
| 63 | Padding |

Table 6.2.1-2 Values of LCID for UL-SCH

| Index | LCID values |
| --- | --- |
| 0 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]) |
| 1–32 | Identity of the logical channel |
| 33–51 | Reserved |
| 52 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]) |
| 53 | Recommended bit rate query |
| 54 | Multiple Entry PHR (four octet Ci) |
| 55 | Configured Grant Confirmation |
| 56 | Multiple Entry PHR (one octet Ci) |
| 57 | Single Entry PHR |
| 58 | C-RNTI |
| 59 | Short Truncated BSR |
| 60 | Long Truncated BSR |
| 61 | Short BSR |
| 62 | Long BSR |
| 63 | Padding |

<End of modification>