3GPP TSG RAN WG2 Meeting #107bis	                        		   R2-1913485

ChongQing, China, 14th - 18th October 2019

Source: 			ZTE Corporation, Sanechips

Title: 	Discussion on conditional PSCell addition/change

Agenda item:		6.9.4

Document for:		Discussion and Decision

# Introduction

At RAN2#107, we agreed to support conditional NR PSCell addition/change for (NG)EN-DC and NR-DC:

Agreements

1: 	Support conditional NR PSCell addition/change and reusing the conditional HO solution being developed. Supported for any architecture option with NR PSCell.

2	From RAN2 perspective conditional NR PSCell change can be supported for both intra-SN and inter-SN

In this contribution, we share some analysis on the support of PSCell addition/change from RAN2 aspect

# For PSCell addition

Considering the PSCell addition will impact the configuration on MN side as well (e.g. DRB level configuration, and corresponding cell group level configuration, including the reconfiguration for the purpose of capability coordination), to ensure the simultaneous activation of new configuration in both MN and SN, the configuration for both MN and SN shall be delivered to UE together as a single target configuration. And the target configuration for PSCell addition should be configured by a RRC reconfiguration message container generated by MN, in which both the configuration generated by MN and configuration generated by SN will be included.

Observation 1: For conditional PSCell addition, since both the new configuration on MN and SN are required, to ensure the simultaneous activation of configuration in both MN and SN, the target RRC reconfiguration container for PSCell addition shall be generated by MN, in which both the configuration generated by MN and configuration generated by SN will be included.

Similar as CHO, to perform conditional PSCell addition, the RRC container, containing the RRC reconfiguration message for PSCell addition, should be saved as candidate target reconfiguration, and the stored RRC container will be used in case the pre-configured criteria are satisfied. Compared to the conditional handover, it seems the signaling procedure for CHO and conditional PSCell is quite similar, and the PSCell addition can be modeled as conditional intra-cell handover with PSCell addition.

Observation 2: PSCell addition can be modeled as conditional intra-cell handover with PSCell addition.

Considering the limited time budget, to save the complexity in both standardization and implementation, we propose to support CHO and conditional PSCell addition in a common framework, and extend conditional handover to cover the conditional RRC reconfiguration with sync. With the conditional RRC reconfiguration, the UE can simply use the stored candidate RRC container in case the associated criteria are satisfied, no matter the stored candidate RRC container is generated for handover or for PSCell addition.

In addition, after going through all the agreements (attached in Annex for reference), we think all the agreements for CHO can be applied to conditional RRC reconfiguration with sync, since intra-cell handover (i.e. reconfiguration with sync) can be considered as handover as well, and we think a common solution is feasible to cover both the conditional handover and conditional RRC reconfiguration with sync.

Proposal 1: Extend conditional handover to cover conditional RRC reconfiguration with sync (i.e. intra-cell handover), with a common solution. All the agreements agreed for CHO shall be applied for conditional RRC reconfiguration with SYNC.

With the conditional RRC reconfiguration with sync, each candidate PSCell for PSCell addition will be configured with a separate RRC container containing RRC reconfiguration message generated by MN, in which both the configuration from MN side and the configuration from SN side will be included.

Proposal 2: With conditional RRC reconfiguration with sync, each candidate PSCell for PSCell addition will be configured with a RRC reconfiguration message container, in which both the configuration from MN side and the configuration from SN will be included.

For the trigger criteria of PSCell addition. For CHO, we have agreed that “cell level A3/A5-like CHO execution condition shall be specified (other events will not be specified without clear justifications)”. To support the conditional PSCell addition, we think A4 and B1 shall be supported as well.

Proposal 3: To support conditional PSCell addition, cell level A4/B1 based CHO execution condition shall be specified.

# For PSCell change

For the PSCell change, based on the impact MN, the following two kind of PSCell change can be considered

- Case1: PSCell change with impact on MN

- Case2: PSCell change without impact on MN

For PSCell change with impact on MN

For PSCell change with impact on MN, similar as PSCell addition, since both the MN and SN need to be reconfigured simultaneously, for each target PSCell, one candidate container for RRC reconfiguration message shall be generated by MN, and both the new configuration in MN side and new configuration in SN side shall be included, which can be considered as a usage for conditional RRC reconfiguration with sync.

Proposal 4: For PSCell change with impact on MN, to ensure the simultaneous activation of new configuration in both MN and SN, the configuration for both MN and SN shall be included in one candidate RRC reconfiguration message (container) generated by MN. And conditional RRC reconfiguration with sync can be used for this case.

For the configuration of execution condition, since the PSCell change can be initiated by either MN or SN, both the execution condition generated by MN and SN shall be supported for the case MN initiated PSCell change and SN initiated PSCell change accordingly.

Considering the execution condition will be associated with the measurement identity, which identifies a measurement configuration, and measurement id is configured independently in MN and SN, to support the execution condition configured by SN, the SN execution condition container shall be introduced. With the SN execution condition container, the execution determined by SN can be transmitted to UE through MN, and the execution condition configured in the container will be associated to the measurement id configured by SN.

Proposal 5: For PSCell change with impact on MN, to support PSCell change initiated by SN, SN execution condition container shall be introduced, with which the execution condition determined by SN can be transmitted to UE through MN, and the execution condition configured in the container will be associated to the measurement id configured by SN.

For PSCell change without impact on MN

For PSCell change without impact on MN, according to current specs, the PSCell change command (RRC reconfiguration message generated by SN) can be sent to UE through either SRB1 and SRB3. Since PSCell change without impact on MN can be transparent to MN, we think both the SRB1 based configuration and SRB3 based configuration can be supported for PSCell change without impact on MN.

Proposal 6: For PSCell change without impact on MN, conditional PSCell change can be configured through either SRB1 or SRB3.

For the case that conditional PSCell change is configured through SRB3, the RRC reconfiguration message generated by SN will be used to carry both the execution condition generated by SN and also the candidate PSCell configuration, which is encapsulated in one RRC container containing the RRC reconfiguration message generated by SN itself.

Considering the RRC reconfiguration message transmitted through SRB3 can also be transmitted in SRB1 in case SRB3 is not configured (e.g. as a RRC container in MRDC-SecondaryCellGroupConfig in MR-DC), to have a common solution for both the SRB1 based configuration and SRB3 based configuration, we propose that:

Proposal 7: For PSCell change without impact on MN, both the execution condition and the configuration for the candidate PSCell (as a container) shall be included in RRC reconfiguration message generated by SN and the message can be transmitted through either SRB1 or SRB3.

To configure the candidate PSCell through the SN RRC reconfiguration message, from signaling aspect, the following two alternative can be considered:

- Alt1: The execution condition and conditional target configuration container received in SN RRC reconfiguration message will be considered as execution condition and conditional target configuration container for PSCell change without MN impact.

- Alt2: Define new IEs for the execution condition and conditional target configuration for PSCell change without MN impact.

Considering the SN RRC reconfiguration message can be clearly identified by UE, and only the execution condition and conditional target configuration for PSCell change without MN impact can be included in the SN RRC reconfiguration message, we think there is no ambiguous in alternative 1. To avoid the introduction of unnecessary IEs, we give our proposal as follow:

Proposal 8: The execution condition and conditional target configuration container received in SN RRC reconfiguration message, either in SRB3 or in SN RRC container in MN generated RRC reconfiguration message through SRB1 (e.g. as RRC container in MRDC-SecondaryCellGroupConfig in MR-DC), will be considered as execution condition and conditional target configuration container for PSCell change without MN impact.

Even for PSCell change without impact on MN, considering the MN may configure CHO and new SN configuration may be included in the target cell information (e.g. for the NR-DC => NR-DC handover), the change of PSCell in SN side will impact the new SN configuration of target cell for configuration CHO, due to the fact that the new SN configuration may be generated based on the current SN configuration in a delta way.

Observation 3: Even for PSCell change without impact on MN, the change of PSCell in SN side will impact the new SN configuration of target cell for CHO, due to the fact that the new SN configuration may be generated based on the current SN configuration in a delta way.

To avoid mismatch between UE and NW side after CHO, the following two alternatives can be considered:

- Alt1: Once conditional PSCell change configured by SN RRC message is executed, the UE release all the conditional target configuration stored on UE side, no matter the conditional target configuration is configured by MN or SN.

- Alt2: Once conditional PSCell change is executed, the UE release all the stored target PSCell configuration configured by SN RRC message and release all the stored conditional target configuration configured by MN, if SN still exist after the execution of the target configuration and ReleaseAndAdd operation is not used for the SN.

Given that MN may use full configuration for the SCG configuration in CHO target cell configure in some cases, the execution of conditional PSCell change will not impact the pre-configured CHO candidate cell configuration. Thus, it seems the alternative 2 can provide more flexibility with limited complexity, and we propose to take alternative 2 in case the PSCell change configured by SN RRC message is executed.

Proposal 9: Once the conditional PSCell change configured by SN RRC message is executed, the UE should release all the stored target PSCell configuration configured by SN RRC message and release all the stored conditional target configuration configured by MN, if SN still exists after the execution of the target configuration and ReleaseAndAdd operation for SCG is not used.

To support proposal 9 and align the configuration in both UE side and NW side, the MN shall be informed once the PSCell change configured by SN RRC message is executed successfully. With this information, the MN knows that some conditional target reconfiguration configured by MN is released on UE side

Proposal 10: The MN shall be informed once the PSCell change configured by SN RRC message is executed successfully.

# Conclusion and proposals

In this contribution, we share some views on the support of the conditional PSCell addition/change. We have the following observations and proposals:

For PSCell addition

Observation 1: For conditional PSCell addition, since both the new configuration on MN and SN are required, to ensure the simultaneous activation of configuration in both MN and SN, the target RRC reconfiguration container for PSCell addition shall be generated by MN, in which both the configuration generated by MN and configuration generated by SN will be included.

Observation 2: PSCell addition can be modeled as conditional intra-cell handover with PSCell addition.

Proposal 1: Extend conditional handover to cover conditional RRC reconfiguration with sync (i.e. intra-cell handover), with a common solution. All the agreements agreed for CHO shall be applied for conditional RRC reconfiguration with SYNC.

Proposal 2: With conditional RRC reconfiguration with sync, each candidate PSCell for PSCell addition will be configured with a RRC reconfiguration message container, in which both the configuration from MN side and the configuration from SN will be included.

Proposal 3: To support conditional PSCell addition, cell level A4/B1 based CHO execution condition shall be specified.

For PSCell change with impact on MN

Proposal 4: For PSCell change with impact on MN, to ensure the simultaneous activation of new configuration in both MN and SN, the configuration for both MN and SN shall be included in one candidate RRC reconfiguration message (container) generated by MN. And conditional RRC reconfiguration with sync can be used for this case.

Proposal 5: For PSCell change with impact on MN, to support PSCell change initiated by SN, SN execution condition container shall be introduced, with which the execution condition determined by SN can be transmitted to UE through MN, and the execution condition configured in the container will be associated to the measurement id configured by SN.

For PSCell change without impact on MN

Proposal 6: For PSCell change without impact on MN, conditional PSCell change can be configured through either SRB1 or SRB3.

Proposal 7: For PSCell change without impact on MN, both the execution condition and the configuration for the candidate PSCell (as a container) shall be included in RRC reconfiguration message generated by SN and the message can be transmitted through either SRB1 or SRB3.

Proposal 8: The execution condition and conditional target configuration container received in SN RRC reconfiguration message, either in SRB3 or in SN RRC container in MN generated RRC reconfiguration message through SRB1 (e.g. as RRC container in MRDC-SecondaryCellGroupConfig in MR-DC), will be considered as execution condition and conditional target configuration container for PSCell change without MN impact.

Observation 3: Even for PSCell change without impact on MN, the change of PSCell in SN side will impact the new SN configuration of target cell for CHO, due to the fact that the new SN configuration may be generated based on the current SN configuration in a delta way.

Proposal 9: Once the conditional PSCell change configured by SN RRC message is executed, the UE should release all the stored target PSCell configuration configured by SN RRC message and release all the stored conditional target configuration configured by MN, if SN still exists after the execution of the target configuration and ReleaseAndAdd operation for SCG is not used.

Proposal 10: The MN shall be informed once the PSCell change configured by SN RRC message is executed successfully.

# Reference

- R2-1911640	Offline discussion report on 800 Intel Corporation

- R2-1913845	Discussion on fast RLF recovery when applying CHO and fast MCG recovery ZTE Corporation, Sanechips

- R3-193147	Baseline CR for introducing Rel-16 NR mobility enhancement

# Annex

### RAN2#107:

Agreements

1	For the scenario of multiple CHO cells being triggered the cell selected by the UE considering beams and beam quality. We will not specify normative requirements for the selection process but can be captured in an informative note in stage 3 spec.

2	No additional optimizations are introduced to improve RACH performance for CHO completion with multi-beam operation.

Agreements

1  For FR1, we will leave it up to UE implementation to select the target cell if more than one candidate cell meets the triggering condition (same as for FR2).

2  Do not introduce “bye” message from UE to the source cell for CHO.

3  If UE receives conventional handover command, it will execute the handover command regardless of stored (configured) conditional handover command. This applies if the HO cmd is received before any CHO triggering condition is satisfied. FFS how HO failure is handled.

4 	The UE can’t receive and perform RRC configuration from source cell while executing CHO command (which means from the time when the UE starts synchronization with target cell).

FFS whether simultaneous connectivity and CHO can work simultaneously.

5	UE is not required to continue evaluating the triggering condition of other candidate cell(s) during CHO execution.

6	We will not change cell selection procedure due to CHO (T310 expiry, T304(-like) expiry, etc.)

7	CHO is optional feature for UEs and networks.

Agreements

1	As part of CHO configuration to be sent to the UE, RRC container is used to carry target cell configuration and source cell is not allowed to alter any content of configuration from the target cell.

2	Use add/mod list + release list to configure multiple CHO candidate cells. CHO execution condition can be updated by modifying the existing CHO configuration, Target cell configuration can be updated by modifying the existing CHO configuration.

3	Reuse the RRCReconfiguration/RRCConnectionReconfiguration procedure to signal CHO configuration to UE.

4	A RRC complete message is required for UE to confirm receipt and proper comprehension of CHO configuration (execution condition, FFS target cell configuration) to the source eNB/gNB.

FFS whether the UE is required to check the compliance of the target cell configuration within CHO configuration upon reception or whether it is allowed to check upon execution.

FFS whether different RRC processing requirements are defined for the reconfiguration with CHO command.

5	After CHO configuration has been sent to the UE, source configuration can be updated.

FFS whether CHO commands need to be updated after source reconfiguration.

6	Delta configuration for CHO commands is based on latest source configuration

7	Allow having multiple triggering conditions (using “and”) for CHO execution of a single candidate cell. Only single RS type per CHO candidate is supported. At most two triggering quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultnaeously. FFS on UE capability.

8	TTT is supported for CHO condition (as per legacy configuration)

Agreements

- S1/N2-based CHO is not supported for Release 16. RAN3 impacst to be discussed in RAN3. If we want to support this, the WID has to be updated and SA2 impacts identified.

Agreements

1: 	Support conditional NR PSCell addition/change and reusing the conditional HO solution being developed. Supported for any architecture option with NR PSCell.

2	From RAN2 perspective conditional NR PSCell change can be supported for both intra-SN and inter-SN

### RAN2#106:

Agreements

2	The source cell decides on the condition for the execution of CHO.

3	The source cell adds the condition for the execution of CHO to the RRC message sent to UE.

4	Multiple CHO candidate cells can be sent in either one or multiple RRC messages. FFS on signalling details. FFS how CHO execution is handled.

5	CHO execution does not trigger measurement report.

6	On cell level A3/A5-like CHO execution condition shall be specified (other events will not be specified without clear justifications)

Agreements

1:	Separate CHO execution condition(s) can be configured for each individual candidate cells.

2	Define a CHO execution condition by the measurement identity which identifies a measurement configuration. (FFS to be addressed in stage 3 which parts of the measurement configuration are used for the CHO triggering)

3	As a baseline CHO can be triggered based on a condition consisting of a single event, single RS type, singe quantity.

3.1	The single trigger quantity can be configured to be RSRP, RSRQ or RS-SINR

3.2	The single RS type can be configured to be SSB or CSI-RS

FFS Whether multiple triggering conditions are required.

Agreements

1	Deconfiguration of CHO candidates is performed by RRC signalling (we will not introduce timer based mechanism for the UE to deconfiguration of the CHO candidates)

2	Baseline that configuration of all CHO candidates are released after successful (any) handover completion (sending complete message to the target cell).

FFS if it might be possible to keep CHO candidates after the HO.

Agreements

1	UE shall not stop T310 and shall not start T304 when it receives configuration of a CHO candidate

2.	The timer T310 is stopped and timer T304-like is started when the UE begins execution of a conditional handover for a target cell. (Stage 3 detail whether we reuse T304 or define a new timer)

Working assumption (to be confirmed next meeting after checking further details)

3	At RLF the UE performs cell selection and if the selected cell is a CHO candidate then the UE attempts CHO execution, otherwise re-establishment is performed

4	At legacy handover failure (T304 expiry) or failure to access a CHO candidate cell (T304-like expiry), the UE performs cell selection and if the selected cell is a CHO candidate then the UE attempts CHO execution, otherwise re-establishment is performed

### RAN2#105bis

0:	CHO is introduced in NR to solve robustness/reliability issue.

1:The LTE agreements below are applicable for NR:

a/ CHO is defined as UE having network configuration for initiating access to a target cell based on configured condition(s).

b/ Usage of conditional handover is decided by network. UE evaluates when the condition is valid.

c/ Support configuration of one or more candidate cells for conditional handover;

=>	FFS how many candidate cells (UE and network impacts should be clarified).

=>	FFS how to include the CHO conditions in UE configuration

d/ The baseline operation for Conditional HO procedure assumes HO command type of message contains HO triggering condition(s) and dedicated RRC configuration(s). UE accesses the prepared target when the relevant condition is met.

e/ The baseline operation for Conditional HO assumes the source RAN remains responsible for RRC until UE successfully sends RRC Reconfiguration Complete message to target RAN.

f/ 	RAN2 assumes late packet forwarding (i.e. not done immediately when the CHO target cells become prepared) could be suitable for CHO when there are multiple candidate target cells. Early packet forwarding can also be considered. Detailed decisions require RAN3 study.

2	Cell level quality is used as baseline for CHO execution condition;

FFS: on whether beam quality is used as input for CHO execution condition.

3	 RS type SSB can be used

FFS: CSI-RS, use of more than one RS type

4	Ax events (entry condition) are used for CHO execution condition and A3/5 as baseline

FFS: on other events

5	Trigger quantity for CHO execution condition(RSRP, RSRQ or RS-SINR) is configured by network.

FFS: on multiple quantities.

FFS: Enhancements to the above CHO framework to specifically address usage in FR2 (e.g. address high number of handovers, RLFs, etc)

### RAN2#105

Agreements

1	We will study at least conditional handover as one solution for handover robustness improvements.

2	We should consider how solutions work in FR2.