3GPP TSG-RAN2 Meeting #107-Bis	R2-1913598

# Chongqing, China, 14-18 October 2019

Agenda Item:	6.19

Source: 	Qualcomm Incorporated

Title: 	Recommended Bit Rate/Query for FLUS and MTSI

Document for:     Discussion/Decision

# Introduction

RAN2 received LS from SA4 in R2-1911499/S4-191031. RAN2 captured the following in RAN2#107:

=>    Can be considered how to respond at the next meeting based on contributions

In the LS, RAN4 explained about E_FLUS work item as well as description of uplink streaming bitrate assistance using RAN signalling. In this contribution, we discuss potential RAN2 impacts of the SA4 LS.

# Background

## General

TR 26.939 clause 11.2 contains a description of uplink streaming bitrate assistance using RAN signalling. In this deployment scenario, the UE (acting as the FLUS Source) interacts with the RAN by requesting a dynamic boost in the uplink streaming data rate from the eNB/gNB.

The envisioned means for fulfilling the above functionality is emulation of the existing ANBR query/response messaging mechanism as defined for the MTSI service in TS 26.114. According to current specifications, as indicated in clauses 10.7.4 (and 10.7.5) of TS 26.114 for LTE (and NR), ANBR Query/Response is mapped to a “Recommended bit rate MAC Control Element" which applies to a specific dedicated bearer.

For quick reference the subauses from TS 26.114 are reproduced below:

### 10.7.4 Message mapping for LTE access When using LTE access, ANBR is mapped to a MAC level message named "Recommended bit rate MAC Control Element" sent by the eNodeB and applicable to a specific dedicated bearer, as described by [85] and [157]. Similarly, when using LTE access, ANBRQ is mapped to a MAC level message named "Recommended bit rate query MAC Control Element" sent to the eNodeB and applicable to a specific, existing dedicated bearer, as described by [85] and [157]. An MTSI client in terminal using LTE access may support ANBR and ANBRQ signaling. ### 10.7.5 Message mapping for NR access When using NR access, ANBR is mapped to a MAC level message named "Recommended bit rate MAC Control Element" sent by the gNB and applicable to a specific logical channel which is mapped to the single media flow (e.g., audio or video) to which the recommended bit rate applies. Similarly, when using NR access, ANBRQ is mapped to a MAC level message named "Recommended bit rate query MAC Control Element" sent to the gNB and applicable to a specific, existing logical channel which is mapped to the single media flow to which the recommended bit rate applies. An MTSI client in terminal using NR access may support ANBR and ANBRQ signaling.

For LTE and NR, ANBR Query/Response is mapped to existing “Recommended bit rate MAC Control Element".

The functionality/semantics of ANBR as defined in TS 26.114 (in relation to bitrate restrictions established by session signaling, duration of validity for corresponding uplink bitrate adaptation, etc.) is expected to apply identically to the FLUS uplink data rate response from the eNB/gNB (e.g., when granting a higher bit rate for uplink streaming from the FLUS Source).

## The issue

The defined set of bit rate values for the MAC CE in TS 36.321 and TS 38.321 were intended for the currently supported speech or speech/video conferencing applications over MTSI services. As such, the maximum value is 8000 kbit/s (in both LTE and NR).

Currently defined maximum value for Recommended bit rate MAC CE is 8000 kbits/s for both LTE and NR.

SA4 explains in their LS that in FLUS, the targeted uplink streaming applications are expected to support significantly higher data rates associated with HD or UHD video streams, and Extended Reality functionality such as 360° VR. Therefore, it is expected that the upper range of recommended bit rate values to be specified in the MAC CE should be much higher than the current maximum value of 8000 kbps. For example, as indicated in clauses 5.4.2 and 5.4.3 of TR 26.925, high-quality 360° VR and UHD-resolution video streams are likely to be transmitted as high as 300 Mbps.

In addition, SA4 is currently standardizing VR support for conversational services based on MTSI as specified in TS 26.114, as part of the ITT4RT Rel-17 work item to support immersive teleconferencing and telepresence applications. Doing so will provide the means of sending high quality 360° VR streams for both uplink and downlink directions over MTSI, and hence the currently defined set of bit rate values for the MAC CE in TS 36.321 and 38.321 will again be below the required bitrates to support such immersive conversational services. In this case, it is desirable to expand the upper range of the recommended bitrate values in the MAC CE.

The currently possible value of 8000kbit/s is insufficient for E_FLUS where the targeted bitrates are as high as 300 Mbps and beyond.

## What SA4 wants?

SA4, with the LS, is requesting RAN2 to extend the data rate range that can be signalled using the existing Recommended bit rate MAC CE, for both LTE and NR.

SA4 is requesting RAN2 to extend the data rate range that can be signalled using the existing Recommended bit rate MAC CE, for both LTE and NR.

# Potential RAN2 spec impacts

The Recommended Bit Rate MAC CE already exists in both TS 36.321 and TS 38.321. Following is a snapshot of the existing specifications:

From TS 38.321 (see TS 36.321 section 6.1.3.13 for LTE)

#### 6.1.3.20 Recommended bit rate MAC CE The Recommended bit rate MAC CE is identified by a MAC subheader with LCID as specified in Tables 6.2.1-1 and 6.2.1-2 for bit rate recommendation message from the gNB to the UE and bit rate recommendation query message from the UE to the gNB, respectively. It has a fixed size and consists of two octets defined as follows (Figure 6.1.3.20-1): - LCID: This field indicates the identity of the logical channel for which the recommended bit rate or the recommended bit rate query is applicable. The length of the field is 6 bits; - Uplink/Downlink (UL/DL): This field indicates whether the recommended bit rate or the recommended bit rate query applies to uplink or downlink. The length of the field is 1 bit. The UL/DL field set to 0 indicates downlink. The UL/DL field set to 1 indicates uplink; - Bit Rate: This field indicates an index to Table 6.1.3.20-1. The length of the field is 6 bits. For bit rate recommendation the value indicates the recommended bit rate. For bit rate recommendation query the value indicates the desired bit rate; - R: reserved bit, set to 0. Figure 6.1.3.20-1: Recommended bit rate MAC CE Table 6.1.3.20-1: Values (kbit/s) for Bit Rate field Index; NR Recommended Bit Rate value [kbit/s]; Index; NR Recommended Bit Rate value [kbit/s]. 0; Note 1; 32; 700. 1; 0; 33; 800. 2; 9; 34; 900. 3; 11; 35; 1000. 4; 13; 36; 1100. 5; 17; 37; 1200. 6; 21; 38; 1300. 7; 25; 39; 1400. 8; 29; 40; 1500. 9; 32; 41; 1750. 10; 36; 42; 2000. 11; 40; 43; 2250. 12; 48; 44; 2500. 13; 56; 45; 2750. 14; 72; 46; 3000. 15; 88; 47; 3500. 16; 104; 48; 4000. 17; 120; 49; 4500. 18; 140; 50; 5000. 19; 160; 51; 5500. 20; 180; 52; 6000. 21; 200; 53; 6500. 22; 220; 54; 7000. 23; 240; 55; 7500. 24; 260; 56; 8000. 25; 280; 57; Reserved. 26; 300; 58; Reserved. 27; 350; 59; Reserved. 28; 400; 60; Reserved. 29; 450; 61; Reserved. 30; 500; 62; Reserved. 31; 600; 63; Reserved. Note 1: For bit rate recommendation message this index is used for indicating that no new recommendation on bit rate is given..

In the following we describe some simple ways to fulfil the SA4 request.

Option 1: using reserved values

Currently, the MAC CE has 7 reserved values remaining for both NR and LTE. Some of the reserved values can be used to define higher bit rates. However, given the range of the desired extension (from 8000 kbit/s to desired value (according to LS) of 300,000 kbit/s), the available reserved values is not be sufficient to have sufficient granularity.

Option 2: using reserved bits as multiplier to existing values

Currently, the MAC CE has 3 reserved bits for NR and 5 reserved bits for LTE. One of the reserved bits can be used to indicate a multiplier for the value indicated by the MAC CE.  For example, if one of the R bits indicate multiplier of 100x, the existing values would correspond to 900 kbit/s to 800,000 kbits/s. This will sufficiently cover the desired range for Rel-16 and will be future proof even for Rel-17 WI that the SA4 LS describes.

Note that if the multiplier is small, some of the values will be “duplicates” (which is not an issue in itself, but effectively means full potential of extensibility is not realized) but if the multiplier is large, the granularity on the higher end is coarser. For example, with multiplier of 100, index 3 with R=1 and index 36 with R = 0 both correspond to 1100 kbits/s. On the other hand, with multiplier of 1000x, there will be no “duplicates”, but the granularity between e.g. index 55 and 56 would be too large (500,000 kbit/s).

The required range extension to fulfil SA4 request can be achieved by multiplier as small as ~40x. In Annex A, we show how many new values defined by different multipliers around this value (40x) correspond to “existing” value as summarized here.

| Multiplier | Upper limit of new range extended up to | Worst granularity | Duplicates (including 0) |
| --- | --- | --- | --- |
| 40x | 320,000 kbits/s | 20,000 kbits/s | 3 |
| 50x | 400,000 kbits/s | 25,000 kbits/s | 6 |
| 60x | 480,000 kbits/s | 30,000 kbits/s | 2 |
| 100x | 800,000 kbits/s | 50,000 kbits/s | 6 |

Option 3: using reserved bit to extend the table

One of the R bits can be used to extend the bit rate table to accommodate up to index 127 (effectively using the R bit as MSB) to define the extended table with enough granularity. However, this involves more specification changes.

In the below table, we summarize pros and cons of each approach:

| Option | Pros | Cons |
| --- | --- | --- |
| 1 | Simple | Granularity cannot be achieved for the extended range |
| 2 | Simple | Granularity can be achieved with selection of a reasonable multiplier. However, some of the signalling pairs (existing index vs multiplier X lower index) may result in same value (i.e., duplication). As such, duplication is not an issue but added possible values are effectively less in number. |
| 3 | Simple but slightly more specification effort compared to above options | Both granularity and desired range can be achieved. Future-proof as more values can be left as “reserved”. |

The TPs for different options are provided in Annex B to illustrate the extent of specification impact.

There are various simple options to fulfil SA4 request without significant RAN2 specification impact.

Based on the above discussion, we propose the following:

RAN2 agrees to extend the bitrate range of Recommended bitrate MAC CE for both NR and LTE in Rel-16.

RAN2 should discussion the above options and agree on one of the options. If option 1, or 3 is chosen, RAN2 should also conclude on new candidate values for the recommended bitrate. If option 2 is chosen, RAN2 should also conclude on the multiplier factor. Formal CR can be discussed once the solution is agreed.

RAN2 to discussion and agree on one of the solutions. The selected option applies to both NR and LTE.

RAN2 should reply to SA4 that RAN2 has agreed to extend the MAC CE to accommodate larger range of bitrates covering the range described in SA4 LS. A draft reply LS is given in R2-19xx.

Reply with an LS that RAN2 has agreed to extend the MAC CE to accommodate larger range of bitrates for both NR and LTE in Rel-16. Draft Reply LS is provided in R2-1913599.

# Summary

In this contribution, we discussed RAN2 aspects of E_FLUS based on SA4 LS. Based on the above discussion, we made the following observations:

Observation 1.	For LTE and NR, ANBR Query/Response is mapped to existing “Recommended bit rate MAC Control Element".

Observation 2.	Currently defined maximum value for Recommended bit rate MAC CE is 8000 kbits/s for both LTE and NR.

Observation 3.	The currently possible value of 8000kbit/s is insufficient for E_FLUS where the targeted bitrates are as high as 300 Mbps and beyond.

Observation 4.	SA4 is requesting RAN2 to extend the data rate range that can be signalled using the existing Recommended bit rate MAC CE, for both LTE and NR.

Observation 5.	There are various simple options to fulfil SA4 request without significant RAN2 specification impact.

Based on the discussion and observations, we propose:

Proposal 1.	RAN2 agrees to extend the bitrate range of Recommended bitrate MAC CE for both NR and LTE.

Proposal 2.	RAN2 to discussion and agree on one of the solutions. The selected option applies to both NR and LTE.

Proposal 3.	Reply with an LS that RAN2 has agreed to extend the MAC CE to accommodate larger range of bitrates. Draft Reply LS is provided in R2-1913599.

# Annex A: Option 2 multipliers and duplicated data rates

| Index | Existing Value | With X=40 | With X = 50 | With X = 60 | With X = 100 |
| --- | --- | --- | --- | --- | --- |
| 0 | NOTE |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 0 |
| 2 | 9 | 360 | 450 | 540 | 900 |
| 3 | 11 | 440 | 550 | 660 | 1,100 |
| 4 | 13 | 520 | 650 | 780 | 1,300 |
| 5 | 17 | 680 | 850 | 1,020 | 1,700 |
| 6 | 21 | 840 | 1,050 | 1,260 | 2,100 |
| 7 | 25 | 1,000 | 1,250 | 1,500 | 2,500 |
| 8 | 29 | 1,160 | 1,450 | 1,740 | 2,900 |
| 9 | 32 | 1,280 | 1,600 | 1,920 | 3,200 |
| 10 | 36 | 1,440 | 1,800 | 2,160 | 3,600 |
| 11 | 40 | 1,600 | 2,000 | 2,400 | 4,000 |
| 12 | 48 | 1,920 | 2,400 | 2,880 | 4,800 |
| 13 | 56 | 2,240 | 2,800 | 3,360 | 5,600 |
| 14 | 72 | 2,880 | 3,600 | 4,320 | 7,200 |
| 15 | 88 | 3,520 | 4,400 | 5,280 | 8,800 |
| 16 | 104 | 4,160 | 5,200 | 6,240 | 10,400 |
| 17 | 120 | 4,800 | 6,000 | 7,200 | 12,000 |
| 18 | 140 | 5,600 | 7,000 | 8,400 | 14,000 |
| 19 | 160 | 6,400 | 8,000 | 9,600 | 16,000 |
| 20 | 180 | 7,200 | 9,000 | 10,800 | 18,000 |
| 21 | 200 | 8,000 | 10,000 | 12,000 | 20,000 |
| 22 | 220 | 8,800 | 11,000 | 13,200 | 22,000 |
| 23 | 240 | 9,600 | 12,000 | 14,400 | 24,000 |
| 24 | 260 | 10,400 | 13,000 | 15,600 | 26,000 |
| 25 | 280 | 11,200 | 14,000 | 16,800 | 28,000 |
| 26 | 300 | 12,000 | 15,000 | 18,000 | 30,000 |
| 27 | 350 | 14,000 | 17,500 | 21,000 | 35,000 |
| 28 | 400 | 16,000 | 20,000 | 24,000 | 40,000 |
| 29 | 450 | 18,000 | 22,500 | 27,000 | 45,000 |
| 30 | 500 | 20,000 | 25,000 | 30,000 | 50,000 |
| 31 | 600 | 24,000 | 30,000 | 36,000 | 60,000 |
| 32 | 700 | 28,000 | 35,000 | 42,000 | 70,000 |
| 33 | 800 | 32,000 | 40,000 | 48,000 | 80,000 |
| 34 | 900 | 36,000 | 45,000 | 54,000 | 90,000 |
| 35 | 1,000 | 40,000 | 50,000 | 60,000 | 100,000 |
| 36 | 1,100 | 44,000 | 55,000 | 66,000 | 110,000 |
| 37 | 1,200 | 48,000 | 60,000 | 72,000 | 120,000 |
| 38 | 1,300 | 52,000 | 65,000 | 78,000 | 130,000 |
| 39 | 1,400 | 56,000 | 70,000 | 84,000 | 140,000 |
| 40 | 1,500 | 60,000 | 75,000 | 90,000 | 150,000 |
| 41 | 1,750 | 70,000 | 87,500 | 105,000 | 175,000 |
| 42 | 2,000 | 80,000 | 100,000 | 120,000 | 200,000 |
| 43 | 2,250 | 90,000 | 112,500 | 135,000 | 225,000 |
| 44 | 2,500 | 100,000 | 125,000 | 150,000 | 250,000 |
| 45 | 2,750 | 110,000 | 137,500 | 165,000 | 275,000 |
| 46 | 3,000 | 120,000 | 150,000 | 180,000 | 300,000 |
| 47 | 3,500 | 140,000 | 175,000 | 210,000 | 350,000 |
| 48 | 4,000 | 160,000 | 200,000 | 240,000 | 400,000 |
| 49 | 4,500 | 180,000 | 225,000 | 270,000 | 450,000 |
| 50 | 5,000 | 200,000 | 250,000 | 300,000 | 500,000 |
| 51 | 5,500 | 220,000 | 275,000 | 330,000 | 550,000 |
| 52 | 6,000 | 240,000 | 300,000 | 360,000 | 600,000 |
| 53 | 6,500 | 260,000 | 325,000 | 390,000 | 650,000 |
| 54 | 7,000 | 280,000 | 350,000 | 420,000 | 700,000 |
| 55 | 7,500 | 300,000 | 375,000 | 450,000 | 750,000 |
| 56 | 8,000 | 320,000 | 400,000 | 480,000 | 800,000 |

# Annex B: Illustration of specification impact for different options

## Option 1:

Table 6.1.3.20-1: Values (kbit/s) for Bit Rate field

| Index | NR Recommended Bit Rate value [kbit/s] | Index | NR Recommended Bit Rate value [kbit/s] |
| --- | --- | --- | --- |
| 0 | Note 1 | 32 | 700 |
| 1 | 0 | 33 | 800 |
| 2 | 9 | 34 | 900 |
| 3 | 11 | 35 | 1000 |
| 4 | 13 | 36 | 1100 |
| 5 | 17 | 37 | 1200 |
| 6 | 21 | 38 | 1300 |
| 7 | 25 | 39 | 1400 |
| 8 | 29 | 40 | 1500 |
| 9 | 32 | 41 | 1750 |
| 10 | 36 | 42 | 2000 |
| 11 | 40 | 43 | 2250 |
| 12 | 48 | 44 | 2500 |
| 13 | 56 | 45 | 2750 |
| 14 | 72 | 46 | 3000 |
| 15 | 88 | 47 | 3500 |
| 16 | 104 | 48 | 4000 |
| 17 | 120 | 49 | 4500 |
| 18 | 140 | 50 | 5000 |
| 19 | 160 | 51 | 5500 |
| 20 | 180 | 52 | 6000 |
| 21 | 200 | 53 | 6500 |
| 22 | 220 | 54 | 7000 |
| 23 | 240 | 55 | 7500 |
| 24 | 260 | 56 | 8000 |
| 25 | 280 | 57 | valueXX |
| 26 | 300 | 58 | ValueYY |
| 27 | 350 | 59 | ValueZZ |
| 28 | 400 | 60 | etc |
| 29 | 450 | 61 | etc |
| 30 | 500 | 62 | Reserved |
| 31 | 600 | 63 | Reserved |
| Note 1: For bit rate recommendation message this index is used for indicating that no new recommendation on bit rate is given. |  |  |  |

## Option 2:

-	Bit Rate: This field indicates an index to Table 6.1.3.20-1. The length of the field is 6 bits. For bit rate recommendation the value indicates the recommended bit rate. For bit rate recommendation query the value indicates the desired bit rate;

-	X: bit rate multiplier. If X is set to 1, the acutal value of indicated bit rate is the value correspnding to the index indicated by Bit Rate field multiplied by YY.

-	R: reserved bit, set to 0.

Figure 6.1.3.20-1: Recommended bit rate MAC CE

## Option 3:

- 	Bit Rate: This field indicates an index to Table 6.1.3.20-1. The length of the field is 6 bits. When X is set to 1, the indicated index is 64 + the value corresponding to 6 bits Bit Rate field. For bit rate recommendation the value indicates the recommended bit rate. For bit rate recommendation query the value indicates the desired bit rate;

-	R: reserved bit, set to 0.

Figure 6.1.3.20-1: Recommended bit rate MAC CE

Table 6.1.3.20-1: Values (kbit/s) for Bit Rate field

| Index | NR Recommended Bit Rate value [kbit/s] | Index | NR Recommended Bit Rate value [kbit/s] | Index | NR Recommended Bit Rate value [kbit/s] | Index | NR Recommended Bit Rate value [kbit/s] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Note 1 | 32 | 700 | 64 | valueX | 96 | etc |
| 1 | 0 | 33 | 800 | 65 | ValueY | 97 |  |
| 2 | 9 | 34 | 900 | 66 | ValueZ | 98 |  |
| 3 | 11 | 35 | 1000 | 67 | Etc. | 99 |  |
| 4 | 13 | 36 | 1100 | 68 |  | 100 |  |
| 5 | 17 | 37 | 1200 | 69 |  | 101 |  |
| 6 | 21 | 38 | 1300 | 70 |  | 102 |  |
| 7 | 25 | 39 | 1400 | 71 |  | 103 |  |
| 8 | 29 | 40 | 1500 | 72 |  | 104 |  |
| 9 | 32 | 41 | 1750 | 73 |  | 105 |  |
| 10 | 36 | 42 | 2000 | 74 |  | 106 |  |
| 11 | 40 | 43 | 2250 | 75 |  | 107 |  |
| 12 | 48 | 44 | 2500 | 76 |  | 108 |  |
| 13 | 56 | 45 | 2750 | 77 |  | 109 | Reserved |
| 14 | 72 | 46 | 3000 | 78 |  | 110 | Reserved |
| 15 | 88 | 47 | 3500 | 79 |  | 111 | Reserved |
| 16 | 104 | 48 | 4000 | 80 |  | 112 | Reserved |
| 17 | 120 | 49 | 4500 | 81 |  | 113 | Reserved |
| 18 | 140 | 50 | 5000 | 82 |  | 114 | Reserved |
| 19 | 160 | 51 | 5500 | 83 |  | 115 | Reserved |
| 20 | 180 | 52 | 6000 | 84 |  | 116 | Reserved |
| 21 | 200 | 53 | 6500 | 85 |  | 117 | Reserved |
| 22 | 220 | 54 | 7000 | 86 |  | 118 | Reserved |
| 23 | 240 | 55 | 7500 | 87 |  | 119 | Reserved |
| 24 | 260 | 56 | 8000 | 88 |  | 120 | Reserved |
| 25 | 280 | 57 | Reserved | 89 |  | 121 | Reserved |
| 26 | 300 | 58 | Reserved | 90 |  | 122 | Reserved |
| 27 | 350 | 59 | Reserved | 91 |  | 123 | Reserved |
| 28 | 400 | 60 | Reserved | 92 |  | 124 | Reserved |
| 29 | 450 | 61 | Reserved | 93 |  | 125 | Reserved |
| 30 | 500 | 62 | Reserved | 94 |  | 126 | Reserved |
| 31 | 600 | 63 | Reserved | 95 |  | 127 | Reserved |
| Note 1: For bit rate recommendation message this index is used for indicating that no new recommendation on bit rate is given. |  |  |  |  |  |  |  |