3GPP TSG-RAN WG2 Meeting #107bis	R2-1913683

Chongqing, China, 14th October – 18th October 2019

Agenda Item	: 7.2.4

Source	: LG Electronics Inc.

Title	: Paging response using D-PUR

Document for	: Discussion and Decision

1.	Introduction

Upon reception of a paging message, the UE immediately triggers RRC connection establishment, resumption or EDT procedure. However, for some cases, downlink data may not be time-critical such that the data can be delivered with delay.

In this contribution, we discuss paging response using D-PUR.

2.	Discussion

When a downlink user data arrives in the idle mode, the network sends a paging message to find the target UE and deliver the DL data to the UE. Upon reception of a paging message, a UE immediately triggers a random access procedure to respond to the paging. However, for some cases, transmitting the user data with delay may be acceptable in a service aspect. Also, if the UE is stationary, the network may not need to find the target UE.

Where UEs are deployed in a fixed area, the network may not need to check the location of the target UE. If downlink user data does not have delay requirement and the network has sudden heavy traffic load, the network may configure UL resource to the UE for future use when the network expects that traffic load becomes light. This may be the example use case for a single shot D-PUR configuration.

To support this, the MME or S-GW may need to keep DL data for longer time than legacy behaviour. We suggest that the decision is left to the network implementation because it may depend on the real-time network situation, the storage capacity, etc.

Also, the NAS layer has impacts because the NAS layer immediately initiates the paging response and starts a timer. Therefore, the indication is needed to inform the case whose paging response is expected to be received via D-PUR. This indication should be included in a paging message.

Proposal. RAN2 is kindly asked to discuss whether D-PUR can be used to send a paging response message.

3.	Conclusion

Proposal. RAN2 is kindly asked to discuss whether D-PUR can be used to send a paging response message.