| 3GPP TR 37.985 V0.2.0 (2019-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Overall description of Radio Access Network (RAN) aspects for Vehicle-to-everything (V2X) based on LTE and NR (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Services and requirements	9

5	LTE V2X	9

5.1	V2X sidelink physical layer	9

5.1.1	Physical sidelink channels and signals	9

5.1.2	Sidelink synchronization	10

5.1.2.1	Synchronization references and priorities	10

5.1.2.2	SLSS	10

5.1.2.2.1	SLSSID	11

5.1.3	Multi-carrier operation and carrier aggregation	11

5.2	V2X sidelink resource allocation	12

5.2.1	Resource pools	12

5.2.2	Resource allocation modes	14

5.2.2.1	Resource allocation mode 3	14

5.2.2.2	Resource allocation mode 4	14

5.2.2.2.1	Zones	15

5.2.2.3	Modes 3 and 4 resource pool sharing	16

5.3	Sidelink congestion control	16

5.4	V2X sidelink higher-layer protocols	17

5.4.1	General	17

5.4.2	Resource pool configuration	17

5.4.3	Measurement and reporting	17

5.4.4	Mobility management 	17

5.4.5	Assistance information for SL SPS configuration	18

5.4.6	Transmission carrier selection	18

5.4.7	SL packet duplication	18

5.4.8	Coordination between UL and V2X SL transmission	18

5.4.9	Multi-PLMN operation	18

5.5	V2X via the Uu interface	18

5.6	Network aspects	18

5.6.1	V2X service authorization	18

5.6.2	Sidelink AMBR	19

6	NR V2X	19

6.1	Sidelink unicast, groupcast, and broadcast	19

6.2	V2X sidelink physical layer	19

6.2.1	Physical sidelink channels and signals	19

6.2.2	Sidelink synchronization	20

6.2.2.1	Synchronization references and priorities	20

6.2.2.2	S-SSB and SLSS	21

6.2.2.2.1	SLSSID	21

6.2.3	Sidelink CSI	21

6.2.4	Sidelink HARQ	21

6.2.5	In-device coexistence between LTE-V2X and NR-V2X sidelinks	22

6.3	V2X sidelink resource allocation	22

6.3.1	Sidelink bandwidth parts and resource pools	22

6.3.1.1	Sidelink bandwidth parts	22

6.3.1.2	Resource pools	22

6.3.2	Resource allocation modes	23

6.3.2.1	Mode 1	23

6.3.2.2	Mode 2	23

6.4	QoS framework and congestion control	24

6.5	V2X sidelink higher-layer protocols	24

6.6	V2X via the Uu interface	24

6.7	Network aspects	24

7	Multi-RAT V2X	24

7.1	Cross-RAT operation	24

7.2	MR-DC	25

8	Transmission profiles	25

9	Pedestrian UE	25

10	Roadside unit	25

Annex <X> (informative): Change history	26

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

NOTE 1:	The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

NOTE 2:	The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

NOTE 3:	The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

NOTE 4:	The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

NOTE 5:	The constructions "is" and "is not" do not indicate requirements.

Introduction

XXX

1	Scope

The present document …

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905 : "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 36.885 : "Study on LTE-based V2X Services".

[3]	ETSI EN 302 637-2 : "Specification of Cooperative Awareness Basic Service".

[4]	SAE J2735 : "Dedicated Short Range Communications (DSRC) Message Set Dictionary".

[5]	ETSI EN 302 637-3 : "Specifications of Decentralized Environmental Notification Basic Service".

[6]	3GPP TS 22.185 : "Service requirements for V2X services".

[7]	3GPP TS 22.186 : "Enhancement of 3GPP support for V2X scenarios".

[8]	3GPP TS 36.211 : "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

[9]	3GPP TS 36.212 : " Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding".

[10]	3GPP TS 36.331 : " Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC)".

[11]	3GPP TS 36.101 : " Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[12]	3GPP TS 36.214 : " Evolved Universal Terrestrial Radio Access (E-UTRA); Measurements".

[13]	3GPP TS 36.300 : " Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2". [14]	3GPP TS 36.321 : " Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC)".

[15]	3GPP TS 36.323 : " Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP)".

[16]	3GPP TS 38.211 : "NR; Physical channels and modulation".

[17]	3GPP TS 38.331 : "NR; Radio Resource Control (RRC) protocol specification".

3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Where the same abbreviation is used for LTE V2X and NR V2X, which is meant can be derived from the section within which it appears, unless otherwise stated.

AMBR	Aggregate maximum bit rate

BSM	Basic safety message

BWP	Bandwidth part

CA	Carrier aggregation

CAM	Cooperative awareness message

CBR	Channel busy ratio

CR	Channel usage ratio

DENM	Decentralized environmental notification message

DMRS	Demodulation reference signal

MBSFN	Multicast-broadcast single-frequency network

MNO	Mobile network operator

PPPP	ProSe per-packet priority

PPPR	ProSe per-packet reliability

PSBCH	Physical sidelink broadcast channel

PSCCH	Physical sidelink control channel

PSSCH	Physical sidelink shared channel

PSSS, S-PSS	Primary sidelink synchronization signal (LTE), sidelink primary synchronization signal (NR)

PT-RS	Phase-tracking reference signal

P-UE	Pedestrian UE

RSU	Roadside unit

SA	Scheduling assignment

SCI	Sidelink control information

SC-PTM	Single-cell point-to-multipoint

SL-BCH	Sidelink broadcast channel

SLSS	Sidelink synchronization signal

S-RSSI	Sidelink received signal strength indicator

S-SSB	Sidelink synchronization signal block

SSSS, S-SSS	Secondary sidelink synchronization signal (LTE), sidelink secondary synchronization signal (NR)

V2I	Vehicle-to-infrastructure

V2P	Vehicle-to-pedestrian

V2V	Vehicle-to-vehicle

V2X	Vehicle-to-everything

4	Services and requirements

LTE-V2X is designed with BSM, CAM, and DENM particularly in mind. BSMs and CAMs have the characteristic of generating periodic messages at intervals, whereas DENMs are event-triggered. As an illustration of the different message types, in [2], BSM/CAM were modelled, for evaluation purposes, as periodically occurring sets of one 300-byte message followed by four 190-byte messages. These types of message regularly broadcast information such as the vehicle’s heading, speed, latitude/longitude, etc. [3], [4]. In [2], DENMs were modelled, for evaluation purposes, as Poisson distributed initiations of six 800-byte messages spaced by 100 ms. DENMs can contain various different messages depending on the cause for their transmission, such as imminent collision, sudden braking, or detection of a traffic jam, amongst others [5]. The requirements relating to traffic size and pattern for LTE-V2X set in [6] can be summarized as follows, although they do not limit the usage of LTE-V2X. Other requirements relating to general system function are also included in [6].

-	Support for periodic broadcast messages with payloads of 50-300 bytes.

-	Support for event-triggered messages with payloads of up to 1200 bytes.

-	Up to 10 messages per second transmitted by a UE.

-	V2V and V2P latency of maximum 100 ms, or for V2V pre-crash sensing, maximum 20 ms.

-	V2I latency, i.e. between a UE and RSU, of maximum 100 ms.

-	V2N latency, i.e. when transferring messages via the cellular network, of maximum 1000 ms.

-	Maximum relative velocity between two vehicles of 500 km/h, and maximum absolute velocity of 250 km/h for V2V and V2P UEs, and of a UE communicating with an RSU.

-	Requirements relating to security, integrity, authorization, and privacy.

[NR V2X to be added later].

5	LTE V2X

5.1	V2X sidelink physical layer

The LTE V2X sidelink supports broadcast transmission of messages in the physical layer, since this is a suitable approach for delivery BSM, CAM, DENM and similar traffic. In the MAC layer, a broadcast address can be mapped to a single UE or a group of UEs by implementation. Such implementation techniques have no particular specification support in LTE, and are transparent to the physical layer.

5.1.1	Physical sidelink channels and signals

The LTE V2X sidelink uses the following physical channels and signals:

-	Physical sidelink broadcast channel (PSBCH), specified in [8, section 9.6]

-	Physical sidelink control channel (PSCCH), specified in [8, section  9.4]

-	Physical sidelink shared channel (PSSCH), specified in [8, section 9.3]

-	Primary and secondary sidelink synchronization signals (PSSS and SSSS) specified in [8, section 9.7]. These can be referred to jointly as the sidelink synchronization signal (SLSS).

-	A demodulation reference signal (DMRS) associated with each of the three physical channels, specified in [8, section 9.8]

LTE-V2X sidelink physical channels are transmitted using SC-FDMA.

PSBCH transmits the SL-BCH transport channel, which carries the sidelink V2X Master Information Block (MIB-V2X) from the RRC layer. When in use, PSBCH transmits MIB-V2X every 160 ms in the central 72 subcarriers of the SL bandwidth. DMRS associated with PSBCH are transmitted in the 5th, 7th, and 10th symbols of the subframe.

PSSS and SSSS are transmitted to allow other UEs to achieve sidelink synchronization when they do not have another source of synchronization available. They jointly convey the SLSS ID selected by the UE. For further details of SLSS and synchronization, refer to Section 5.1.2. PSSS/SSSS also allow UEs to detect the sidelink subframe boundary, with subframe number and frame number signalled in MIB-V2X.

PSSCH transmits the SL-SCH transport channel, which carries the TBs of data for transmission over SL. The resources in which PSSCH is transmitted can either be scheduled by an eNB and granted to the UE by a DCI (termed resource allocation mode 3, see Section 5.2.2.1) or determined through a sensing procedure conducted autonomously by the transmitting UE (termed resource allocation mode 4, see Section 5.2.2.2). A given TB can be transmitted once or twice, with a second transmission occurring a time gap after the first which is indicated in the scheduling SCI.

PSCCH transmits physical layer sidelink control information (SCI), also known as a scheduling assignment (SA). For V2X, PSCCH is transmitted in two frequency-adjacent PRBs, and always carries SCI format 1, defined in [9, section 5.4.3.1.2]. To receive PSCCH, a UE has to monitor each defined pair of PRBs to determine whether PSCCH has been transmitted in them. PSCCH is transmitted in the same subframe(s) as the associated PSSCH, and can be transmitted in PRBs that are either frequency adjacent or frequency non-adjacent to the PSSCH.

DMRS associated with PSSCH and PSCCH are transmitted in the 3rd, 6th, 9th, and 12th symbols of a subframe.

5.1.2	Sidelink synchronization

5.1.2.1	Synchronization references and priorities

There are four basic sources, or references, from which a V2X UE can derive its own synchronization: GNSS, its serving eNB, another UE transmitting SLSS (a SyncRef UE), or its own internal clock. In general, GNSS or eNB are regarded as the highest-quality sources. SyncRef UEs are distinguished between those which are directly synchronized to GNSS or an eNB, those which are 1 further step away, and those which are ≥2 further steps away from GNSS/eNB. As a last resort, a UE unable to find any other synchronization reference will use its own internal clock to transmit SLSS. The V2X synchronization procedure defines a hierarchy or set of priorities among such synchronization references and requires all UEs to continuously search the hierarchy to get to the highest-quality one they can find. The general preference order is as follows, with details specified in [10, section 5.10.8]:

Level 1.	Either GNSS or eNB, according to (pre-)configuration.

Level 2.	A SyncRef UE directly synchronized to a Level 1 source.

Level 3.	A SyncRef UE synchronized to a Level 2 source, i.e. indirectly synchronized to a Level 1 source.

-	When the Level 1 preference is (pre-)configured as GNSS, in Level 2 and 3, SyncRef UEs in hierarchies derived from GNSS and eNB are of equal preference as a synchronization source.

Level 4.	Any other SyncRef UE.

Level 5.	UE’s internal clock.

Within a Level 2, or 3, or 4 set of SyncRef UEs, the one with the highest S-RSRP is selected. The hierarchy also allows fallback between eNB-derived and GNSS-derived chains should the Level 1 type chosen by (pre-)configuration be unsuitable.

5.1.2.2	SLSS

The transmission and reception of SLSS (and PSBCH) is an optional V2X UE capability, and they can be transmitted on one or multiple synchronization carriers.

A UE which is deriving its own synchronization from eNB or GNSS can be configured by the network to transmit SLSS on a synchronization carrier, or the network can permit it to do so if the RSRP of a reference cell falls below a threshold. If the network is not available, and the UE is synchronized to GNSS, it will transmit SLSS if permitted by its pre-configuration. When such a UE does not have GNSS synchronization, it will transmit SLSS, if permitted by its pre-configuration, while it cannot find a SyncRef UE with sufficiently high S-RSRP. Full details of the conditions for SLSS transmission are specified in [10, section 5.10.7.2].

In a V2X system, there can be a variety of UEs which are deriving synchronization from different sources, and since a UE can transmit SLSS on multiple carriers, it can have different synchronization sources among them. SLSS is therefore transmitted in different subframes depending on what synchronization source the UE is using. The system can be configured with either two or three different offsets with respect to the start of the 160 ms PSBCH period can be defined, which can allow different subframes for transmission according to whether the UE is synchronized to GNSS, eNB, or another SyncRef UE.

PSSS and SSSS use the same sequences as PSS and SSS, respectively, with different root indices. When in use, both sequences are transmitted in the central 62 subcarriers of the SL bandwidth, in the same subframe as PSBCH, i.e. every 160 ms. PSSS is in the 2nd and 3rd symbols of the subframe, and SSSS in the 12th and 13th symbols. The two symbols of each signal are the same, which allows detectors to benefit from phase tracking between the two symbols. Figure 5.1.2.1-1 shows the relevant contents of a subframe which contain PSBCH, PSSS, and SSSS.

Figure 5.1.2.2-1: Contents of an LTE-V2X synchronization subframe.

5.1.2.2.1	SLSSID

The SLSSID itself conveys information about the synchronization source of the transmitting UE. In general, the further a UE is away from a high-quality source of GNSS or eNB, the lower quality will be its own synchronization and thus the quality of an SLSS it transmits. There are a series of association rules among SLSS IDs, designed to allow the identification, and propagation through the system of, high-quality synchronization sources. Thus, for example, a UE which is in-coverage and directly synchronized to an eNB (a Level 2 SyncRef UE) uses an SLSSID which is configured by the network from 1-167, allowing SLSSID planning, and a UE using such a UE as a SyncRef UE (a Level 3 SyncRef UE) uses the same SLSSID, but indicates in V2X MIB that it is not directly eNB-synchronized. Subsequent UEs  in this example hierarchy (Level 4 SyncRef UEs) use the SLSSID+168 to indicate their source is not directly synchronized to an eNB. A similar propagation hierarchy is used starting with a UE which is directly synchronized to GNSS, whose SLSSID is always 0, resulting in any UE which is more than one further step from GNSS using SLSSID 168 (or, in some cases, 169) so that a synchronization hierarchy based on GNSS can always be identified. Finally, a UE which cannot find GNSS, eNB, nor a SyncRef UE, selects an SLSSID randomly from 170-335, and UEs which synchronize to it propagate the same SLSSID.

5.1.3	Multi-carrier operation and carrier aggregation

V2X operation is defined in band 47 [11, section 5.5G], which supports single-carrier and multi-carrier operation:

Table 5.1.3-1: V2X operating band

| E-UTRA Operating Band | E-UTRA V2X Operating Band | V2X UE transmit |  |  | V2X UE receive |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| 47 | 47 | 5855 MHz |  | 5925 MHz | 5855 MHz |  | 5925 MHz | HD | PC5 |

The V2X sidelink in band 47 can be operated concurrently with Uu FDD bands 3, 5, 7, 8, 20, 28, and Uu TDD bands 34, 39, 41, and 71.

Sidelink CA is defined for resource allocation modes 3 and 4. When operating in CA, a given (sidelink) MAC PDU is transmitted, and if necessary re-transmitted, on a single sidelink carrier, and multiple MAC PDUs can be transmitted in parallel on different carriers. This provides a throughput gain in a similar way as for Uu CA.

Sidelink CA in resource allocation mode 3 using a dynamic grant is similar to on the Uu interface, by including a carrier indication field (CIF) in the DCI from the eNB. This indicates which among the up to 8 configured sidelink carriers the allocation in the DCI applies to.

Sidelink CA in resource allocation mode 4 uses the sensing procedure described in Section 5.2.2.2 to select resources independently on each involved carrier. The same carrier is used for all MAC PDUs of the same sidelink process at least until the process triggers resource re-selection. Procedures to avoid unexpected UE behaviour when the demands of CA become high are also specified, which allow a UE to drop a transmission which uses an unmanageable amount of resources or transmit chains, or to reject and re-select resources for which it cannot meet the RF requirements under CA.

Sidelink synchronization can also operate on multiple carriers, as mentioned in Section 5.1.2. In addition, in sidelink CA operation, a SyncRef UE uses a single synchronization reference for all aggregated carriers, and may transmit SLSS/PSBCH on one or multiple of them according to capability. A receiving UE likewise uses the same synchronization reference (not necessarily a SyncRef UE) for all its aggregated carriers, and it uses the highest priority synchronization reference present among the available synchronization carriers.

Another form of CA is PDCP duplication, where the same PDCP packet is transmitted in parallel on multiple sidelink carriers, to increase reliability. See Section 5.4.7 for more discussion.

5.2	V2X sidelink resource allocation

5.2.1	Resource pools

PSCCH and PSSCH resources are defined within resource pools for the respective channels. This concept is used because in general PSCCH/PSSCH cannot be transmitted (and thus are not expected to be received) in all PRBs and subframes in the system bandwidth, nor within a given frequency span configured for V2X sidelink. The notion of a resource pool also reflects, in resource allocation mode 4, that a UE will make its resource selections based on sensing within the (PSSCH) pool.

For PSSCH, a resource pool is divided into sub-channels in the frequency domain, which are consecutively non-overlapping sets of ≥4 PRBs in a subframe, the size depending on (pre-)configuration. Resource allocation, sensing, and resource selection are performed in units of a sub-channel. A sub-channel is allocated to a single UE’s PSSCH, although it is possible that a small number of PRBs within a sub-channel are not used by the transmission.

For PSCCH, the resource pool definition depends on whether adjacent or non-adjacent transmission of PSCCH and PSSCH has been (pre-)configured for the resource pool. Adjacent transmission is achieved by defining the lowest pair of PRBs in each subchannel as candidates to contain PSCCH, and PSSCH is transmitted in the contiguous PRBs above them for the allocated number of subchannels. Non-adjacent transmission is achieved by defining sequential pairs of PRBs starting at the lowest frequency of the resource pool for PSCCH, and associating the nth pair to PSSCH transmission occupying from the nth PSSCH sub-channel for the allocated number of subchannels. Figure 5.2.1-1 shows examples of adjacent and non-adjacent transmission. To receive PSCCH, a UE has to monitor each defined pair of PRBs to determine whether PSCCH has been transmitted in them.

Figure 5.2.1-1a: Illustration of adjacent allocations of PSCCH and associated PSSCH.

Figure 5.2.1-1b: Illustration of non-adjacent allocations of PSCCH and associated PSSCH.

Resource pools are (pre-)configured to a UE separately from the transmission perspective (TX pools) and the reception perspective (RX pools). This allows a UE to monitor for PSCCH, and hence receive PSSCH transmissions, in resource pools other than those in which it transmits, so that it can attempt to receive transmissions made by other UEs in those RX pools.

In addition, there are exceptional resource pools configured to a UE, in its serving cell's SIB21 or in dedicated signalling. These can be used e.g. during RLF, handover, transition from RRC IDLE to RRC CONNECTED, or during change of dedicated V2X sidelink resource pools within a cell. In these cases, a UE may not have a stable configuration of TX resource pools but nevertheless should not be removed from the V2X system, so it can randomly select resources in the exceptional pool provided in its serving cell's SIB21 or in dedicated signalling, and use them temporarily. Likewise, UEs need to monitor the exceptional pools for PSCCH transmissions.

5.2.2	Resource allocation modes

LTE sidelink defines four resource allocation modes: 1, 2, 3, and 4. Modes 1 and 2 apply to D2D, while modes 3 and 4 apply to LTE V2X.

5.2.2.1	Resource allocation mode 3

Mode 3 is for resource allocation scheduled by eNB. As described in Section 4, among the primary use cases for LTE-V2X is transmission of periodically-occurring messages, and for this reason there is particular support for sidelink SPS. However, dynamic sidelink grants are equally supported.

The eNB scheduling activity is driven by the UE needing to send data on sidelink performing a sidelink BSR procedure similar to that on Uu to request a sidelink resource allocation from eNB. Depending on the type of traffic the UE has, eNB can provide a dynamic sidelink grant, or an activation of a SPS sidelink grant.

A dynamic sidelink grant DCI provides, amongst other details [9, section 5.3.3.1.9A], the resources for up to two transmissions of the same TB. This is to allow higher reliability to be achieved without a feedback-based HARQ procedure, since LTE-V2X physical layer supports only broadcast transmission. Unlike Uu UL grants, the MCS information can optionally be provided by RRC signalling instead of the traditional DCI. For the case where RRC does not provide the MCS,  the transmitting UE is left to select an appropriate MCS/TBS itself based on the knowledge it has of the TB to be transmitted and, potentially, the sidelink radio conditions. The transmitting UE populates its SCI [9, section 5.4.3.1.2] with the information from eNB and other fields related to sidelink operation, and then transmits it and the associated PSSCH.

The UE can be configured by eNB with up to 8 sidelink SPS configurations. Each configuration has an identifying index, and provides a different periodicity of sidelink transmission resource. A sidelink SPS configuration is not used by the UE until the eNB sends the UE a DCI which indicates it is now active. The activating DCI also provides all the same fields as a dynamic sidelink scheduling DCI described above, allowing the precise resource allocation for SPS to be determined by eNB at the time it is needed. A transmitting UE can use the thus activated sidelink SPS resources, at the configured periodicity, until they are released (i.e. de-activated) by the eNB transmitting a special DCI. Note that each time the UE uses the resources, it either uses the RRC configured MCS/TBS or selects one itself, i.e. the same way as dynamic operation.

5.2.2.2	Resource allocation mode 4

Mode 4 is for UE autonomous resource selection. Its basic structure is of a UE sensing, within a (pre-)configured resource pool, which resources are not in use by other UEs with higher-priority traffic, and choosing an appropriate amount of such resources for its own PSCCH/PSSCH transmission. Having selected such resources, the UE can transmit in them on a periodic (i.e. SPS) basis for a certain number of times, or until a cause of resource reselection is triggered.

As described in Section 5.2.2.1, the SCIs transmitted by UEs on PSCCH indicate the time-frequency resources in which the UE will transmit a PSSCH. The same SCI contents is used in mode 4, and also indicates the periodicity with which the UE will use the same resources. These SCI transmissions are used by sensing UEs to maintain a moving sensing window in the immediate past of which resources have been reserved by other UEs. This window is 1000 ms long for FDD systems. A sensing UE also measures the PSSCH-RSRP in the subframes of the sensing window, which implies the level of interference which would be caused and experienced if the sensing UE were to transmit in them.

The sensing UE then selects resources for its first transmission from within a resource selection window. This window begins ≤4 ms after the trigger for transmission, and is bounded by the latency requirement of the traffic, up to 100 ms (see Section 4). The sensing UE assumes the same resources will be used by other UEs in the future as have been found reserved during the sensing window, according to the indicated periodicities and durations. Reserved resources in the selection window with PSSCH-RSRP above a threshold are excluded from being candidates by the sensing UE, with the threshold set according to the priorities (PPPP) of the traffic of the sensing and transmitting UE. Thus, a higher priority transmission from a sensing UE can occupy resources which are reserved by a transmitting UE with sufficiently low PSSCH-RSRP and sufficiently lower-priority traffic.

From the set of resources in the selection window which have not been excluded, the sensing UE identifies those containing the lowest total received energy as a way to account for transmissions which were not found during decoding of PSCCHs, and identifies resources totalling 20% of the available resources within the traffic’s latency bound, including gradual relaxation of the PSSCH-RSRP exclusion thresholds if necessary. The UE then selects a resource at random from the identified 20% and uses this resource semi-persistently for its transmissions.

There are a number of triggers for resource re-selection. They are designed to support high mobility, and ensure that a UE cannot assume occupation of a resource for an excessive period, nor when the selected resource is either insufficient or excessive for what is needed by the UE’s traffic, amongst other causes.

Figure 5.2.2.2-1: Summary of sensing and resource (re-)selection procedures.

5.2.2.2.1	Zones

Cellular networks are designed to support resource re-use over a given geographical area, to manage interference and improve area spectral efficiency. A similar concept is available in LTE-V2X. LTE-V2X can optionally divide the world into zones, which have a (pre-)configurable width and height. Nearby zones are assigned different resource pools, with spatially-periodic re-use over a distance controlled by the zone width and height. A UE performs transmission within the resource pool(s) associated with its current zone, for mode 3. Figure 5.2.3-1 shows an example of the configuration of zones, where those with the same zone_id use the same transmission resource pools.

Figure 5.2.3-1: Illustration of zones. x’ = ceil(x/L) mod Nx; y’ = ceil(y/W) mod Ny; zone_id= y’×Nx+x’; where x and y are the longitude and latitude of the UE’s location, L, W are the length and width of each zone, respectively, and Nx, Ny are the number of zones in length and width respectively.

5.2.2.3	Modes 3 and 4 resource pool sharing

Resource pools for mode 3 and mode 4 are (pre-)configured separately from one another. In  Rel-14, a receiving UE whether operating in mode 3 or mode 4 does not know in any particular way that a transmission in mode 3 has SPS characteristics, since the mode 3 SCI always indicates that its PSSCH transmission is single-shot. Thus, a mode 4 UE will not exclude the occupied resources during its resource selection procedure. In Rel-15, resource pool sharing is enabled by the mode 3 SCI also indicating a SPS reservation period, and mode 4 UEs of both releases are therefore able to account for such transmissions in the resource (re-)selection procedure.

It is also possible for a UE operating in mode 3 to perform the mode 4 sensing procedure in order to report to eNB regarding the occupancy of resources in the mode 4 pools. This provides information which the eNB can use in determining its scheduling behaviour for mode 3.

5.3	Sidelink congestion control

Each sidelink packet is associated with a PPPP and PPPR passed down ultimately to the physical layer from upper layers, which determine the values based on the QoS requirements of the message. PPPR allows management of reliability, for example see Section 5.4.7. The PPPP of a packet is indicated dynamically in the SCI which schedules the PSSCH for the packet. A physical measurement of CBR is also defined in each subframe [12, section 5.1.30], which measures the portion of the resource in a resource pool which has a high received signal energy (S-RSSI) in the most recent 100 subframes. CBR is a measurement of the congestion present recently in the resource pool. Another measurement, CR [12, section 5.1.31], counts the total number of subchannels a UE has and will transmit in during a window of up to 1000 ms including the current subframe. CR is thus a measurement of how much resource a UE has recently, and will soon, claim.

A UE can be (pre-)configured with a set of CBR ranges to each of which is linked a CR-limit. When a UE finds its CR exceeds the CR-limit for the CBR range it currently measures, it  must reduce its CR to not exceed the limit. How this is done is up to UE implementation, and can include increasing MCS to reduce resource occupation, dropping (re-)transmissions, etc. PPPP can also be (pre-)configured with a mapping to the UE’s maximum permitted transmit power, the limitation on which acts to reduce the CBR measured by sufficiently distant UEs.

PPPP is used as described in Section 5.2.2 to aid distributed sidelink congestion control based on the relative priorities of traffic from UEs that consider occupying a given resource. PPPP and CBR can each also be (pre-)configured with mappings to ranges of values of transmission parameters, e.g. a range of MCS values, and/or a range of numbers of subchannels, etc. In this case, the UE has to choose its transmission parameters from within the range corresponding to the prevailing PPPP and/or CBR.

5.4	V2X sidelink higher-layer protocols

5.4.1	General

Figure 5.4.1-1 shows the user plane protocol stack for V2X sidelink communication. The Access Stratum protocol stack for user plane in the PC5 interface consists of PDCP, RLC, MAC and PHY as shown below in Figure 5.4.1-1. The detailed functions performed by PDCP, RLC and MAC sublayers are listed in [13, section 6], and some other details specific to V2X sidelink are described in [13, sections 23.10.2.1 and 23.14.1.1]. The PHY layer performs the functions described in Section 5.1.

Figure 5.4.1-1: User-Plane protocol stack for V2X sidelink communication

Figure 5.4.1-2 shows the control plane protocol stack for V2X sidelink communication. The Access Stratum protocol stack for control plane in the PC5 interface consists of RRC, RLC, MAC and PHY as shown below in Figure 5.4.1-2. The control plane protocol for V2X sidelink communication is mainly used for the transmission of sidelink broadcast control channel (SBCCH), and its functions are detailed in [13, section 23.10.2.1].

Figure 5.4.1-2: Control-Plane protocol stack for V2X sidelink communication

5.4.2	Resource pool configuration

When the UE is in network coverage, it may use the resource configurations provided by the eNB via dedicated signalling or system information. When the UE is out of coverage, it may use pre-configured resources for V2X sidelink communication. Details of how the resources are (pre)configured and how the UE uses them for V2X sidelink communication transmission and reception are specified in [13, sections 23.10.2.1 and 5.10.13].

5.4.3	Measurement and reporting

Some measurement and reporting mechanisms are supported specifically for V2X sidelink communication, including CBR measurement and reporting, reporting of its sensing results and geo-location reporting. Details of above listed measurement and reporting mechanism specific for V2X sidelink communication are specified in [10, section 5.5].

5.4.4	Mobility management

UE can perform V2X sidelink transmission and reception during handover and cell reselection. Related details are specified in [10, section 5.10.13.1].

5.4.5	Assistance information for SL SPS configuration

The UE can report UE assistance information to the eNB to facilitate the configuration of sidelink SPS described in Section 5.2.2.1. Details of the UE assistance information for sidelink SPS configuration are specified in [10, section 5.6.10].

5.4.6	Transmission carrier selection

For V2X sidelink communication, the UE is allowed to transmit and receive on multiple sidelink carriers (pre)configured by the network, and the UE selects specific sidelink carriers among them for transmission. The (pre)configured sidelink carriers can be provided by the network as specified in [10, section 5.10.13.1]. Details of transmission carrier selection are specified in [14, section 5.14.1.5].

5.4.7	SL packet duplication

To improve the reliability of V2X sidelink communication, a sidelink PDCP duplication mechanism is supported, where a PDCP PDU is duplicated into two instances and transmitted on two different sidelink carriers. Details of sidelink PDCP duplication are specified in [15, section 5.1.3 and 5.1.4] as well as [14, section 5.14.1.3.1].

5.4.8	Coordination between UL and V2X SL transmission

There is the possibility that the UE needs to perform UL transmission and V2X sidelink transmission overlapped in time domain, but the UE cannot transmit both. In this case, the UE will prioritize between the two transmissions. Details of the prioritization between UL transmission and V2X sidelink transmission are specified in [14, sections 5.4.2.2 and 5.14.1.2.2].

5.4.9	Multi-PLMN operation

To support multi-PLMN operation for V2X sidelink communication, the UE may receive the V2X sidelink communication of other PLMNs. However, V2X sidelink transmission in other PLMNs is not allowed. The same sidelink resource pool may be configured to the eNBs within a PLMN. Details of multi-PLMN operation for V2X sidelink communication are specified in [10, section 23.14.1.1].

5.5	V2X via the Uu interface

LTE-V2X, as mentioned above, is focused on transmission of a few types of periodic traffic, which can have different periodicities. Apart from using the sidelink, it is possible to transfer V2X data via the cellular network UL/DL, taking advantage of its centrally-scheduled operation, predictable latency bounds, and inter-cell communications.

An extended form of UL SPS for V2X is supported, to allow firstly that V2X UL SPS and normal UL SPS can be used together, and also that multiple V2X UL SPS configurations can be defined to account for the different periodic V2X traffic types. Whereas in Rel-8 UL SPS a single SPS configuration can be provided by RRC, for LTE-V2X, up to 8 UL SPS configurations can be provided, similar to on the sidelink, which the eNB is assisted in the configuration of by information on traffic characteristics, etc., reported provided by the UE. A specialised version of DCI format 0 is then used to activate/release UL V2X SPS configurations as needed, and separately from the normal UL SPS configuration.

LTE supports multicast/broadcast operation via SC-PTM and MBSFN transmission. These transmission types can also be used for V2X messages, and some adaptations for LTE-V2X are included to allow scheduling more frequent transmissions of data and service (re-)configuration information.

5.6	Network aspects

5.6.1	V2X service authorization

Considering the different UE types based on subscription information in the HSS, the eNB receives the authorization status of the UE provided by a MME or a neighbour eNB, to know whether the UE is authorized as a Vehicle UE and/or a Pedestrian UE.

Only authorized UEs can perform V2X sidelink communication. Sidelink radio resources are provided to UEs in different ways according to the different services supported.

5.6.2	Sidelink AMBR

UE sidelink AMBR is provided to the eNB by a MME or a neighbour eNB, and used for the UE’s sidelink communication in mode 3. The sidelink AMBR is based on subscription information from the HSS and the operator’s policy, which is used to cap the PC5 transmission from a particular UE.

Figure 5.6-1 Network Infrastructure for LTE V2X.

6	NR V2X

Explanatory note: The detail of the following section headings depends on further agreements during Rel-16.

6.1	Sidelink unicast, groupcast, and broadcast

Explanatory note: Summary of functional and feature contents and differences for/among unicast, groupcast, and broadcast.

6.2	V2X sidelink physical layer

6.2.1	Physical sidelink channels and signals

The NR V2X sidelink uses the following physical channels and signals:

-	Physical sidelink broadcast channel (PSBCH) and its DMRS, specified in [16, sections XX and XX]

-	Physical sidelink control channel (PSCCH) and its DMRS, specified in [16, sections XX and XX]

-	Physical sidelink shared channel (PSSCH) and its DMRS, specified in [16, sections XX and XX]

-	Physical sidelink feedback channel (PSFCH), specified in [16, section XX]

-	Sidelink primary and secondary synchronization signals (S-PSS and S-SSS) specified in [16, section XX], which are organized into the sidelink synchronization signal block (S-SSB) together with PSBCH. S-PSS and S-SSS can be referred to jointly as the sidelink synchronization signal (SLSS).

-	Phase-tracking reference signal (PT-RS), specified in [16, section XX].

-	Channel state information reference signal (CSI-RS), specified in [16, section XX]. See section 6.2.3 for CSI.

NR-V2X sidelink supports the same subcarrier spacings, and their associations to CPs and frequency ranges, as for NR UL/DL, but uses only the CP-OFDM waveform.

PSBCH transmits the SL-BCH transport channel, which carries the sidelink V2X Master Information Block (MIB-V2X) from the RRC layer. When in use, PSBCH transmits MIB-V2X every 160 ms in 11 RBs of the SL bandwidth [add information of S-SSB raster as appropriate], with possible repetitions in the period. DMRS associated with PSBCH are transmitted in the [XXth] symbols of the slot. S-PSS and S-SSS are transmitted together with PSBCH in the S-SSB. They [jointly convey the SLSS ID used by the UE, and …]. For further details of S-SSB, SLSS, and synchronization, refer to Section 6.2.2.

PSSCH transmits the SL-SCH transport channel, which carries the TBs of data for transmission over SL. The resources in which PSSCH is transmitted can be scheduled or configured by a gNB (termed resource allocation mode 1, see Section 6.3.2.1) or determined through a sensing procedure conducted autonomously by the transmitting UE (termed resource allocation mode 2, see Section 6.3.2.2). A given TB can be transmitted multiple times, as described in Sections 6.2.4 and 6.3.2. DMRS associated with PSSCH [are…].Sidelink control information (SCI) in NR V2X is transmitted in two stages. The first-stage SCI is carried on PSCCH and contains information to enable sensing operations, as well as information about the resource allocation of the second-stage SCI. [More summary to be added later]. Multiplexing between PSCCH and PSSCH is in time and frequency within a slot, illustrated generally as shown in Figure 6.2.1-1.

[add when 1st stage and 2nd stage can be illustrated].

Figure 6.2.1-1: General relationship between 1st-stage SCI in PSCCH, 2nd-stage SCI, and PSSCH.

PSFCH carries HARQ feedback over the sidelink from a UE which is an intended recipient of a PSSCH transmission (henceforth an Rx UE) to the UE which performed the transmission (henceforth a Tx UE). Sidelink HARQ feedback may be in the form of conventional ACK/NACK, or NACK-only with nothing transmitted in case of successful decoding; for more information refer to section 6.2.4. PSFCH transmits a  Zadoff-Chu sequence [according to…] in one OFDM symbol in [the last few sidelink symbols of a slot, etc,.]. The time resources for PSFCH are (pre-)configured to occur in every 1, 2, or 4 slots, and the frequency/code resources are derived implicitly from those used by the associated PSSCH transmission.

6.2.2	Sidelink synchronization

6.2.2.1	Synchronization references and priorities

There are four basic sources, or references, from which a V2X UE can derive its own synchronization: GNSS, a gNB/eNB, another UE transmitting SLSS (here termed a SyncRef UE), or its own internal clock. In general, GNSS or eNB/gNB are regarded as the highest-quality sources. SyncRef UEs are distinguished between those which are directly synchronized to GNSS or a gNB/eNB, those which are 1 further step away, and those which are ≥2 further steps away from GNSS or gNB/eNB. As a last resort, a UE unable to find any other synchronization reference will use its own internal clock to transmit S-SSB. The V2X synchronization procedure defines a hierarchy or set of priorities among such synchronization references [pending agreements: …and requires all UEs to continuously search the hierarchy to get to the highest-quality one they can find]. The general preference order is as follows, with details specified in [17, section 5.10.8]:

Level 1.	Either GNSS or eNB/gNB, according to (pre-)configuration.

Level 2.	A SyncRef UE directly synchronized to a Level 1 source.

Level 3.	A SyncRef UE synchronized to a Level 2 source, i.e. indirectly synchronized to a Level 1 source.

Level 4.	Whichever of GNSS or eNB/gNB was not (pre-)configured as the Level 1 source.

Level 5.	A SyncRef UE directly synchronized to a Level 4 source.

Level 6.	A SyncRef UE synchronized to a Level 5 source, i.e. indirectly synchronized to a Level 4 source.

Level 7.	Any other SyncRef UE.

Level 8.	UE’s internal clock.

The NR V2X scheme is intended to allow the merging of otherwise-separate hierarchies derived from GNSS and gNB/eNB, so that a UE is able to move between nearby such hierarchies without loss of sidelink service. However, since it is possible that a gNB/eNB does not itself have synchronization to GNSS, use of Levels 4-6 can be disabled when GNSS is used as Level 1, so that there is no deviation from the hierarchy being derived from GNSS.

6.2.2.2	S-SSB and SLSS

S-PSS and S-SSS use the same types of sequence as NR PSS and SSS respectively, i.e. an M-sequence and a Gold sequence. When in use, both sequences are transmitted [in the XX subcarriers of the S-SSB bandwidth], in the same slots as PSBCH. S-PSS is in the 2nd and 3rd symbols of the slot, S-SSS is in the 4th and 5th symbols, and PSBCH and its DMRS in the remaining symbols. The two symbols of each synchronization signal are the same, which allows detectors to benefit from phase tracking between the two symbols [note: working assumption]. Figure 6.2.2.2-1 shows the contents of a slot containing S-SSB.

[add when DMRS locations are known]

Figure 6.2.2.2-1: Contents of a slot containing S-SSB for normal CP.

The MIB-V2X is transmitted on PSBCH every 160 ms, and thus S-SSB occurs on the same period. Within a period, there can be multiple transmissions of S-SSB, with the number depending on subcarrier spacing and frequency range. The purpose of this is to provide approximately the same maximum sidelink coverage for all subcarrier spacings and bands. For details refer to [XX, section XX].

6.2.2.2.1	SLSSID

The SLSSID itself conveys information about […].

6.2.3	Sidelink CSI

To provide the Tx UE with information it can use for sidelink link adaptation and rank adaptation in unicast transmissions, the Tx UE can configure aperiodic sidelink CSI reporting from the Rx UE. CQI and RI are reported, in a PSSCH transmission. Calculation of the reported values is enabled by the Tx UE transmitting sidelink CSI-RS.

6.2.4	Sidelink HARQ

Explanatory note: This section may include content relevant to PHY and MAC.

NR-V2X supports HARQ based on transmission of ACK/NACK (or DTX) for sidelink unicast and groupcast services, as well as a NACK-only HARQ scheme particular to groupcast services. In addition, it supports blind re-transmission schemes, which are described in sections 6.3.2.1 and 6.3.2.2 on resource allocation modes 1 and 2, respectively.

Sidelink HARQ feedback is carried on PSFCH from an Rx UE to its Tx UE. [Add stage-2 information about how it is carried when further agreed]. In addition, when under the control of a gNB in resource allocation mode 1, the Tx UE can forward the sidelink HARQ feedback to the gNB via PUCCH to assist the scheduling of re-transmissions and allocation of sidelink resources.

When ACK/NACK (or DTX) operation is used, the HARQ procedure is similar to the Uu scheme for non-codeblock group feedback, i.e. the ACK/NACK is delivered based on the success or failure of the whole transport block.

NACK-only operation is defined for groupcast to allow a potentially lower sidelink resource demand to be created when a larger number of Rx UEs need to send feedback to the same Tx UE. A typical use case is an extended sensors scenario where UEs within a given radius all receive the same sensor information from the Tx UE, and re-transmission will occur if any UE fails to decode successfully. Since such information may only be relevant within a given radius around the Tx UE (e.g. a few tens or hundreds of meters around a road junction), the transmission of NACK-only feedback can be restricted to UEs within such a radius, and any UE beyond it does not provide any HARQ feedback. The minimum range requirement of a service is provided together with the associated QoS parameters from service layers.

6.2.5	In-device coexistence between LTE-V2X and NR-V2X sidelinks

It is envisaged that there will exist devices that support both LTE-V2X and NR-V2X, and which will be operating in both systems concurrently. If the two RATs are widely spaced in frequency, e.g. being in different bands, then there need be no particular issues to consider since it is assumed that a separate RF chain will be provided for each band.

If, however, a sufficiently close frequency spacing is deployed, then it is desirable to enable a single RF chain to be used in the implementation, and also to adhere to the sidelink half-duplex principle established in LTE-V2X, i.e.  that the UE is not required to simultaneously transmit and receive on sidelink. The former constraint means that interference between the two RATs’ receptions can occur in the device if they are placed sufficiently close together in the frequency domain, and that simultaneous transmission on both RATs is prevented by the UE’s single power budget. The latter constraint implies that one RAT cannot be received/transmitted while the other RAT is doing the opposite.

It is possible to (pre)-configure the resource pools for the two sidelinks such that overlapping is avoided entirely. When this is not arranged, the general rule is that (at least) one of the RATs may be dropped at times when both occur simultaneously, but that in some cases where the priority of the V2X service on both RATs is known, the higher priority one is automatically selected:

-	Receive/receive overlap handling is always left to UE implementation decision.

-	Transmit/transmit overlap handling will automatically select the RAT with the higher priority service if both priorities are known. If the priorities are equal or not both known, he handling is left to UE implementation to decide.

-	Transmit/receive overlap will […pending agreement…]  if both priorities are known. Otherwise, the handling is left to UE implementation to decide.

6.3	V2X sidelink resource allocation

6.3.1	Sidelink bandwidth parts and resource pools

Temporary note: Any relationship between resource pools and PSFCH will be clarified after agreements.

6.3.1.1	Sidelink bandwidth parts

BWPs are defined for the sidelink in a similar way as for UL/DL, to provide a convenient way to specify aspects relating to a UEs RF hardware chain implementation. In sidelink, a UE is configured with one active sidelink BWP when in connected mode to a gNB, and uses a single sidelink BWP for idle mode or out-of-coverage operation.

The subcarrier spacing used on sidelink is provided in the sidelink BWP (pre-)configuration, from the same set of values and associations to frequency ranges as for the Uu interface (i.e. 15, 30, or 60 kHz for FR1; and 60 or 120 kHz for FR2).  Sidelink transmission and reception for a UE are thus contained within a sidelink BWP, and the same sidelink BWP is used for both transmitting and receiving. This means that resource pools, S-SSB, etc. must also be contained within an appropriate sidelink BWP from the UE’s perspective.

6.3.1.2	Resource pools

PSCCH and PSSCH resources are defined within resource pools for the respective channels. This concept is used because in general PSCCH/PSSCH cannot be transmitted (and thus are not expected to be received) in all RBs and slots in the NR system bandwidth, nor within the frequency span configured for V2X sidelink. The notion of a resource pool also reflects, in resource allocation mode 2, that a UE will make its resource selections based on sensing within the pool.

[Summary to be added after more agreements, including information about subchannels, slot formats, etc.].

Resource pools are (pre-)configured to a UE separately from the transmission perspective (TX pools) and the reception perspective (RX pools). This allows a UE to monitor for PSCCH, and hence receive PSSCH transmissions, in resource pools other than those in which it transmits, so that it can attempt to receive transmissions made by other UEs in those RX pools.

In addition, there are exceptional resource pools configured to a UE, in its serving cell's broadcast or in dedicated signalling. These can be used e.g. during RLF, handover, transition from RRC IDLE to RRC CONNECTED, or during change of dedicated V2X sidelink resource pools within a cell. In these cases, a UE may not have a stable configuration of TX resource pools but nevertheless should not be removed from the V2X system, and so it can randomly select resources in the exceptional pool, and use them temporarily. Likewise, UEs need to monitor the exceptional TX pools for PSCCH transmissions.

6.3.2	Resource allocation modes

6.3.2.1	Mode 1

Mode 1 is for resource allocation by gNB. As described in Section 4, the use cases intended for NR V2X can generate a diverse array of periodic and aperiodic message types. Therefore, resource allocation mode 1 provides dynamic grants of sidelink resources from a gNB, as well as grants of periodic sidelink resources configured semi-statically by RRC, termed sidelink configured grants.

A dynamic sidelink grant DCI can provide resources for one or multiple transmissions of a transport block, in order to allow control of reliability. The transmission(s) can be subject to the sidelink HARQ procedure, if that operation is enabled.

A sidelink configured grant can be such that it is configured once and can be used by the UE immediately, until it is released by RRC signalling (known as Type 1). A UE is allowed to continue using this type of sidelink configured grant when beam failure or physical layer problems occur in NR Uu […until...]. The other type of sidelink configured grant is configured once but cannot be used until the gNB sends the UE a DCI indicating it is now active, and only until another DCI indicates de-activation (known as Type 2). The resources in both types are a set of sidelink resources recurring with a periodicity which a gNB will desire to match to the characteristics of the V2X traffic.

The gNB scheduling activity is driven by the UE reporting its sidelink traffic characteristics to the gNB, or by performing a sidelink BSR procedure similar to that on Uu to request a sidelink resource allocation from gNB.

6.3.2.2	Mode 2

Mode 2 is for UE autonomous resource selection. Its basic structure is of a UE sensing, within a (pre-)configured resource pool, which resources are not in use by other UEs with higher-priority traffic, and choosing an appropriate amount of such resources for its own transmissions. Having selected such resources, the UE can transmit and re-transmit in them a certain number of times, or until a cause of resource reselection is triggered.

The mode 2 sensing procedure can select and then reserve resources for a variety of purposes reflecting that NR V2X introduces sidelink HARQ in support of unicast and groupcast in the physical layer. It may reserve resources to be used for a number of blind (re-)transmissions or HARQ-feedback-based (re-)transmissions of a transport block, in which case the resources are indicated in the SCI(s) scheduling the transport block. Alternatively, it may select resources to be used for the initial transmission of a later transport block, in which case the resources are indicated in an SCI scheduling a current transport block, in a manner similar to the LTE-V2X scheme (section 5.2.2.2). Finally, an initial transmission of a transport block can be performed after sensing and resource selection, but without a reservation.

The first-stage SCIs transmitted by UEs on PSCCH indicate the time-frequency resources in which the UE will transmit a PSSCH. These SCI transmissions are used by sensing UEs to maintain [add statements about sensing window when agreed]. A sensing UE also measures the sidelink RSRP in the slots of the sensing window, which implies the level of interference which would be caused and experienced if the sensing UE were to transmit in them.

The sensing UE then selects resources for its (re-)transmission(s) from within a resource selection window. The window is starts no earlier than […], and cannot be longer than the remaining latency budget of the packet due to be transmitted. Reserved resources in the selection window with sidelink RSRP above a threshold are excluded from being candidates by the sensing UE, with the threshold set according to the priorities of the traffic of the sensing and transmitting UEs. Thus, a higher priority transmission from a sensing UE can occupy resources which are reserved by a transmitting UE with sufficiently low sidelink RSRP and sufficiently lower-priority traffic.

From the set of resources in the selection window which have not been excluded, the sensing UE identifies […].There are a number of triggers for resource re-selection. They are […].

[To be added]

Figure 6.3.2.2: Summary of sensing and resource (re-)selection procedures

6.4	QoS framework and congestion control

Explanatory note: Discuss NR priority, CBR, CR, Range uses in RAN.

6.5	V2X sidelink higher-layer protocols

6.5.1	General

Explanatory note: General descriptions on some RAN higher layer aspects, e.g. protocol stacks, etc.

6.5.2	Measurement and reporting related to NR sidelink communication

Explanatory note: Including those designed for NR sidelink communication, e.g. CBR measurement and reporting, etc.

6.5.3	Mobility management for NR SL transmission/reception

6.5.4	Assistance information and SL configured grant configuration

6.5.5	Coordination between UL and NR SL transmission

Explanatory note: prioritisation between NR UL and NR SL.

6.5.6	QoS mechanism

Explanatory note: per flow based QoS model, etc.

6.5.7	Sidelink RRC

Explanatory note: AS layer configuration transfer, UE capability interaction, SL RLF, etc.

6.6	V2X via the Uu interface

Some advanced V2X services are provided by an application server residing in the Internet which processes information received from the UE and issues instructions back to control the vehicle. Such applications can allow a remote driver or a V2X application to operate a remote vehicle for those passengers who cannot drive themselves. Other services, termed advanced driving, can incorporate video sharing among vehicles via an application server. In such applications, the communication is UE to/from the server via a cellular network’s NR Uu interface.

Therefore, the Uu interface can activate multiple PUSCH configured grants (which are periodical resource grants configured by RRC, similar to LTE SPS in section 5.5) simultaneously so that traffic sources with different requirements on latency, reliability, etc. can be handled at the same time. It is also possible for a PUSCH configured grant to send more than one PUSCH repetition in a slot or across a slot boundary, for the purpose of reducing UL latency.

6.7	Network aspects

7	Multi-RAT V2X

Explanatory note: This section covers cross-RAT control of sidelink, and MR-DC aspects. The material and section structure will also depend on what RAN2 agree.

7.1	Cross-RAT operation

It is envisaged that NR-V2X and LTE-V2X will be at least initially deployed in similar timeframes, and also during a time when MNOs may have either or both of NR and LTE cellular networks available. An optional UE design response to this is supported where a device has both an LTE-V2X RAT and an NR-V2X RAT which are able to inter-communicate. This would enable the UE to perform sidelink communications with other UEs which implement only one of the sidelinks, and to do so whether they are in a cellular network of LTE or NR. Hence, for example, an LTE cellular network can be involved in controlling the NR sidelink via such dual-mode UEs, and an NR cellular network can similarly be involved in controlling LTE sidelink UEs.

LTE Uu can control NR resource allocation mode 1 by providing configured grant Type 1 configurations via LTE RRC signalling. This allows provision of periodically occurring sets of NR resources for the NR sidelink – see Section 6.3.2.1. Control of NR resource allocation mode 2 is by LTE Uu RRC providing the semi-static configurations relevant to resource pools, sensing, etc. within which the NR-V2X RAT autonomously selects resources for sidelink transmission.

NR Uu can control LTE resource allocation mode 3 by transmitting NR DCI which contains the information needed to dynamically control the LTE sidelink. The UE transfers this information internally to its LTE RAT, for which a certain amount of processing time is allowed, and then follows normal LTE sidelink procedures. Control of LTE resource allocation mode 4 is by NR Uu RRC providing the necessary semi-static configurations within which the LTE-V2X RAT autonomously selects resources for sidelink transmission.

7.2	MR-DC

7.3	Prioritization handling

8	Transmission profiles

LTE-V2X was developed over the course of two 3GPP Releases, Rel-14 and Rel-15. In Rel-15, non-backward compatible changes to physical transmission formats were introduced, primarily to support the use of 64-QAM on PSSCH compared to the maximum of 16-QAM in Rel-14. Such transmissions cannot be decoded by Rel-14 UEs. However, on sidelink, a transmitting UE cannot know the Release of specifications supported by the UEs which will receive the PSSCH transmission, since the transmission is broadcast in the physical layer. Therefore, the concept of transmission profiles was introduced, which associates to each transmission by a Rel-15 UE a (pre-)configuration of which Release’s transmission format to use. A transmitting UE performs transmission format selection based on the profile indicated by higher layer of the 3GPP protocol stack. If a Rel-14 format is indicated, then the transmission can be decoded by both Releases of UE, whereas if Rel-15 format is indicated, it can be decoded only by Rel-15 UEs. When a UE has traffic from different application sources to transmit at the same time, the transmission profile associated with the highest-priority traffic is applied. The association of transmission profiles to traffic sources (specifically, logical channels) is provided by 3GPP protocol layers outside of RAN.

In the present Release of this TR, transmission profiles are not defined in NR-V2X.

9	Pedestrian UE

As discussed in Section 4, LTE-V2X includes communications between Pedestrian LTE UEs and Vehicular LTE UEs, i.e. V2P. Whereas a vehicular LTE UE is assumed to be attached the vehicle’s power supply, and thus to have no particular battery life concerns, the situation is different for a P-UE. A P-UE could be, e.g. a conventional smartphone running suitable applications, or a specialised device attached to a pedestrians clothing, etc. In either case, battery life has to be considered so that the device will provide the LTE-V2P services for a reasonable length of time without need of re-charging, and without imposing such battery drain that LTE-V2P applications could become unattractive.

As described in Section 5.2.2.2, a LTE-V2X UE performs sensing continuously in a 1000 ms historical window, implying an amount of ongoing power consumption due to the sensing procedure. It is allowed for a LTE P-UE to not support sidelink reception, so that it is only broadcasting packets relating to its own location and direction. This type of P-UE is allowed to select transmission resources randomly, with no sensing procedure. For a LTE P-UE which does support sidelink reception, it can be (pre-)configured to perform partial sensing. In partial sensing, only a subset of the subframes in the typically 1000 ms sensing window have to be monitored. The LTE UE implementation can choose how few subframes it wishes to monitor, by trading off the reliability of its transmissions with the power saving, subject to monitoring a (pre-)configured minimum number. (Pre-)configuration can also set how far into the past the sensing window extends, and can require that the UE performs partial sensing in a number of these truncated sensing windows.

In the present Release of this TR, functions particular to Pedestrian UEs are not defined in NR-V2X.

10	Roadside unit

The development of LTE and NR V2X contemplates three entities in the network: base stations, i.e. eNBs and gNBs; UEs; and RSUs. An RSU is defined in [6], [7] as a stationary infrastructure entity supporting V2X applications that can exchange messages with other entities supporting V2X applications. Note that RSU is a term frequently used in existing ITS specifications, and the reason for introducing the term in the 3GPP specifications is to make the documents easier to read for the ITS industry. RSU is a logical entity that supports V2X application logic using the functionality provided by either a 3GPP network (referred to as an eNB/gNB-type RSU) or a UE (referred to as UE-type RSU).

Since an RSU is essentially only different from a eNB/gNB or UE in terms of its deployment nature, e.g. it may be deployed by a national roads authority instead of an MNO, there is no specification of features particular to RSUs. They operate simply according to sidelink or UL/DL specifications as appropriate.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-05 | RAN1#97 | R1-1906663 |  |  |  | Initial version of TR and skeleton of LTE-V2X sections | 0.0.1 |
| 2019-08 | RAN1#98 | R1-1909651 |  |  |  | LTE-V2X first version, and skeleton of NR-V2X sections | 0.1.0 |
| 2019-10 | RAN1#98bis | R1-1910616 |  |  |  | Addition of RAN2 and RAN3 LTE-V2X, and NR-V2X first version | 0.2.0 |