3GPP TSG-RAN WG2 Meeting #107bis	R2-1913825

# Chongqing, China, 14th  – 18th October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR |  | rev |  | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Running CR] Introduction of 5G V2X with NR Sidelink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL |  |  |  |  |  | Date: |  |  | 2019-10-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 5G V2X with NR sidelink is introduced in REL-16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This version of a running CR was drafted based on RAN1/2 agreements considering the structure and procedural texts of LTE sidelink parts in 36.321 that were modified or enhanced based on UL-SCH/DL-SCH procedures in 38.321 e.g. for HARQ feedback and alignment with 38.321 style. The following miscellaneous aspects are proposed to be discussed in offline discussion for progress of this CR: Issue 5.4.2.2-1 - In LTE UL/SL prioritization, if there are more than one UL transmission which the MAC entity is not able to perform simultaneously, it is up to UE implementation whether this UL transmission is performed. (These were copied from 36.321) Issue 5.8.x-1: - Can we reuse UL CG behaviour for cancelling the triggered configured sidelink grant confirmation? Issue 5.12-1: - Can MAC reset have any impact on Sidelink LCP? Issue 5.12-2: - Cancelling Sidelink BSR in the same way with cancelling Uu BSR upon MAC reset Issue 5.13-1: - In 36.321, the UE ignores the MAC PDU subheaders containing reserved values and the corresponding MAC SDUs. Issue 5.15-1: - Separation of SL BWP from UL/DL BWP under 5.15 Issue 5.x.1.1-1: - Need for PSCCH duration and PSSCH duration Issue 5.x.1.1-3: - It is proposed to introduce this new section in which conditions triggering TX resource re-selection are specified in a 5.x.1.2 in order to avoid lengthy repeated steps (see 36.321). Issue 5.x.1.2-1: - In LTE V2X, this triggering condition came from RAN1. It is proposed to come back to this condition if RAN1 agree to reuse probResourceKeep. Issue 5.x.1.2-2: - This triggering condition could be easily agreed in RAN2. The last second can be further discussed later. Issue 5.x.1.2-3: - This triggering condition could be easily agreed in RAN2. Issue 5.x.1.2-4: - This triggering condition could be easily agreed in RAN2. Issue 5.x.1.2-5: - This triggering condition could be easily agreed in RAN2. MCS parameter can be further discussed later. Issue 5.x.1.2-6: - This triggering condition could be easily agreed in RAN2. Issue 5.x.1.2-7: - This triggering condition could be easily agreed in RAN2. Issue 5.x.1.3.1-1: - Can we draft SL LCP procedure based on the UL LCP procedure (as shown in 5.x.1.3.1)? Issue 5.x.1.5-1: - Can we use this new acronym, SBSR for simplicity, instead of SL-BSR)? Issue 5.x.1.5-2: - Can we confirm these names for the listed parameters? Issue 5.x.1.5-3: - Whether or not logical channel SR mask is needed for SL Issue 6.1.4-1: - Transparent MAC is used for SL-BCH as in LTE V2X. Issue 6.1.x-1: - SL MAC subPDU(s) with MAC SDU(s) is placed after the SL-SCH subheader Issue 6.2.x-1: - Do we need V field for SL-SCH? Issue 7-1-1: - Can we use the same term ‘SL Semi-Persistent Scheduling V-RNTI’ with a new RNTI value for NR controlling LTE SL SPS? It is proposed to have the same RNTI name for both LTE controlling LTE sidelink and NR controlling LTE sidelink. Issue 7.1-1: - Can we confirm these RNTI names: SL-RNTI and SLCS-RNTI? RAN2#107bis: - New abbreviations and definitions are captured in 3. - Sidelink specific functions are captured in 4.4. - Sidelink specific channels are captured in 4.5.2, 4.5.3, 4.5.4.1, and 4.5.4.x. - Prioritization bewteen NR UL and NR/LTE SL is captured in 5.4.2.2 - LTE controlling NR sidelink communication is captured in 5.x.1.1 and 7.1 - Scheduling request for sidelink is captured in 5.4.4, 5.x.1.4 and 5.x.1.5. - Impact on MAC reset and erroneous data handling are captured in 5.12 and 5.13 respectively. - SL BWP is captured in 5.15.y following 5.15.x. - New section 5.x is added for introduction of SL-SCH data transfer. - New section 5.y is added for introduction of SL-BCH data transfer. - MAC PDU for SL-SCH, SL-BCH and Sidelink BSR are captured in 6.1.3.x, 6.1.4, 6.1.x, 6.2.1, and 6.2.x. - New RNTIs ar added in 7.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G V2X with NR Sidelink will not be introduced in Rel-16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, 4.4, 4.5.2, 4.5.3, 4.5.4.1, 4.5.4.x, 5.4.2.2, 5.4.3.1.3, 5.4.4, 5.8.x, 5.12, 5.13, 5.15.x, 5.15.y, 5.x, 5.y, 6.1.3.x, 6.1.4, 6.1.x, 6.2.1, 6.2.x, 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.331 CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF THE CHANGE

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: "NR; Overall description; Stage 2".

[3]	3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification".

[4]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) protocol specification".

[5]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[6]	3GPP TS 38.213: "NR; Physical Layer Procedures for control".

[7]	3GPP TS 38.214: "NR; Physical Layer Procedures for data".

[8]	3GPP TS 38.211: "NR; Physical channels and modulation".

[9]	3GPP TS 38.212: "NR; Multiplexing and channel coding".

[10]	Void.

[11]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[12]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[13]	3GPP TS 26.114: "Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[14]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[15]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[16]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[17]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Layer Procedures".

[xa]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services ".

[xb]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[xy]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[xz]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC); Protocol specification".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

HARQ information: HARQ information for DL-SCH or for UL-SCH transmissions consists of New Data Indicator (NDI), Transport Block size (TBS), Redundancy Version (RV), and HARQ process ID.

Msg3: Message transmitted on UL-SCH containing a C-RNTI MAC CE or CCCH SDU, submitted from upper layer and associated with the UE Contention Resolution Identity, as part of a Random Access procedure.

NR Sidelink Communication: AS functionality enabling at least V2X Communication as defined in TS 23.287 [xa], between two or more nearby UEs, using NR technology but not traversing any network node.

PDCCH occasion: A time duration (i.e. one or a consecutive number of symbols) during which the MAC entity is configured to monitor the PDCCH.

Serving Cell: A PCell, a PSCell, or an SCell in TS 38.331 [5].

Special Cell: For Dual Connectivity operation the term Special Cell refers to the PCell of the MCG or the PSCell of the SCG depending on if the MAC entity is associated to the MCG or the SCG, respectively. Otherwise the term Special Cell refers to the PCell. A Special Cell supports PUCCH transmission and contention-based Random Access, and is always activated.

Timing Advance Group: A group of Serving Cells that is configured by RRC and that, for the cells with a UL configured, using the same timing reference cell and the same Timing Advance value. A Timing Advance Group containing the SpCell of a MAC entity is referred to as Primary Timing Advance Group (PTAG), whereas the term Secondary Timing Advance Group (STAG) refers to other TAGs.

V2X sidelink communication: AS functionality enabling V2X Communication as defined in TS 23.285 [xb], between nearby UEs, using E-UTRA technology but not traversing any network node.

NOTE:	A timer is running once it is started, until it is stopped or until it expires; otherwise it is not running. A timer can be started if it is not running or restarted if it is running. A Timer is always started or restarted from its initial value. The duration of a timer is not updated until they are stopped or expires (e.g. due to BWP switching).

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BSR	Buffer Status Report

BWP	Bandwidth Part

CE	Control Element

CSI	Channel State Information

CSI-IM	CSI Intereference Measurement

CSI-RS	CSI Reference Signal

CS-RNTI	Configured Scheduling RNTI

INT-RNTI	Interruption RNTI

LCG	Logical Channel Group

LCP	Logical Channel Prioritization

MCG	Master Cell Group

NUL	Normal Uplink

NZP CSI-RS	Non-Zero Power CSI-RS

PHR	Power Headroom Report

PTAG	Primary Timing Advance Group

QCL	Quasi-colocation

RS	Reference Signal

SCG	Secondary Cell Group

SFI-RNTI	Slot Format Indication RNTI

SI	System Information

SpCell	Special Cell

SP	Semi-Persistent

SP-CSI-RNTI	Semi-Persistent CSI RNTI

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SS	Synchronization Signals

SSB	Synchronization Signal Block

STAG	Secondary Timing Advance Group

SUL	Supplementary Uplink

TAG	Timing Advance Group

TCI	Transmission Configuration Indicator

TPC-SRS-RNTI	Transmit Power Control-Sounding Reference Symbols-RNTI

V2X				Vehicle to everything

ZP CSI-RS	Zero Power CSI-RS

NEXT CHANGE

4.2.2	MAC Entities

The MAC entity of the UE handles the following transport channels:

-	Broadcast Channel (BCH);

-	Downlink Shared Channel(s) (DL-SCH);

-	Paging Channel (PCH);

-	Uplink Shared Channel(s) (UL-SCH);

-	Random Access Channel(s) (RACH).

When the UE is configured with SCG, two MAC entities are configured to the UE: one for the MCG and one for the SCG.

The functions of the different MAC entities in the UE operate independently unless otherwise specified. The timers and parameters used in each MAC entity are configured independently unless otherwise specified. The Serving Cells, C-RNTI, radio bearers, logical channels, upper and lower layer entities, LCGs, and HARQ entities considered by each MAC entity refer to those mapped to that MAC entity unless otherwise specified.

If the MAC entity is configured with one or more SCells, there are multiple DL-SCH and there may be multiple UL-SCH as well as multiple RACH per MAC entity; one DL-SCH, one UL-SCH, and one RACH on the SpCell, one DL-SCH, zero or one UL-SCH and zero or one RACH for each SCell.

If the MAC entity is not configured with any SCell, there is one DL-SCH, one UL-SCH, and one RACH per MAC entity.

Figure 4.2.2-1 illustrates one possible structure of the MAC entity when SCG is not configured.

Figure 4.2.2-1: MAC structure overview

Figure 4.2.2-2 illustrates one possible structure for the MAC entities when MCG and SCG are configured.

Figure 4.2.2-2: MAC structure overview with two MAC entities

In addition, the MAC entity of the UE handles the following transport channel for sidelink:

-	Sidelink Shared Channel (SL-SCH).

Figure 4.2.2-x illustrates one possible structure for the MAC entity when sidelink is configured.

Figure 4.2.1-x: MAC structure overview for sidelink

4.4	Functions

The MAC sublayer supports the following functions:

-	mapping between logical channels and transport channels;

-	multiplexing of MAC SDUs from one or different logical channels onto transport blocks (TB) to be delivered to the physical layer on transport channels;

-	demultiplexing of MAC SDUs to one or different logical channels from transport blocks (TB) delivered from the physical layer on transport channels;

-	scheduling information reporting;

-	error correction through HARQ;

-	logical channel prioritisation.

The relevance of MAC functions for uplink and downlink is indicated in Table 4.4-1.

Table 4.4-1: The link direction association of MAC functions.

| MAC function | Downlink | Uplink |
| --- | --- | --- |
| Mapping between logical channels and transport channels | X | X |
| Multiplexing |  | X |
| Demultiplexing | X |  |
| Scheduling information reporting |  | X |
| Error correction through HARQ | X | X |
| Logical Channel prioritisation |  | X |

In addition, the MAC sublayer supports the following functions for sidelink:

-	radio resource selection.

The relevance of MAC functions for sidelink transmission and reception is indicated in Table 4.4-x.

Table 4.4-x: The link direction association of MAC functions for sidelink.

| MAC function | Sidelink TX | Sidelink RX |
| --- | --- | --- |
| Mapping between logical channels and transport channels | X | X |
| Multiplexing | X |  |
| Demultiplexing |  | X |
| Scheduling information reporting |  |  |
| Error correction through HARQ | X | X |
| Logical Channel prioritisation | X |  |
| Radio resource selection | X |  |

NEXT CHANGE

4.5.2	Transport Channels

The MAC sublayer uses the transport channels listed in Table 4.5.2-1 below.

Table 4.5.2-1: Transport channels used by MAC

| Transport channel name | Acronym | Downlink | Uplink | Sidelink |
| --- | --- | --- | --- | --- |
| Broadcast Channel | BCH | X |  |  |
| Downlink Shared Channel | DL-SCH | X |  |  |
| Paging Channel | PCH | X |  |  |
| Uplink Shared Channel | UL-SCH |  | X |  |
| Random Access Channel | RACH |  | X |  |
| Sidelink Broadcast Channel | SL-BCH |  |  | X |
| Sidelink Shared Channel | SL-SCH |  |  | X |

4.5.3	Logical Channels

The MAC sublayer provides data transfer services on logical channels. To accommodate different kinds of data transfer services, multiple types of logical channels are defined i.e. each supporting transfer of a particular type of information.

Each logical channel type is defined by what type of information is transferred.

The MAC sublayer provides the control and traffic channels listed in Table 4.5.3-1 below.

Table 4.5.3-1: Logical channels provided by MAC.

| Logical channel name | Acronym | Control channel | Traffic channel |
| --- | --- | --- | --- |
| Broadcast Control Channel | BCCH | X |  |
| Paging Control Channel | PCCH | X |  |
| Common Control Channel | CCCH | X |  |
| Dedicated Control Channel | DCCH | X |  |
| Dedicated Traffic Channel | DTCH |  | X |
| Sidelink Broadcast Control Channel | SBCCH | X |  |
| Sidelink Control Channel | SCCH | X |  |
| Sidelink Traffic Channel | STCH |  | X |

4.5.4	Mapping of Transport Channels to Logical Channels

4.5.4.1	General

The MAC entity is responsible for mapping logical channels onto transport channels. This mapping depends on the multiplexing that is configured by RRC.

NEXT CHANGE

4.5.4.x	Sidelink mapping

The sidelink logical channels can be mapped as described in Table 4.5.4.x-1.

Table 4.5.4.x-1: Sidelink channel mapping.

| Transport channel Logical channel | SL-BCH | SL-SCH |
| --- | --- | --- |
| SBCCH | X |  |
| SCCH |  | X |
| STCH |  | X |

NEXT CHANGE

5.4.2.2	HARQ process

Each HARQ process is associated with a HARQ buffer.

New transmissions are performed on the resource and with the MCS indicated on either PDCCH, Random Access Response, or RRC. Retransmissions are performed on the resource and, if provided, with the MCS indicated on PDCCH, or on the same resource and with the same MCS as was used for last made transmission attempt within a bundle.

If the HARQ entity requests a new transmission for a TB, the HARQ process shall:

1>	store the MAC PDU in the associated HARQ buffer;

1>	store the uplink grant received from the HARQ entity;

1>	generate a transmission as described below.

If the HARQ entity requests a retransmission for a TB, the HARQ process shall:

1>	store the uplink grant received from the HARQ entity;

1>	generate a transmission as described below.

To generate a transmission for a TB, the HARQ process shall:

1>	if the MAC PDU was obtained from the Msg3 buffer; or

1>	if there is no measurement gap at the time of the transmission and, in case of retransmission, the retransmission does not collide with a transmission for a MAC PDU obtained from the Msg3 buffer:

2>	if there are neither sidelink transmission of the MAC entity nor transmission of V2X sidelink communication of the other MAC entity (i.e. E-UTRA MAC entity) at the time of the transmission; or

2>	if there is a configured grant for transmission of V2X sidelink communication on SL-SCH of the other MAC entity (i.e. E-UTRAN MAC entity) at the time of the transmission, and either none of the transmissions of V2X sidelink communication is prioritized as described in TS 36.321 clause 5.4.2.2 [xz] or both the MAC entity and the other MAC entity is able to perform this UL transmission and the transmissions of V2X sidelink communication which are prioritized as described in TS 36.321 clause 5.14.1.2.2 [xz] simultaneously; or

2>	if there is a sidelink grant for sidelink transmission at the time of the transmission, and the sidelink transmission is not prioritized or the MAC entity is able to perform this UL transmission and the sidelink transmission prioritized as described in clause 5.x.1.3.2 simultaneously:

NOTE:	Among the UL transmissions where the MAC entity is able to perform all transmissions of V2X sidelink communication prioritized simultaneously, if there are more than one UL transmission which the MAC entity is not able to perform simultaneously, it is up to UE implementation whether this UL transmission is performed.

3>	instruct the physical layer to generate a transmission according to the stored uplink grant.

NEXT CHANGE

5.4.3.1.3	Allocation of resources

The MAC entity shall, when a new transmission is performed:

1>	allocate resources to the logical channels as follows:

2>	logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

2>	if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

NOTE:	The value of Bj can be negative.

If the MAC entity is requested to simultaneously transmit multiple MAC PDUs, or if the MAC entity receives the multiple UL grants within one or more coinciding PDCCH occasions (i.e. on different Serving Cells), it is up to UE implementation in which order the grants are processed.

The UE shall also follow the rules below during the scheduling procedures above:

-	the UE should not segment an RLC SDU (or partially transmitted SDU or retransmitted RLC PDU) if the whole SDU (or partially transmitted SDU or retransmitted RLC PDU) fits into the remaining resources of the associated MAC entity;

-	if the UE segments an RLC SDU from the logical channel, it shall maximize the size of the segment to fill the grant of the associated MAC entity as much as possible;

-	the UE should maximise the transmission of data;

-	if the MAC entity is given a UL grant size that is equal to or larger than 8 bytes while having data available and allowed (according to clause 5.4.3.1) for transmission, the MAC entity shall not transmit only padding BSR and/or padding.

The MAC entity shall not generate a MAC PDU for the HARQ entity if the following conditions are satisfied:

-	the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant; and

-	there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

-	the MAC PDU includes zero MAC SDUs; and

-	the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

Editor’s Note: RAN2 working assumption is to pursue the need of flexible priority for the non-padding SL BSR in NR

-	C-RNTI MAC CE or data from UL-CCCH;

-	Configured Grant Confirmation MAC CE;

-	MAC CE for BSR, with exception of BSR included for padding;

-	Single Entry PHR MAC CE or Multiple Entry PHR MAC CE;

-	data from any Logical Channel, except data from UL-CCCH;

-	MAC CE for Recommended bit rate query;

-	MAC CE for BSR included for padding;

-	MAC CE for SBSR included for padding.

NEXT CHANGE

5.4.4	Scheduling Request

The Scheduling Request (SR) is used for requesting UL-SCH resources for new transmission.

The MAC entity may be configured with zero, one, or more SR configurations. An SR configuration consists of a set of PUCCH resources for SR across different BWPs and cells. For a logical channel, at most one PUCCH resource for SR is configured per BWP.

Each SR configuration corresponds to one or more logical channels. Each logical channel may be mapped to zero or one SR configuration, which is configured by RRC. The SR configuration of the logical channel that triggered the BSR (clause 5.4.5) (if such a configuration exists) is considered as corresponding SR configuration for the triggered SR.

RRC configures the following parameters for the scheduling request procedure:

-	sr-ProhibitTimer (per SR configuration);

-	sr-TransMax (per SR configuration).

The following UE variables are used for the scheduling request procedure:

-	SR_COUNTER (per SR configuration).

If an SR is triggered and there are no other SRs pending corresponding to the same SR configuration, the MAC entity shall set the SR_COUNTER of the corresponding SR configuration to 0.

When an SR is triggered, it shall be considered as pending until it is cancelled.

All pending SR(s) triggered for requesting UL-SCH resources prior to the MAC PDU assembly shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the MAC PDU is transmitted and this PDU includes a Long or Short BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR (see clause 5.4.5) prior to the MAC PDU assembly. All pending SR(s) triggered for requesting UL-SCH resources shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the UL grant(s) can accommodate all pending data available for transmission.

Only PUCCH resources on a BWP which is active at the time of SR transmission occasion are considered valid.

As long as at least one SR is pending, the MAC entity shall for each pending SR:

1>	if the MAC entity has no valid PUCCH resource configured for the pending SR:

2>	initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel the pending SR.

1>	else, for the SR configuration corresponding to the pending SR:

2>	when the MAC entity has an SR transmission occasion on the valid PUCCH resource for SR configured; and

2>	if sr-ProhibitTimer is not running at the time of the SR transmission occasion; and

2>	if the PUCCH resource for the SR transmission occasion does not overlap with a measurement gap; and

2>	if the PUCCH resource for the SR transmission occasion does not overlap with a UL-SCH resource:

3>	if SR_COUNTER < sr-TransMax:

4>	increment SR_COUNTER by 1;

4>	instruct the physical layer to signal the SR on one valid PUCCH resource for SR;

4>	start the sr-ProhibitTimer.

3>	else:

4>	notify RRC to release PUCCH for all Serving Cells;

4>	notify RRC to release SRS for all Serving Cells;

4>	clear any configured downlink assignments and uplink grants;

4>	clear any PUSCH resources for semi-persistent CSI reporting;

4>	initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel all pending SRs.

NOTE 1:	The selection of which valid PUCCH resource for SR to signal SR on when the MAC entity has more than one overlapping valid PUCCH resource for the SR transmission occasion is left to UE implementation.

NOTE 2:	If more than one individual SR triggers an instruction from the MAC entity to the PHY layer to signal the SR on the same valid PUCCH resource, the SR_COUNTER for the relevant SR configuration is incremented only once.

The MAC entity may stop, if any, ongoing Random Access procedure due to a pending SR which has no valid PUCCH resources configured, which was initiated by MAC entity prior to the MAC PDU assembly. Such a Random Access procedure may be stopped when the MAC PDU is transmitted using a UL grant other than a UL grant provided by Random Access Response, and this PDU includes a BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR (see clause 5.4.5) prior to the MAC PDU assembly, or when the UL grant(s) can accommodate all pending data available for transmission.

NEXT CHANGE

5.8.x	Sidelink

There are two types of transmission without dynamic grant:

-	configured grant Type 1 where an sidelink grant is provided by RRC, and stored as configured sidelink grant;

-	configured grant Type 2 where an sidelink grant is provided by PDCCH, and stored or cleared as configured sidelink grant based on L1 signalling indicating configured sidelink grant activation or deactivation.

Type 1 and Type 2 are configured with a single BWP. Multiple configurations can be active simultaneously on the BWP. For Type 2, activation and deactivation are independent.

RRC configures the following parameters when the configured grant Type 1 is configured:

-	[slcs-RNTI: SLCS-RNTI] for retransmission;

-	periodicity: periodicity of the configured grant Type 1;

-	timeDomainOffset: Offset of a resource with respect to [SFN = 0] in time domain;

-	[FFS for additional parameters]

RRC configures the following parameters when the configured grant Type 2 is configured:

-	[slcs-RNTI: SLCS-RNTI] for activation, deactivation, and retransmission;

-	periodicity: periodicity of the configured grant Type 2;

-	[FFS for additional parameters]

Upon configuration of a configured grant Type 1, the MAC entity shall for each configured sidelink grant:

1>	initialise or re-initialise the configured sidelink grant to determine the set of PSCCH durations and the set of PSSCH durations for transmissions of multiple MAC PDUs according to [xx].

When a configured sidelink grant is released by upper layers, all the corresponding configurations shall be released and all corresponding sidelink grants shall be cleared.

The MAC entity shall:

1>	if the configured sidelink grant confirmation has been triggered and not cancelled; and

1>	if the MAC entity has SL resources allocated for new transmission:

2>	instruct the Multiplexing and Assembly procedure to generate a Configured Grant Confirmation as defined in clause [x];

2>	cancel the triggered configured sidelink grant confirmation.

For a configured grant Type 2, the MAC entity shall clear the configured uplink grant immediately after first transmission of Configured Grant Confirmation triggered by the configured sidelink grant deactivation.

NEXT CHANGE

5.12	MAC Reset

If a reset of the MAC entity is requested by upper layers, the MAC entity shall:

1>	initialize Bj for each logical channel to zero, except for sidelink logical channels;

1>	stop (if running) all timers;

1>	consider all timeAlignmentTimers as expired and perform the corresponding actions in clause 5.2;

1>	set the NDIs for all uplink HARQ processes to the value 0;

1>	stop, if any, ongoing RACH procedure;

1>	discard explicitly signalled contention-free Random Access Resources, if any;

1>	flush Msg3 buffer;

1>	cancel, if any, triggered Scheduling Request procedure;

1>	cancel, if any, triggered Buffer Status Reporting procedure;

1>	cancel, if any, triggered Power Headroom Reporting procedure;

1>	cancel, if any, triggered Sidelink Buffer Status Reporting procedure;

1>	flush the soft buffers for all DL HARQ processes;

1>	for each DL HARQ process, consider the next received transmission for a TB as the very first transmission;

1>	release, if any, Temporary C-RNTI;

1>	reset BFI_COUNTER.

5.13	Handling of unknown, unforeseen and erroneous protocol data

When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment, containing a Reserved LCID value, or an LCID value the MAC Entity does not support, the MAC entity shall at least:

1>	discard the received subPDU and any remaining subPDUs in the MAC PDU.

When a MAC entity receives a MAC PDU for the MAC entity's C-RNTI or CS-RNTI, or by the configured downlink assignment, containing an LCID value which is not configured, the MAC entity shall at least:

1>	discard the received subPDU.

When a MAC entity receives a MAC PDU on SL-SCH containing reserved values, the MAC entity shall:

-	ignore the MAC PDU subheaders containing reserved values and the corresponding MAC SDUs.

Editor’s Note: FFS on MAC PDU on SL-SCH containing reserved values.

NEXT CHANGE

5.15	Bandwidth Part (BWP) operation

5.15.x	Downlink and Uplink

In addition to clause 12 of TS 38.213 [6], this clause specifies requirements on BWP operation for downlink and uplink.

A Serving Cell may be configured with one or multiple BWPs, and the maximum number of BWP per Serving Cell is specified in TS 38.213 [6].

The BWP switching for a Serving Cell is used to activate an inactive BWP and deactivate an active BWP at a time. The BWP switching is controlled by the PDCCH indicating a downlink assignment or an uplink grant, by the bwp-InactivityTimer, by RRC signalling, or by the MAC entity itself upon initiation of Random Access procedure. Upon RRC (re-)configuration of firstActiveDownlinkBWP-Id and/or firstActiveUplinkBWP-Id for SpCell or activation of an SCell, the DL BWP and/or UL BWP indicated by firstActiveDownlinkBWP-Id and/or firstActiveUplinkBWP-Id respectively (as specified in TS 38.331 [5]) is active without receiving PDCCH indicating a downlink assignment or an uplink grant. The active BWP for a Serving Cell is indicated by either RRC or PDCCH (as specified in TS 38.213 [6]). For unpaired spectrum, a DL BWP is paired with a UL BWP, and BWP switching is common for both UL and DL.

For each activated Serving Cell configured with a BWP, the MAC entity shall:

1>	if a BWP is activated:

2>	transmit on UL-SCH on the BWP;

2>	transmit on RACH on the BWP, if PRACH occasions are configured;

2>	monitor the PDCCH on the BWP;

2>	transmit PUCCH on the BWP, if configured;

2>	report CSI for the BWP;

2>	transmit SRS on the BWP, if configured;

2>	receive DL-SCH on the BWP;

2>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 on the active BWP according to the stored configuration, if any, and to start in the symbol according to rules in clause 5.8.2.

1>	if a BWP is deactivated:

2>	not transmit on UL-SCH on the BWP;

2>	not transmit on RACH on the BWP;

2>	not monitor the PDCCH on the BWP;

2>	not transmit PUCCH on the BWP;

2>	not report CSI for the BWP;

2>	not transmit SRS on the BWP;

2>	not receive DL-SCH on the BWP;

2>	clear any configured downlink assignment and configured uplink grant of configured grant Type 2 on the BWP;

2>	suspend any configured uplink grant of configured grant Type 1 on the inactive BWP.

Upon initiation of the Random Access procedure on a Serving Cell, after the selection of carrier for performing Random Access procedure as specified in clause 5.1.1, the MAC entity shall for the selected carrier of this Serving Cell:

1>	if PRACH occasions are not configured for the active UL BWP:

2>	switch the active UL BWP to BWP indicated by initialUplinkBWP;

2>	if the Serving Cell is an SpCell:

3>	switch the active DL BWP to BWP indicated by initialDownlinkBWP.

1>	else:

2>	if the Serving Cell is an SpCell:

3>	if the active DL BWP does not have the same bwp-Id as the active UL BWP:

4>	switch the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP.

1>	stop the bwp-InactivityTimer associated with the active DL BWP of this Serving Cell, if running.

1>	if the Serving Cell is SCell:

2>	stop the bwp-InactivityTimer associated with the active DL BWP of SpCell, if running.

1>	perform the Random Access procedure on the active DL BWP of SpCell and active UL BWP of this Serving Cell.

If the MAC entity receives a PDCCH for BWP switching of a Serving Cell, the MAC entity shall:

1>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

1>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in clauses 5.1.4 and 5.1.5):

2>	perform BWP switching to a BWP indicated by the PDCCH.

If the MAC entity receives a PDCCH for BWP switching for a Serving Cell while a Random Access procedure associated with that Serving Cell is ongoing in the MAC entity, it is up to UE implementation whether to switch BWP or ignore the PDCCH for BWP switching, except for the PDCCH reception for BWP switching addressed to the C-RNTI for successful Random Access procedure completion (as specified in clauses 5.1.4 and 5.1.5) in which case the UE shall perform BWP switching to a BWP indicated by the PDCCH. Upon reception of the PDCCH for BWP switching other than successful contention resolution, if the MAC entity decides to perform BWP switching, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure after performing the BWP switching; if the MAC decides to ignore the PDCCH for BWP switching, the MAC entity shall continue with the ongoing Random Access procedure on the Serving Cell.

Upon reception of RRC (re-)configuration for BWP switching for a Serving Cell while a Random Access procedure associated with that Serving Cell is ongoing in the MAC entity, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure after performing the BWP switching.

The MAC entity shall for each activated Serving Cell configured with bwp-InactivityTimer:

1>	if the defaultDownlinkBWP-Id is configured, and the active DL BWP is not the BWP indicated by the defaultDownlinkBWP-Id; or

1>	if the defaultDownlinkBWP-Id is not configured, and the active DL BWP is not the initialDownlinkBWP:

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received on the active BWP; or

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received for the active BWP; or

2>	if a MAC PDU is transmitted in a configured uplink grant or received in a configured downlink assignment:

3>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

3>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in clauses 5.1.4 and 5.1.5):

4>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

2>	if the bwp-InactivityTimer associated with the active DL BWP expires:

3>	if the defaultDownlinkBWP-Id is configured:

4>	perform BWP switching to a BWP indicated by the defaultDownlinkBWP-Id.

3>	else:

4>	perform BWP switching to the initialDownlinkBWP.

NOTE:	If a Random Access procedure is initiated on an SCell, both this SCell and the SpCell are associated with this Random Access procedure.

1>	if a PDCCH for BWP switching is received, and the MAC entity switches the active DL BWP:

2>	if the defaultDownlinkBWP-Id is configured, and the MAC entity switches to the DL BWP which is not indicated by the defaultDownlinkBWP-Id; or

2>	if the defaultDownlinkBWP-Id is not configured, and the MAC entity switches to the DL BWP which is not the initialDownlinkBWP:

3>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

5.15.y	Sidelink

In addition to clause xx of TS 38.213 [6], this clause specifies requirements on BWP operation for sidelink.

The MAC entity is configured with at most a single SL BWP where sidelink transmission and reception are performed.

For a BWP, the MAC entity shall:

1>	if the BWP is activated for sidelink transmission:

2>	transmit PSBCH on the BWP, if configured;

2>	transmit PSCCH on the BWP;

2>	transmit SL-SCH on the BWP;

2>	transmit CSI-RS on the BWP;

2>	receive CSI reporting on the BWP, if configured and enabled;

2>	receive PSFCH on the BWP, if configured.

1>	if the BWP is activated for sidelink reception:

2>	receive PSBCH on the BWP, if configured;

2>	receive PSCCH on the BWP;

2>	receive SL-SCH on the BWP;

2>	receive CSI-RS on the BWP;

2>	report CSI on the BWP, if configured and enabled;

2>	transmit PSFCH on the BWP, if configured.

Editor’s Note: FFS on need of BWP deactivation.

NEXT CHANGE

5.x	SL-SCH Data transfer

5.x.1	SL-SCH Data transmission

5.x.1.1	SL Grant reception and SCI transmission

Sidelink grant is received dynamically on the PDCCH, configured semi-persistently by RRC or autonomously selected by the MAC entity. The MAC entity shall have a sidelink grant on an active SL BWP to determine a set of PSSCH duration(s) in which transmission of SCI occurs and a set of PSSCH duration(s) in which transmission of SL-SCH associated with the SCI occurs. To perform the requested transmissions, the MAC layer receives HARQ information from lower layers.

If the MAC entity has a [SL-RNTI] or [SLCS-RNTI], or the other MAC entity (i.e. E-UTRA MAC entity) has a [SLCS-RNTI], the MAC entity shall for each PDCCH occasion and for each grant received for this PDCCH occasion:

1>	if a sidelink grant has been received on the PDCCH for the MAC entity's [SL-RNTI]:

2>	store the sidelink grant [and the associated HARQ information] as configured sidelink grant;

Editor’s Note: FFS how the HARQ information is indicated in PDCCH and determined.

2>	use the received sidelink grant to determine the set of PSCCH durations and the set of PSSCH durations for one or more transmissions of a single MAC PDU according to [38.2xx];

1>	else if a sidelink grant has been received on the PDCCH for the MAC entity's [SLCS-RNTI] or the other MAC entity’s [SLCS-RNTI]:

2> if PDCCH contents indicate configured grant Type 2 deactivation for a configured sidelink grant:

3> clear the configured sidelink grant, if available;

3>	trigger configured sidelink grant confirmation for the configured sidelink grant;

2> else if PDCCH contents indicate configured grant Type 2 activation for a configured sidelink grant:

3>	trigger configured sidelink grant confirmation for the configured sidelink grant;

3>	initialise or re-initialise the configured sidelink grant to determine the set of PSCCH durations and the set of PSSCH durations for transmissions of multiple MAC PDUs according to [xx].

2> else if PDCCH contents indicate [retransmission]:

Editor’s Note: FFS how HARQ retransmissions are supported for sidelink CG.

3>	use the received sidelink grant to determine the set of PSCCH durations and the set of PSSCH durations for retransmission of a single MAC PDU according to [38.2xx].

If the MAC entity is configured by RRC to transmit using pool(s) of resources in a carrier as indicated in TS 38.331 [5] or TS 36.331 [xy] based on sensing, or partial sensing, or random selection, the MAC entity shall for each Sidelink process:

1>	if upper layers indicates that transmissions of multiple MAC PDUs are allowed according to TS 36.331 [5], and the MAC entity selects to create a configured sidelink grant corresponding to transmissions of multiple MAC PDUs, and SL data is available in a logical channel:

2> perform [the TX resource (re-)selection check as specified in clause 5.x.1.2;]

2>	if the TX resource (re-)selection is triggered as the result of the TX resource (re-)selection check;

3>	select a set of resources from the resources indicated by the physical layer according to TS 38.xxx;

Editor’s Note: FFS how to select a set of resources for HARQ (re)transmissions.

3>	use the selected sidelink grant to determine the set of PSCCH durations and the set of PSSCH durations according to [xx];

3>	consider the selected sidelink grant to be a configured sidelink grant.

1>	if the MAC entity selects to create a configured sidelink grant corresponding to transmission(s) of a single MAC PDU, and SL data is available in a logical channel:

2> perform the TX resource (re-)selection check as specified in clause 5.x.1.2;

2>	if the TX resource (re-)selection is triggered as the result of the TX resource (re-)selection check;

3>	select a set of resources from the resources indicated by the physical layer according to TS 38.xxx;

3>	use the selected sidelink grant to determine the set of PSCCH durations and the set of PSSCH durations according to [xx];

3>	consider the selected sidelink grant to be a configured sidelink grant.

The MAC entity shall for each PSSCH duration:

1>	for each configured sidelink grant occurring in this PSSCH duration:

2>	set the HARQ Process ID to the HARQ Process ID associated with this PSSCH duration;

Editor’s Note: FFS how to determine new TX, ReTX and the HARQ Process IDs for Sidelink Mode 1 and 2.

2>	deliver the sidelink grant and the associated HARQ information to the Sidelink HARQ Entity for this PSSCH duration.

5.x.1.2	TX resource (re-)selection check

If the TX resource (re-)selection check procedure is triggered for a Sidelink process according to clause 5.x.1.1, the MAC entity shall for the Sidelink process:

Editor’s Note: FFS whether the below LTE conditions triggering TX resource reselection can be used in NR Sidelink.

[1>	if SL_RESOURCE_RESELECTION_COUNTER = 0 and when SL_RESOURCE_RESELECTION_COUNTER was equal to 1 the MAC entity randomly selected, with equal probability, a value in the interval [0, 1] which is above the probability configured by upper layers in probResourceKeep; or

1>	if neither transmission nor retransmission has been performed by the MAC entity on any resource indicated in the configured sidelink grant during [the last second]; or

1>	if sl-ReselectAfter is configured and the number of consecutive unused transmission opportunities on resources indicated in the configured sidelink grant is equal to sl-ReselectAfter; or

1>	if there is no configured sidelink grant; or

1>	if the configured sidelink grant cannot accommodate a RLC SDU [by using the maximum allowed MCS configured by upper layers in maxMCS-PSSCH] and the MAC entity selects not to segment the RLC SDU; or

NOTE:	If the configured sidelink grant cannot accommodate the RLC SDU, it is left for UE implementation whether to perform segmentation or sidelink resource reselection.

1>	if transmission(s) with the configured sidelink grant cannot fulfil the latency requirement of the data in a logical channel according to the associated priority, and the MAC entity selects not to perform transmission(s) corresponding to a single MAC PDU; or

NOTE:	If the latency requirement is not met, it is left for UE implementation whether to perform transmission(s) corresponding to single MAC PDU or sidelink resource reselection.

1>	if a pool of resources is configured or reconfigured by upper layers:]

2>	clear the configured sidelink grant associated to the Sidelink process, if available;

2>	trigger the TX resource (re-)selection.

5.x.1.3	Sidelink HARQ operation

5.x.1.3.1	Sidelink HARQ Entity

The MAC entity includes a Sidelink HARQ entity for transmission on SL-SCH, which maintains a number of parallel Sidelink processes.

Editor’s Note: FFS how many transmitting Sidelink HARQ entities can be configured at the MAC entity for SL-SCH. e.g. One HARQ entity for broadcast, one HARQ entity for each group and one HARQ entity for each unicast link.

The maximum number of transmitting Sidelink processes associated with each Sidelink HARQ Entity is [x]. A sidelink process may be configured for transmissions of multiple MAC PDUs. For transmissions of multiple MAC PDUs, the maximum number of transmitting Sidelink processes associated with each Sidelink HARQ Entity is [y].

Editor’s Note: FFS whether the maximum number of transmitting Sidelink processes is defined per destination e.g. for unicast and groupcast.

A delivered sidelink grant and its associated HARQ information are associated with a Sidelink process. Each Sidelink process supports one TB. Each Sidelink process is associated with a HARQ process identifier.

For each sidelink grant, the Sidelink HARQ Entity shall:

1>	identify the Sidelink process associated with this grant, and for each identified Sidelink process:

2>	if the sidelink grant is used for initial transmission according to [x], and if no MAC PDU has been obtained:

Editor’s Note: FFS how initial grant and reTX grant are determined for SL mode 1 and 2.

3>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity, if any;

3>	if a MAC PDU to transmit has been obtained:

4>	deliver the MAC PDU and the sideink grant and the HARQ information and the QoS information of the TB to the identified HARQ process;

4>	instruct the identified HARQ process to trigger a new transmission;

3>	else:

4>	flush the HARQ buffer of the identified HARQ process.

2>	else (i.e. retransmission):

3>	if the sidelink grant received on PDCCH was addressed to [SLCS-RNTI] and if the HARQ buffer of the identified process is empty; or

3>	if no negative acknowledgement is obtained for the most recent (re-)transmission of the TB according to clause 5.x.1.3.3:

4>	ignore the sidelink grant.

3>	else if HARQ feedback is configured for the Sidelink process and a negative acknowledgement is obtained for the most recent (re-)transmission of the TB according to clause 5.x.1.3.3:

4>	deliver the sidelink grant and the HARQ information (redundancy version) of the TB to the identified HARQ process;

4>	instruct the identified Sidelink process to trigger a retransmission.

5.x.1.3.2	Sidelink process

The Sidelink process is associated with a HARQ buffer.

New transmissions and retransmissions are performed on the resource indicated in the sidelink grant as specified in clause 5.x.1.1 and with the MCS selected as specified in [clause 5.x.1.1].

[If the sidelink process is configured to perform transmissions of multiple MAC PDUs the process maintains a counter SL_RESOURCE_RESELECTION_COUNTER. For other configurations of the sidelink process, this counter is not available.]

Editor’s Note: FFS on support of SL_RESOURCE_RESELECTION_COUNTER.

If the Sidelink HARQ Entity requests a new transmission, the Sidelink process shall:

1> consider the NDI to have been toggled for the Sidelink process;

1>	store the MAC PDU in the associated HARQ buffer;

1>	store the sidelink grant received from the Sidelink HARQ Entity;

1>	generate a transmission as described below;

If the Sidelink HARQ Entity requests a retransmission, the Sidelink process shall:

1> consider the NDI to have been not toggled for the Sidelink process;

1>	generate a transmission as described below;

To generate a transmission, the Sidelink process shall:

1>	if there is no uplink transmission; or if the MAC entity is able to simultaneously perform uplink transmission(s) and sidelink transmission at the time of the transmission; or if the other MAC entity (i.e. E-UTRA MAC entity) and the MAC entity are able to simultaneously perform uplink transmission(s) and sidelink transmission at the time of the transmission respectively:

2>	instruct the physical layer to transmit SCI according to the stored sidelink grant with the associated HARQ information including the values of the NDI, the HARQ Process ID and the value of the highest priority of the logical channel(s) in the MAC PDU;

2>	instruct the physical layer to generate a transmission according to the stored sidelink grant;

2> monitor PSFCH for the transmission as specified in clause 5.x.1.3.3, if configured.

1>	if this transmission corresponds to the last transmission of the MAC PDU:

2>	[FFS: decrement SL_RESOURCE_RESELECTION_COUNTER by 1, if available.]

The transmission of the MAC PDU is prioritized over uplink transmissions if the following conditions are met:

1>	if the MAC entity is not able to simultaneously perform all uplink transmissions and this sidelink transmission at the time of the transmission; and

1>	[Additional condition(s) are FFS].

5.x.1.3.3	PSFCH reception

If HARQ feedback is configured for a Sidelink process, the MAC entity shall:

1>	for a PSFCH duration during which the MAC entity monitors PSFCH for the Sidelink process:

2> if an acknowledgement corresponding to the transmission in clause 5.x.1.3.2 is obtained from the physical layer:

3> deliver the acknowledgement to the corresponding Sidelink HARQ entity;

2> else:

3> deliver a negative acknowledgement to the corresponding Sidelink HARQ entity;

2> if the MAC entity has been configured [for conveying SL HARQ feedback to the gNB]:

3> instruct the physical layer to signal [the acknowledgement] on the PUCCH according to TS 38.2xx [x].

Editor’s Note: FFS on details of retransmission resource request for mode 1.

5.x.1.3	Multiplexing and assembly

For PDU(s) associated with one SCI, MAC shall consider only logical channels with the same Source Layer-2 ID-Destination Layer-2 ID pair. Multiple transmissions for different Sidelink processes are allowed to be independently performed in different PSSCH durations.

5.x.1.3.1	Logical channel prioritization

5.x.1.3.1.1	General

The sidelink Logical Channel Prioritization procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of sidelink data by signalling for each logical channel:

-	[sl-priority] where an increasing priority value indicates a lower priority level;

-	[sl-prioritisedBitRate] which sets the sidelink Prioritized Bit Rate (sPBR);

-	[sl-bucketSizeDuration] which sets the sidelink Bucket Size Duration (sBSD).

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel:

-	[configuredSLGrantType1Allowed] which sets whether a configured grant Type 1 can be used for sidelink transmission.

The following UE variable is used for the Logical channel prioritization procedure:

-	[SBj] which is maintained for each logical channel j.

The MAC entity shall initialize [SBj] of the logical channel to zero when the logical channel is established.

For each logical channel j, the MAC entity shall:

1>	increment [SBj] by the product sPBR × T before every instance of the LCP procedure, where T is the time elapsed since [SBj] was last incremented;

1>	if the value of [SBj] is greater than the sidelink bucket size (i.e. sPBR × sBSD):

2>	set [SBj] to the sidelink bucket size.

NOTE:	The exact moment(s) when the UE updates [SBj] between LCP procedures is up to UE implementation, as long as [SBj] is up to date at the time when a grant is processed by LCP.

5.x.1.3.1.2	Selection of logical channels

The MAC entity shall for each SCI corresponding to a new transmission:

1>	select a Destination, having the logical channel with the highest priority, among the logical channels having data available for transmission;

2>	consider logical channels allowed on the carrier where the SCI is transmitted for the selected Destination, if the carrier is configured by upper layers according to TS 38.331 [5] or TS 36.331 [xy];

1>	select the logical channels for each SL grant that satisfy all the following condition:

2>	[configuredSLGrantType1Allowed], if configured, is set to true in case the SL grant is a Configured Grant Type 1.

5.x.1.3.1.3	Allocation of sidelink resources

The MAC entity shall for each SCI corresponding to a new transmission:

1>	allocate resources to the logical channels as follows:

2>	logical channels selected in clause 5.x.1.3.1.2 for the SL grant with [SBj] > 0 are allocated resources in a decreasing priority order. If the SL-PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the sPBR of the lower priority logical channel(s);

2>	decrement [SBj] by the total size of MAC SDUs served to logical channel j above;

2>	if any resources remain, all the logical channels selected in clause 5.x.1.3.1.2 are served in a strict decreasing priority order (regardless of the value of [SBj]) until either the data for that logical channel or the SL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

NOTE:	The value of [SBj] can be negative.

The UE shall also follow the rules below during the SL scheduling procedures above:

-	the UE should not segment an RLC SDU (or partially transmitted SDU or retransmitted RLC PDU) if the whole SDU (or partially transmitted SDU or retransmitted RLC PDU) fits into the remaining resources of the associated MAC entity;

-	if the UE segments an RLC SDU from the logical channel, it shall maximize the size of the segment to fill the grant of the associated MAC entity as much as possible;

-	the UE should maximise the transmission of data;

-	if the MAC entity is given a sidelink grant size that is equal to or larger than [x] bytes while having data available and allowed (according to clause 5.x.1.3.1) for transmission, the MAC entity shall not transmit only padding.

5.x.1.3.2	Multiplexing of MAC SDUs

The MAC entity shall multiplex MAC SDUs in a MAC PDU according to clauses 5.x.1.3.1 and 6.x.

5.x.1.4	Scheduling Request

In addition to clause 5.4.4, the Scheduling Request (SR) is also used for requesting SL-SCH resources for new transmission. If configured, the MAC entity performs the SR procedure as specified in this clause unless otherwise specified in clause 5.4.4.

The SR configuration of the logical channel that triggered the Sidelink BSR (clause 5.x.1.4) (if such a configuration exists) is also considered as corresponding SR configuration for the triggered SR (clause 5.4.4).

All pending SR(s) triggered only for requesting SL-SCH resources prior to the MAC PDU assembly shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the MAC PDU is transmitted and this PDU includes a Sidelink BSR MAC CE which contains buffer status up to (and including) the last event that triggered a Sidelink BSR (see clause 5.x.1.4) prior to the MAC PDU assembly. All pending SR(s) triggered for requesting SL-SCH resources shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the SL grant(s) can accommodate all pending data available for transmission in sidelink.

5.x.1.5	Buffer Status Reporting

The Sidelink Buffer Status reporting (SBSR) procedure is used to provide the serving gNB with information about SL data volume in the MAC entity.

RRC configures the following parameters to control the SBSR:

-	periodicBSR-TimerSL;

-	retxBSR-TimerSL;

-	logicalChannelSR-DelayTimerAppliedSL;

-	logicalChannelSR-DelayTimerSL;

-	logicalChannelSR-MaskSL;

-	logicalChannelGroupSL.

Each logical channel which belongs to a Destination is allocated to an LCG as specified in TS 38.331 [5] or TS 36.331 [xy]. The maximum number of LCGs is eight.

The MAC entity determines the amount of SL data available for a logical channel according to the data volume calculation procedure in TSs 38.322 [3] and 38.323 [4].

A SBSR shall be triggered if any of the following events occur:

-	if the MAC entity has a [SL-RNTI] or [SLCS-RNTI]:

-	SL data, for a logical channel of a Destination, becomes available to the MAC entity; and either

-	this SL data belongs to a logical channel with higher priority than the priorities of the logical channels containing available SL data which belong to any LCG belonging to the same Destination; or

-	none of the logical channels which belong to an LCG belonging to the same Destination contains any available SL data.

in which case the SBSR is referred below to as 'Regular SBSR';

-	UL resources are allocated and number of padding bits remaining after a Padding BSR has been triggered is equal to or larger than the size of the SBSR MAC CE plus its subheader, in which case the SBSR is referred below to as 'Padding SBSR';

-	retxBSR-TimerSL expires, and at least one of the logical channels which belong to an LCG contains SL data, in which case the SBSR is referred below to as 'Regular SBSR';

-	periodicBSR-TimerSL expires, in which case the SBSR is referred below to as 'Periodic SBSR'.

For Regular SBSR, the MAC entity shall:

1>	if the SBSR is triggered for a logical channel for which logicalChannelSR-DelayTimerAppliedSL with value true is configured by upper layers:

2>	start or restart the logicalChannelSR-DelayTimerSL.

1>	else:

2>	if running, stop the logicalChannelSR-DelayTimerSL.

For Regular and Periodic SBSR, the MAC entity shall:

1>	if the number of bits in the UL grant is equal to or larger than the size of a SBSR containing buffer status for all LCGs having data available for transmission plus its subheader:

2>	report SBSR containing buffer status for all LCGs having data available for transmission;

1>	else:

2> report Truncated SBSR containing buffer status for as many LCGs having data available for transmission as possible, taking the number of bits in the UL grant into consideration.

For Padding BSR:

1>	if the number of padding bits remaining after a Padding BSR has been triggered is equal to or larger than the size of a SBSR containing buffer status for all LCGs having data available for transmission plus its subheader:

2>	report SBSR containing buffer status for all LCGs having data available for transmission;

1>	else:

2>	report Truncated SBSR containing buffer status for as many LCGs having data available for transmission as possible, taking the number of bits in the UL grant into consideration.

For SBSR triggered by retxBSR-TimerSL expiry, the MAC entity considers that the logical channel that triggered the SBSR is the highest priority logical channel that has data available for transmission at the time the SBSR is triggered.

The MAC entity shall:

1>	if the sidelink Buffer Status reporting procedure determines that at least one SBSR has been triggered and not cancelled:

2>	if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the SBSR MAC CE plus its subheader as a result of logical channel prioritization according to clause 5.4.3.1:

3>	instruct the Multiplexing and Assembly procedure in clause 5.4.3 to generate the SBSR MAC CE(s);

3>	start or restart periodicBSR-TimerSL except when all the generated SBSRs are Truncated SBSRs;

3>	start or restart retxBSR-TimerSL.

2>	if a Regular SBSR has been triggered and logicalChannelSR-DelayTimerSL is not running:

3>	if there is no UL-SCH resource available for a new transmission; or

3>	if the MAC entity is configured with configured uplink grant(s) and the Regular SBSR was triggered for a logical channel [for which logicalChannelSR-MaskSL is set to false:]

4>	trigger a Scheduling Request.

NOTE:	UL-SCH resources are considered available if the MAC entity has an active configuration for either type of configured uplink grants, or if the MAC entity has received a dynamic uplink grant, or if both of these conditions are met. If the MAC entity has determined at a given point in time that UL-SCH resources are available, this need not imply that UL-SCH resources are available for use at that point in time.

A MAC PDU shall contain at most one SBSR MAC CE, even when multiple events have triggered a SBSR. The Regular SBSR and the Periodic SBSR shall have precedence over the padding SBSR.

The MAC entity shall restart retxBSR-TimerSL upon reception of a SL grant.

All triggered SBSRs may be cancelled when the SL grant(s) can accommodate all pending data available for transmission. All BSRs triggered prior to MAC PDU assembly shall be cancelled when a MAC PDU is transmitted and this PDU includes a SBSR MAC CE which contains buffer status up to (and including) the last event that triggered a SBSR prior to the MAC PDU assembly. All triggered SBSRs shall be cancelled, and retx-BSR-TimerSL and periodic-BSR-TimerSL shall be stopped, when upper layers configure autonomous resource selection.

NOTE:	MAC PDU assembly can happen at any point in time between uplink grant reception and actual transmission of the corresponding MAC PDU. SBSR and SR can be triggered after the assembly of a MAC PDU which contains a SBSR MAC CE, but before the transmission of this MAC PDU. In addition, SBSR and SR can be triggered during MAC PDU assembly.

5.x.2	SL-SCH Data reception

5.x.2.1	SCI reception

SCI indicate if there is a transmission on SL-SCH and provide the relevant HARQ information. A SCI consists of two parts: an initial part of the SCI on PSCCH and the remaining part of the SCI on [PSSCH] as specified in [x].

Editor’s Note: FFS on support of a single SCI in RAN1.

The MAC entity shall:

1>	for each PSCCH duration during which the MAC entity monitors PSCCH:

2>	if an initial part of a SCI for this PSSCH duration has been received on the PSCCH:

3>	determine the set of PSSCH durations in which reception of the remaining part of the SCI and the transport block occur using the received part of the SCI;

3>	if the remaining part of the SCI for this PSSCH duration has been received on the [PSSCH]:

4>	store the SCI and associated HARQ information as SCI valid for the PSSCH durations corresponding to transmission(s) of the transport block;

1>	for each PSSCH duration for which the MAC entity has a valid SCI:

2>	deliver the SCI and the associated HARQ information to the Sidelink HARQ Entity.

5.x.2.2	Sidelink HARQ operation

5.x.2.2.1	Sidelink HARQ Entity

There is/are [x] Sidelink HARQ Entity at the MAC entity for reception of the SL-SCH, which maintains a number of parallel Sidelink processes.

Editor’s Note: FFS how many receiving Sidelink HARQ entities can be configured at the MAC entity for SL-SCH. e.g. One HARQ entity for broadcast, one HARQ entity for each group and one HARQ entity for each unicast link.

Each Sidelink process is associated with SCI in which the MAC entity is interested. This interest is as determined by the Destination ID of the SCI. The Sidelink HARQ Entity directs HARQ information and associated TBs received on the SL-SCH to the corresponding Sidelink processes.

Editor’s Note: FFS how this interest is determined based on SCI depending on RAN1 progress.

The number of Receiving Sidelink processes associated with the Sidelink HARQ Entity is defined in TS 38.331 [5] or TS 36.331 [xy].

For each PSSCH duration, the Sidelink HARQ Entity shall:

1>	for each SCI valid for this PSSCH duration:

2>	allocate the TB received from the physical layer and the associated HARQ information to a Sidelink process, associate this Sidelink process with this SCI and consider this transmission to be a new transmission.

1>	for each Sidelink process:

2>	if this PSSCH duration corresponds to retransmission opportunity for the Sidelink process according to its associated SCI:

3>	allocate the TB received from the physical layer and the associated HARQ information to the Sidelink process and consider this transmission to be a retransmission.

5.14.2.2.2	Sidelink process

For each PSSCH duration where a transmission takes place for the Sidelink process, one TB and the associated HARQ information is received from the Sidelink HARQ Entity.

For each received TB and associated HARQ information, the Sidelink process shall:

1>	if this is a new transmission:

2>	attempt to decode the received data.

1>	else if this is a retransmission:

2>	if the data for this TB has not yet been successfully decoded:

3>	instruct the physical layer to combine the received data with the data currently in the soft buffer for this TB and attempt to decode the combined data.

1>	if the data which the MAC entity attempted to decode was successfully decoded for this TB; or

1>	if the data for this TB was successfully decoded before:

2>	if this is the first successful decoding of the data for this TB and [if the DST field of the decoded MAC PDU subheader is equal to the [x] MSB of any of the Destination Layer-2 ID(s) of the UE for which the [y] LSB are equal to the Destination ID in the corresponding SCI]:

Editor’s Note: FFS for details of packet filetering.

3>	deliver the decoded MAC PDU to the disassembly and demultiplexing entity.

1>	else:

2>	instruct the physical layer to replace the data in the soft buffer for this TB with the data which the MAC entity attempted to decode.

1>	if HARQ feedback is configured with [a separate PSFCH resource i.e. option 2] for the Sidelink process; or

1>	if HARQ feedback corresponding to this TB is configured with [a shared PSFCH resource i.e. option 1] and the communication range calculated based on the SCI valid for this PSSCH duration according to [TS 38.xxx] is smaller or equal to the requirement indicated in the SCI valid for this PSSCH duration:

2>	instruct the physical layer to generate acknowledgement(s) of the data in this TB.

5.14.2.3	Disassembly and demultiplexing

The MAC entity shall disassemble and demultiplex a MAC PDU as defined in clause 6.x.

5.y	SL-BCH data transfer

5.y.1	SL-BCH data transmission

When instructed to send SL-BCH, the MAC entity shall:

-	obtain the MAC PDU to transmit from SBCCH;

-	deliver the MAC PDU to the physical layer and instruct it to generate a transmission.

5.y.2	SL-BCH data reception

When the MAC entity needs to receive SL-BCH, the MAC entity shall:

-	receive and attempt to decode the SL-BCH;

-	if a TB on the SL-BCH has been successfully decoded:

-	deliver the decoded MAC PDU to upper layers.

NEXT CHANGE

6.1.3.x	Sidelink Buffer Status Report MAC CEs

Sidelink Buffer Status Report (SBSR) MAC CEs consist of either:

-	SBSR format (variable size); or

-	Truncated SBSR format (variable size).

SBSR and Truncated SBSR MAC control elements consist of one Destination Index field, one LCG ID field and one corresponding Buffer Size field per reported target group.

The SBSR formats are identified by MAC subheaders with LCIDs as specified in in Table 6.2.1-2.

The fields in the SBSR MAC CE are defined as follows:

-	Destination Index: The Destination Index field identifies the destination. The length of this field is 5 bits. The value is set to one index among index(es) associated to same destination reported in [v2x-DestinationInfoList]. If multiple such lists are reported, the value is indexed sequentially across all the lists in the same order as specified in TS 38.331 [5];

-	LCG ID: The Logical Channel Group ID field identifies the group of logical channel(s) whose SL buffer status is being reported. The length of the field is 3 bits;

-	LCGi: For the SBSR format, this field indicates the presence of the Buffer Size field for the logical channel group i. The LCGi field set to 1 indicates that the Buffer Size field for the logical channel group i is reported. The LCGi field set to 0 indicates that the Buffer Size field for the logical channel group i is not reported. For the Truncated SBSR format, this field indicates whether logical channel group i has data available. The LCGi field set to 1 indicates that logical channel group i has data available. The LCGi field set to 0 indicates that logical channel group i does not have data available;

-	Buffer Size: The Buffer Size field identifies the total amount of data available according to the SL data volume calculation procedure in TSs 38.322 [3] and 38.323 [4] across all logical channels of a logical channel group of a destination after the MAC PDU has been built (i.e. after the logical channel prioritization procedure, which may result the value of the Buffer Size field to zero). The amount of data is indicated in number of bytes. The size of the RLC and MAC headers are not considered in the buffer size computation. The length of this field is 8 bits. The values for the Buffer Size field are shown in Table 6.1.3.1-2, respectively. For the SBSR format and the Truncated SBSR format, the Buffer Size fields are included in ascending order based on the LCGi. For the Truncated SBSR format the number of Buffer Size fields included is maximised, while not exceeding the number of padding bits.

NOTE:	The number of the Buffer Size fields in the SBSR and Truncated SBSR format can be zero.

Figure 6.1.3.x-1: SBSR and Truncated SBSR MAC control element

Editor’s Note: FFS on structure of SBSR and Truncated SBSR MAC CEs.

NEXT CHANGE

6.1.4	MAC PDU (transparent MAC)

A MAC PDU consists solely of a MAC SDU whose size is aligned to a TB; as described in Figure 6.1.4-1. This MAC PDU is used for transmissions on PCH, BCH, DL-SCH including BCCH, and SL-BCH.

Figure 6.1.4-1: Example of MAC PDU (transparent MAC)

NEXT CHANGE

6.1.x	MAC PDU (SL-SCH)

A MAC PDU consists of one SL-SCH subheader and one or more MAC subPDUs. Each MAC subPDU consists of one of the following:

-	A MAC subheader only (including padding);

-	A MAC subheader and a MAC SDU;

-	A MAC subheader and padding.

The MAC SDUs are of variable sizes.

Each MAC subheader except SL-SCH subheader corresponds to either a MAC SDU or padding.

The SL-SCH subheader is of a fixed size and consists of the seven header fields [V/R/R/R/R/SRC/DST].

Figure 6.1.x-1: SL-SCH MAC subheader

Editor’s Note: FFS on need of V field, and sizes of SRC/DST.

A MAC subheader except for padding consists of the four header fields R/F/LCID/L as depicted in Figure 6.1.2-1 (with 8-bit L field) and Figure 6.1.2-2 (with 16-bit L field). A MAC subheader for padding consists of the two header fields R/LCID as depicted in Figure 6.1.2-3.

SL MAC subPDU(s) with MAC SDU(s) is placed after the SL-SCH subheader and before the MAC subPDU with padding in the MAC PDU as depicted in Figure 6.1.x-1. The size of padding can be zero.

Figure 6.1.x-2: Example of a SL MAC PDU

A maximum of one MAC PDU can be transmitted per TB per MAC entity.

NEXT CHANGE

6.2.1	MAC subheader for DL-SCH and UL-SCH

The MAC subheader consists of the following fields:

-	LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE or padding as described in Tables 6.2.1-1 and 6.2.1-2 for the DL-SCH and UL-SCH respectively. There is one LCID field per MAC subheader. The LCID field size is 6 bits;

-	L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC CE in bytes. There is one L field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the L field is indicated by the F field;

-	F: The Format field indicates the size of the Length field. There is one F field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the F field is 1 bit. The value 0 indicates 8 bits of the Length field. The value 1 indicates 16 bits of the Length field;

-	R: Reserved bit, set to 0.

The MAC subheader is octet aligned.

Table 6.2.1-1 Values of LCID for DL-SCH

| Index | LCID values |
| --- | --- |
| 0 | CCCH |
| 1–32 | Identity of the logical channel |
| 33-46 | Reserved |
| 47 | Recommended bit rate |
| 48 | SP ZP CSI-RS Resource Set Activation/Deactivation |
| 49 | PUCCH spatial relation Activation/Deactivation |
| 50 | SP SRS Activation/Deactivation |
| 51 | SP CSI reporting on PUCCH Activation/Deactivation |
| 52 | TCI State Indication for UE-specific PDCCH |
| 53 | TCI States Activation/Deactivation for UE-specific PDSCH |
| 54 | Aperiodic CSI Trigger State Subselection |
| 55 | SP CSI-RS/CSI-IM Resource Set Activation/Deactivation |
| 56 | Duplication Activation/Deactivation |
| 57 | SCell Activation/Deactivation (four octets) |
| 58 | SCell Activation/Deactivation (one octet) |
| 59 | Long DRX Command |
| 60 | DRX Command |
| 61 | Timing Advance Command |
| 62 | UE Contention Resolution Identity |
| 63 | Padding |

Table 6.2.1-2 Values of LCID for UL-SCH

| Index | LCID values |
| --- | --- |
| 0 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]) |
| 1–32 | Identity of the logical channel |
| 33–[49] | Reserved |
| [50] | Sidelink BSR |
| [51] | Truncated Sidelink BSR |
| 52 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]) |
| 53 | Recommended bit rate query |
| 54 | Multiple Entry PHR (four octets Ci) |
| 55 | Configured Grant Confirmation |
| 56 | Multiple Entry PHR (one octet Ci) |
| 57 | Single Entry PHR |
| 58 | C-RNTI |
| 59 | Short Truncated BSR |
| 60 | Long Truncated BSR |
| 61 | Short BSR |
| 62 | Long BSR |
| 63 | Padding |

NEXT CHANGE

6.2.x	MAC subheader for SL-SCH

The MAC subheader consists of the following fields:

-	[V]: The MAC PDU format version number field indicates which version of the SL-SCH subheader is used. The V field size is [4] bits;

-	SRC: The Source Layer-2 ID field carries the identity of the source. The Source Layer-2 ID is set to the identifier provided by upper layers as defined in TS 23.287 [xx]. The SRC field size is [24] bits;

-	DST: The DST field size is [24] bits. The Destination Layer-2 ID is set to the identifier provided by upper layers as defined in TS 23.287 [xx]. [If the V field is set to "0001", this identifier is a groupcast identifier. If the V field is set to "0010", this identifier is a unicast identifier];

Editor’s Note: FFS on need of V field, and sizes of SRC/DST.

-	LCID: The Logical Channel ID field identifies the logical channel instance within the scope of one Source Layer-2 ID and Destination Layer-2 ID pair of the corresponding MAC SDU or padding as described in Tables 6.2.x-1 for SL-SCH. There is one LCID field per MAC subheader except for SL-SCH subheader. The LCID field size is 6 bits;

-	L: The Length field indicates the length of the corresponding MAC SDU in bytes. There is one L field per MAC subheader except for subheaders corresponding to the SL-SCH subheader or padding. The size of the L field is indicated by the F field;

-	F: The Format field indicates the size of the Length field. There is one F field per MAC subheader except for subheaders corresponding to the SL-SCH subheader or padding. The size of the F field is 1 bit. The value 0 indicates 8 bits of the Length field. The value 1 indicates 16 bits of the Length field;

-	R: Reserved bit, set to 0.

The MAC subheader is octet aligned.

Table 6.2.x-1 Values of LCID for SL-SCH

| Index | LCID values |
| --- | --- |
| 0 | Reserved |
| 1-16 | Identity of the logical channel |
| 17-62 | Reserved |
| 63 | Padding |

Editor’s Note: FFS on LCID values for SCCH.

NEXT CHANGE

7.1	RNTI values

RNTI values are presented in Table 7.1-1.

Table 7.1-1: RNTI values.

| Value (hexa-decimal) | RNTI |
| --- | --- |
| 0000 | N/A |
| 0001–FFF2 | RA-RNTI, Temporary C-RNTI, C-RNTI, MCS-C-RNTI, CS-RNTI, TPC-PUCCH-RNTI, TPC-PUSCH-RNTI, TPC-SRS-RNTI, INT-RNTI, SFI-RNTI, SP-CSI-RNTI, SL-RNTI, SLCS-RNTI and SL Semi-Persistent Scheduling V-RNTI |
| FFF3–FFFD | Reserved |
| FFFE | P-RNTI |
| FFFF | SI-RNTI |

Table 7.1-2: RNTI usage.

| RNTI | Usage | Transport Channel | Logical Channel |
| --- | --- | --- | --- |
| P-RNTI | Paging and System Information change notification | PCH | PCCH |
| SI-RNTI | Broadcast of System Information | DL-SCH | BCCH |
| RA-RNTI | Random Access Response | DL-SCH | N/A |
| Temporary C-RNTI | Contention Resolution (when no valid C-RNTI is available) | DL-SCH | CCCH, DCCH |
| Temporary C-RNTI | Msg3 transmission | UL-SCH | CCCH, DCCH, DTCH |
| C-RNTI, MCS-C-RNTI | Dynamically scheduled unicast transmission | UL-SCH | DCCH, DTCH |
| C-RNTI | Dynamically scheduled unicast transmission | DL-SCH | CCCH, DCCH, DTCH |
| MCS-C-RNTI | Dynamically scheduled unicast transmission | DL-SCH | DCCH, DTCH |
| C-RNTI | Triggering of PDCCH ordered random access | N/A | N/A |
| CS-RNTI | Configured scheduled unicast transmission (activation, reactivation and retransmission) | DL-SCH, UL-SCH | DCCH, DTCH |
| CS-RNTI | Configured scheduled unicast transmission (deactivation) | N/A | N/A |
| TPC-PUCCH-RNTI | PUCCH power control | N/A | N/A |
| TPC-PUSCH-RNTI | PUSCH power control | N/A | N/A |
| TPC-SRS-RNTI | SRS trigger and power control | N/A | N/A |
| INT-RNTI | Indication pre-emption in DL | N/A | N/A |
| SFI-RNTI | Slot Format Indication on the given cell | N/A | N/A |
| SP-CSI-RNTI | Activation of Semi-persistent CSI reporting on PUSCH | N/A | N/A |
| [SL-RNTI] | Dynamically scheduled sidelink transmission | SL-SCH | SCCH, STCH |
| [SLCS-RNTI] | Configured scheduled sidelink transmission (activation, reactivation and retransmission) | SL-SCH | SCCH, STCH |
| [SLCS-RNTI] | Configured scheduled sidelink transmission (deactivation) | N/A | N/A |
| SL Semi-Persistent Scheduling V-RNTI (NOTE y) | Semi-Persistently scheduled sidelink transmission for V2X sidelink communication (activation, reactivation and [retransmission]) | SL-SCH (for the other MAC entity) | STCH (for the other MAC entity) |
| SL Semi-Persistent Scheduling V-RNTI (NOTE y) | Semi-Persistently scheduled sidelink transmission for V2X sidelink communication (deactivation) | N/A | N/A |
| NOTE x: The usage of MCS-C-RNTI is equivalent to that of C-RNTI in MAC procedures (except for the C-RNTI MAC CE). NOTE y: The MAC entity uses SL Semi-Persistent Scheduling V-RNTI to control SL Semi-Persistent Scheduling in V2X sidelink communication of the other MAC entity (i.e. E-UTRA MAC entity) as specified in TS 36.321 [xy]. |  |  |  |

END OF THE CHANGE