3GPP TSG-RAN WG2 Meeting #107bis	R2-1913834

# Chongqing, China, 14th - 18th October 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0173 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on measurement gap for LTE inter-RAT measurement in NR SA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., Nokia (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 10/14/2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For release-15, RAN2 has agreed that UE capability for the need for measurement gaps is not needed. It is agreed that the network always configures measurement gaps when the UE needs it. In 38.300 CR 0133 (R2-1818870), it is clarified (in section 9.2.4) that which cases the measurement gap is required for SSB-based intra and inter frequency measurement in NR SA. However, it is still unclear whether the measurement gap is required for inter-RAT E-UTRA measurement in NR SA. We understand that LTE Inter-RAT measurement applies the same principle as NR inter-frequency measurements: - For UEs only support per-UE gap, gap is always needed; - For UEs support per-FR gap, gap is needed if serving cell(s) and target cell(s) are in the same frequency range. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following text in 9.3.1.3 and 9.3.2.3: For a UE configured with E-UTRA Inter RAT measurements, a measurement gap configuration is always provided when: - The UE only supports per-UE measurement gaps; or - The UE supports per-FR measurement gaps and any of the configured BWP frequencies of any of the NR serving cells is in the same frequency range of the E-UTRA measurement object. Impact analysis Impacted 5G architecture options: Standalone Impacted functionality: Measurement gap configuration for inter-RAT E-UTRA measurement Inter-operability: If the UE implements this CR and the network does not, the UE will not be able to measure E-UTRA when network doesn’t configure measurement gap when needed. If the network implements this CR and the UE does not, there is no impact to the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear that in which cases the UE can perform inter-RAT E-UTRA measurements without gaps and so networks might not configure gaps in cases where they are needed. If gaps are not provided in these cases then the UE will not report measurements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.3, 9.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 9.3	Inter RAT

### 9.3.1	Intra 5GC

#### 9.3.1.1	Cell Reselection

Cell reselection is characterised by the following:

-	Cell reselection between NR RRC_IDLE and E-UTRA RRC_IDLE is supported;

-	Cell reselection from NR RRC_INACTIVE to E-UTRA RRC_IDLE is supported.

#### 9.3.1.2	Handover

Inter RAT mobility is characterised by the following:

-	The Source RAT configures Target RAT measurement and reporting.

-	The source RAT decides on the preparation initiation and provides the necessary information to the target RAT in the format required by the target RAT:

-	For handover preparation from E-UTRA to NR, the source RAT issues a handover preparation request message to the target RAT passing a transparent RRC container with necessary information to prepare the handover at the target side. The information for the target RAT is the same type as specified in section 9.2.3.2.1 including the current QoS flow to DRB mapping applied to the UE and RRM configuration.

-	The details of RRM configuration are the same type as specified for NR in section 9.2.3.2.1 including beam measurement information for the listed cells if the measurements are available.

-	Radio resources are prepared in the target RAT before the handover.

-	The RRC reconfiguration message from the target RAT is delivered to the source RAT via a transparent container, and is passed to the UE by the source RAT in the handover command:

-	The inter-RAT handover command message carries the same type of information required to access the target cell as specified for NR baseline handover in section 9.2.3.2.1.

-	The in-sequence and lossless handover is supported for the handover between gNB and ng-eNB.

-	Both Xn and NG based inter-RAT handover between NG-RAN nodes is supported. Whether the handover is over Xn or CN is transparent to the UE.

-	In order to keep the SDAP and PDCP configurations for in-sequence and lossless inter-RAT handover, delta-configuration for the radio bearer configuration is used.

#### 9.3.1.3	Measurements

Inter RAT measurements in NR are limited to E-UTRA.

For a UE configured with E-UTRA Inter RAT measurements, a measurement gap configuration is always provided when:

-	The UE only supports per-UE measurement gaps; or

-	The UE supports per-FR measurement gaps and any of the configured BWP frequencies of any of the NR serving cells is in the same frequency range of the E-UTRA measurement object.

### 9.3.2	From 5GC to EPC

#### 9.3.2.1	Cell Reselection

Cell reselection is characterised by the following:

-	Cell reselection between NR RRC_IDLE and E-UTRA RRC_IDLE is supported;

-	Cell reselection from NR RRC_INACTIVE to E-UTRA RRC_IDLE is supported.

#### 9.3.2.2	Handover and redirection

The source NG-RAN node decides between handover or redirection to EPS based on radio criteria and availability of the N26 interface.

NOTE:	Information about the availability of the N26 interface may be configured by OAM at the NG-RAN.

Inter RAT handover is characterised by the following:

-	The Source RAT configures Target RAT measurement and reporting.

-	The source RAT decides on the preparation initiation and provides the necessary information to the target RAT in the format required by the target RAT.

-	Radio resources are prepared in the target RAT before the handover.

-	The RRC reconfiguration message from the target RAT is delivered to the source RAT via a transparent container, and is passed to the UE by the source RAT in the handover command.

-	In-sequence and lossless handovers are not supported.

-	Security procedures for handover to E-UTRA/EPC should follow legacy inter-RAT handover procedures.

#### 9.3.2.3	Measurements

Inter RAT measurements in NR are limited to E-UTRA.

For a UE configured with E-UTRA Inter RAT measurements, a measurement gap configuration is always provided when:

-	The UE only supports per-UE measurement gaps; or

-	The UE supports per-FR measurement gaps and any of the configured BWP frequencies of any of the NR serving cells is in the same frequency range of the E-UTRA measurement object.