3GPP TSG-RAN2 Meeting #107bis 	R2-1913851

# Chongqing, China, 14 – 18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1346 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to mobility from NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In mobility from NR to E-UTRA/5GC, the UE should not release the associated SDAP and PDCP for all estabablished DRBs. The UE releases the associated SDAP and PDCP for all estabablished DRBs in mobility from NR to E-UTRA/EPC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The UE releases the associated SDAP and PDCP for all estabablished DRBs in mobility from NR to E-UTRA/EPC. Impact analysis Impacted 5G architecture options: NR SA, NR-NR DC, NE-DC Impacted functionality: Mobility from NR to E-UTRA Inter-operability: If the UE is implemented according to this CR and network is not, there is no inter-operability issue given that the changes are only relevant to SDAP and PDCP handling at the UE. If network is implemented according to this CR and UE is not, there is no inter-operability issue given that the changes are only relevant to SDAP and PDCP handling at the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In mobility from NR to E-UTRA/5GC, the UE releases the associated SDAP and PDCP for all estabablished DRBs in E-UTRA/5GC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.4.3.4	Successful completion of the mobility from NR

Upon successfully completing the handover, at the source side the UE shall:

1>	reset MAC;

1>	stop all timers that are running;

1>	release ran-NotificationAreaInfo, if stored;

1>	release the AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key, if stored;

1>	release all radio resources, including release of the RLC entity and the MAC configuration;

NOTE :	PDCP and SDAP configurations configured by the source RAT prior to the handover that are reconfigured and re-used by target RAT when delta signalling (i.e., during inter-RAT intra-system handover when fullConfig is not present) is used, are not released as part of this procedure.

1>	if the targetRAT-Type is set to eutra and the nas-SecurityParamFromNR is included:

2>	release the associated PDCP entity and SDAP entity for all established RBs;

2>	indicate the release of the RRC connection to upper layers together with the release cause 'other'.