3GPP TSG-RAN WG2 Meeting #107bis     		 	     R2-1913874

Chongqing, China, 14th - 18th October 2019                 Revision of R2-1909828

Agenda Item	: 6.13.3 (MAC PDU format)

Source	: LG Electronics Inc.

Title	: MAC PDU formats for msgB

Document for	: Discussion and Decision

1.	Introduction

At RAN2 #106, the following agreements for 2-step RACH were made:

Agreements

- From RAN2 perspective, 2-step RACH selections can be based on indicating to all UEs via SIB, or dedicated configuration in RRC_CONNECTED/INACTIVE/IDLE states.  FFS if radio quality is used for 2-step RACH selection.

- From RAN2 perspective, for msgA retransmission (i.e. preamble and PUSCH) we assume that the UE retries on 2-step RACH

- FFS whether the UE can fallback to 4-step RACH after certain time.  Ask RAN1 whether the preamble transmission performance for 2-step RACH and 4-step RACH is the same.

- For MsgA with C-RNTI, the UE shall monitor the PDCCH addressed to C-RNTI for success response and msgB-RNTI (e.g. RA-RNTI or new RNTI)

- Contention resolution:

  - If the PDU PDCCH addressed to the C-RNTI (i.e. C-RNTI included in MsgA) containing the 12 bit TA command is received, the UE should consider the contention resolution to be successful and stop the reception of MsgB or with UL grant if the UE is synchronized already.

  - If the corresponding fallback RAR is detected, the UE should stop the monitoring of PDCCH addressed to the corresponding C-RNTI for success response and process the fallback operation accordingly.

  - If neither corresponding fallback RAR nor PDCCH addressed C-RNTI is detected within the response window, the UE should consider the msgA attempt failed and do back off operation based on the backoff indicator if received in MsgB.

  - FFS if a new MAC CE with 12bits Timing Advanced Command shall be introduced

- For CCCH, MsgB can include the SRB RRC message.  The format should be designed for both with and without RRC message.

- For CCCH, for success or fallback RAR MsgB can multiplex messages for multiple UEs.  FFS if we can multiplex SRB RRC messages of multiple UEs.

- Network response to msgA (i.e. msgB/msg2) can include the following:

  - SuccessRAR

  - FallbackRAR

  - Backoff Indication

FFS: format of successRAR and whether successRAR is split into more than one message and format of fallbackRAR and whether legacy msg2 can be reused for fallbackRAR

- Proposal 10: The following fields can be included in the successRAR when CCCH message is included in msgA.

  - Contention resolution ID

  - C-RNTI

  - TA command

- Upon receiving the fallbackRAR, the UE shall proceed to msg3 step of 4-step RACH procedure

- FallbackRAR should contain the following fields

  - RAPID

  - UL grant (to retransmit the msgA payload).  FFS on restrictions on the grant and UE behavior if different grant and rebuilding

  - TC-RNTI

  - TA command

- From RAN2 perspective, no further offset is needed for the start of msgB monitoring window (i.e. no offset is needed to cover the RRC processing delay and/or F1 delay).

- The UE will monitor for response message using the single msgB agreed window

- MsgB containing the succcessRAR shall not be multiplexed with the legacy 4-step RACH RAR in the same MAC PDU

During the RAN2#107 meeting, the following agreements were made:

Agreements:

- Working assumption: SRB RRC messages of multiple UEs cannot be multiplexed in same msg B (i.e. same MAC PDU).

- successRAR cannot be split into more than one message (i.e.Contention resolution ID will also be included in successRAR).

- SuccessRAR and fallbackRAR can be multiplexed

Agreements:

- The fallback RAR shall be included in the general MsgB format, ie., be able to be multiplexed with the successRAR for 2-step RACH.

- TB size offered in UL grant in the fallback RAR shall be the same as the TB size offered for payload transmission in MsgA; otherwise, the UE behavior is not defined (i.e. it is up to UE implementation).

- RA type selection is performed before beam selection

- No need to reexecute RA selection criteria upon fallback failure (i.e if reception of msg3 fails).  The UE re-transmits using msgA

In this contribution, we’d like to address the possible ways based on the remaining issues which can affect the MAC PDU formats of msgB including fallbackRAR, successRAR and BI, and suggest the potential MAC PDU formats according to the candidates.

2.	Discussion

2-1. MAC subheader for msgB

In the last meeting, since the RAN2 agreed that the fallback RAR shall be included in the general MsgB format, RAN2 should design a new msgB for either fallbackRAR, successRAR or BI. Given that the msgB is designed based on the legacy msg2, we may use one of the reserved bits for indicating the successRAR from the fallbackRAR or BI. For efficient parsing of the msgB, several companies suggested to include the RAPID even in the successRAR. If then, the successRAR payload may be indicated by the remaining reserved 1-bit of the legacy RAR payload, but we don’t think that the successRAR does not need to include the RAPID because the contention of 2-step RACH can be totally resolved by the CRID, not RAPID. So, if we should indicate three different types of RARsin a MAC PDU, we propose to use the reserved bit as a new Type field in the BI sub-header as shown in Figure 2.

Figure 2. MAC PDU format for MsgB candidate #2

That is, a UE receiving the msgB checks T1 field indicating whether the MAC subheader contains a RAPID or others. If the T1 indicates that the MAC sub-header contains others, the UE additionally checks the T2 field indicating whether the MAC sub-header contains a Backoff Indicator or a CRID. If the T2 indicates to include a CRID, the UE can detect a CRID sub-header and a successRAR payload following the CRID sub-header. In this format, the fallbackRAR can be detected by the T1 in the same way as the legacy RAR.

Proposal 1. RAN2 defines MAC subPDU consisting of CRID sub-header including a CRID and successRAR payload including at least a C-RNTI and a TA command.

Proposal 2. The CRID sub-header consists of at least E/T1/T2/CRID and the BI sub-header consists of E/T1/T2/R/BI.

- The T1 field indicates whether the MAC sub-header contains a RAPID or others.

- The T2 field indicates whether the MAC sub-header contains a BI or a CRID.

2-2. Structure of successRAR

According to the agreements at the last meeting, SRB RRC messages of multiple UEs cannot be multiplexed in the same MsgB. However, it is still a working assumption. The structure of the successRAR can vary depending on whether the SRB RRC messages can be multiplexed into one MAC PDU. In our understanding, if RAN2 allows multiplexing SRB RRC messages, it means that the successRARs with SRB RRC message can be multiplexed into a single MAC PDU. If we don’t allow multiplexing SRB RRC messages, it is understood that only successRARs without SRB RRC message should be multiplexed into a single MAC PDU. In the latter case, the UE can receive the SRB RRC message addressed to the C-RNTI after successfully receiving the successRAR and only one successRAR for a UE will solely be transmitted even if the successRAR with SRB RRC message is transmitted. But, the former case would need a method to efficiently extract the desired SRB RRC message from the multiplexed MAC PDU.

[SuccessRAR candidate #1] If we allow multiplexing SRB RRC messages into one MAC PDU, we should design the structure of the successRAR considering the efficient parsing due to the variable-sized MAC SDU (i.e., SRB RRC message) per successRAR. As described in section 1, if receiving the CRID matching with the CCCH SDU which was transmitted in msgA, the UE can acquire C-RNTI and TA command corresponding to the CRID. Else, the UE should try to decode the following sub-header. In the legacy RAR, MAC subPDU for a RAR (i.e., RAPID sub-header + RAR payload) has a fixed-size. This helps in easy parsing of the following sub-header by skipping the undesired RAR payload. However, since the successRAR with SRB RRC message can have a variable size, this makes it difficult to directly jump to the following subheader if the successRAR does not contain its CRID. . In order for a UE to easily detect its successRAR or the following subheader, therefore, we propose to include the length field indicating the length of the SRB RRC message in the CRID sub-header. The Figure 3 shows an example of the MAC PDU format, which a BI and successRARs are multiplexed into one MAC PDU.

Figure 3. MAC PDU format for SuccessRAR candidate #1

As shown in Figure 3, if a UE detects a CRID sub-header including a CRID which does not match the CCCH SDU transmitted in msgA and this is not the last MAC subPDU, the UE can jump to the next sub-header based on the length field without parsing the undesired successRAR. Also, if a UE detects a CRID sub-header including a CRID that matches the CCCH SDU transmitted in msgA, the UE can only acquire its C-RNTI, TAC and SRB RRC message based on the length field.

[SuccessRAR candidate #2] If we don’t allow multiplexing SRB RRC messages into one MAC PDU, we can design the structure of the fixed-sized successRAR. But, since the successRAR can include a SRB RRC message even if the SRB RRC messages are not multiplexed, a new field for indicating whether the SRB RRC message is included in this MAC PDU is required. Since a SRB RRC message can correspond to a successRAR of a UE and in this candidate it will be solely transmitted without any other RARs, it seems to be best way to define the 1-bit indicator as a content of successRAR. Figure 4 shows examples for the MAC PDU formats including the 1-bit indicator, marked with “S”.

- SuccessRAR w/ SRB RRC message            (b) Multiplexed successRARs (w/o SRB RRC message)

Figure 4. MAC PDU format for SuccessRAR candidate #2

As shown in Figure 4-(a), if a successRAR with SRB RRC message is solely transmitted without any other RARs, the S field in the successRAR payload indicates whether the SRB RRC message is included or not in this MAC PDU. If the successRARs without SRB RRC message are multiplexed into one MAC PDU as in Figure 4-(b), the indicator will be always set the value indicating that there is no SRB RRC message.

Proposal 3-1. If RAN2 allows multiplexing SRB RRC messages into one MAC PDU, the CRID sub-header additionally includes a Length field indicating the length of the SRB RRC message.

Proposal 3-2. If RAN2 does NOT allow multiplexing SRB RRC messages into one MAC PDU, the successRAR payload additionally includes a new S field indicating whether the SRB RRC message is included in this MAC PDU.

3.	Conclusion

In this contribution, we address the possible ways based on the remaining issues which can affect the MAC PDU formats of msgB including fallbackRAR, successRAR and BI, and our proposals are as follows:

Proposal 1. RAN2 defines MAC subPDU consisting of CRID sub-header including a CRID and successRAR payload including at least a C-RNTI and a TA command.

Proposal 2. The CRID sub-header consists of at least E/T1/T2/CRID and the BI sub-header consists of E/T1/T2/R/BI.

- The T1 field indicates whether the MAC sub-header contains a RAPID or others.

- The T2 field indicates whether the MAC sub-header contains a BI or a CRID.

Proposal 3-1. If RAN2 allows multiplexing SRB RRC messages into one MAC PDU, the CRID sub-header additionally includes a Length field indicating the length of the SRB RRC message.

Proposal 3-2. If RAN2 does NOT allow multiplexing SRB RRC messages into one MAC PDU, the successRAR payload additionally includes a new S field indicating whether the SRB RRC message is included in this MAC PDU.