3GPP TSG-RAN2 Meeting #107bis 	R2-1913970

# Chongqing, China,  14 – 18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1295 | rev | 1 | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reconfiguration with sync for NE-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google Inc., Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In NE-DC, a UE suspends all SCG DRBs due to SCG failure as specified in 36.331. ### 5.6.13 SCG failure information … The purpose of this procedure is to inform E-UTRAN about an SCG failure the UE has experienced i.e. SCG radio link failure, SCG change failure. #### 5.6.13.2 Initiation A UE initiates the procedure to report SCG failures when SCG transmission is not suspended and when one of the following conditions is met: 1> upon detecting radio link failure for the SCG, in accordance with 5.3.11; or 1> upon SCG change failure, in accordance with 5.3.5.7a; or 1> upon stopping uplink transmission towards the PSCell due to exceeding the maximum uplink transmission timing difference when powerControlMode is configured to 1, in accordance with clause 7.17.2 of TS 36.133 [29]. In case of DC, upon initiating the procedure, the UE shall: 1> suspend all SCG DRBs and suspend SCG transmission for split DRBs; 1> reset SCG-MAC; 1> stop T307; 1> if the UE is configured with NE-DC: 2> initiate transmission of the SCGFailureInformationEUTRA message via the NR MCG as specified in TS 38.331 [82], clause 5.7.3a; 1> else: 2> initiate transmission of the SCGFailureInformation message in accordance with 5.6.13.3;. The current text prevents the network from initiating reconfiguration with sync for the UE with only SCG DRBs which are suspended due to SCG failure. - the reconfigurationWithSync is included in masterCellGroup only when AS security has been activated, and SRB2 with at least one DRB are setup and not suspended. If a condition for reconfiguration with sync is triggered for the UE with suspended SCG DRBs, the network is not able to initiate reconfiguration with sync for the UE. |
| Summary of change: |  | The reconfigurationWithSync is included in masterCellGroup only when AS security has been activated, SRB2 is setup and not suspended, at least one DRB is setup. Impact analysis Impacted 5G architecture options: NE-DC Impacted functionality: Reconfiguration with sync in MCG Inter-operability: If the UE is implemented according to the CR but the network is not, there is no inter-operability issue. However, the network is not able to initiate reconfiguration with sync for the UE which has a suspended DRB in SCG. If the network is implemented according to the CR but the UE is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network is not able to initiate reconfiguration with sync for the UE which has a suspended DRB in SCG. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R2-1913268 |  |  |  |  |  |  |  |  |

#### 5.3.5.2	Initiation

The Network may initiate the RRC reconfiguration procedure to a UE in RRC_CONNECTED. The Network applies the procedure as follows:

-	the establishment of RBs (other than SRB1, that is established during RRC connection establishment) is performed only when AS security has been activated;

-	the addition of Secondary Cell Group and SCells is performed only when AS security has been activated;

-	the reconfigurationWithSync is included in secondaryCellGroup only when at least one DRB is setup in SCG;

-	the reconfigurationWithSync is included in masterCellGroup only when AS security has been activated, SRB2 is setup and not suspended, and at least one DRB is setup.