3GPP TSG-RAN WG2 Meeting #107bis 	R2-1914068

# Chongqing, China, 14th - 18th October 2019

Title:	LS on PHR reporting for NR-U

Response to:

Release:	Rel-16

Work Item:	NR_unlic-Core

Source:	RAN2

To:	RAN1

Cc:

Contact Person:

#### Name: 	Joachim Löhr

Tel. Number:	+49172 6991470

E-mail Address:	jlohr@lenovo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 acknowledged one problem with current defined power headroom reporting functionality for NR-U when PHR is reported for a serving cell configured with two UL carriers.  Due to LBT failures, gNB is not aware of whether a PHR type-1 or PHR type-3 is reported when the PHR MAC CE is transmitted on a configured grant, since gNB is not aware of the first transmission attempt of the TB including the PHR MAC CE.

RAN2 discussed several options on how to solve the identified problem, e.g. reporting a predefined existing PHR-type (e.g. PHR type 1 or type 3) for cases when the PHR MAC CE is transmitted on a configured grant on an unlicensed cell, but did not conclude on any solution.

2. Actions:

To RAN1

ACTION: 	RAN2 respectfully asks RAN1 to discuss this problem and determine whether a solution is needed.

3. Date of Next TSG-RAN2 Meetings:

RAN2#108 		18-22 November 2019 		Reno, Nevada, US

RAN2#109		24-28 Feb 2020			Athens, GR