3GPP TSG RAN WG2#108	R2-1914308

Reno, USA, 18 – 22 November, 2019

3GPP TSG-RAN WG1 Meeting #98bis 	R1-1911617

# Chongqing, China, October 14th – 20th, 2019

Title:	LS on simultaneous TCI state activation and spatial relation update across multiple CCs/BWPs by MAC-CE

Response to:

Release:	Rel-16

Work Item:	NR_eMIMO-Core

Source:	3GPP TSG-RAN WG1

To:	3GPP TSG-RAN WG2

Contact Person:

#### Name:	Jonghyun Park

E-mail Address:	jonghyun10.park@lge.com

Attachments:	None

1. Overall Description:

RAN1 has discussed simultaneous TCI state activation across multiple CCs/BWPs by MAC-CE and simultaneous spatial relation update across multiple CCs/BWPs by MAC-CE. RAN1 has made the following agreements and working assumptions:

Agreement

When a set of TCI-state IDs for PDSCH are activated by a MAC CE for a set of CCs/BWPs at least for the same band, where the applicable list of CCs is indicated by RRC signalling, the same set of TCI-state IDs are applied for the all BWPs in the indicated CCs.

- Further signaling details are up to RAN2.

- Whether to support the inter-band CA for this feature will be decided in RAN1#99.

- Whether to indicate the applicable list of bands for the feature of single MAC-CE to activate the same set of PDSCH TCI state IDs for multiple CCs/BWPs is up to capability discussion.

  - FFS on the UE capability signaling details

- Note: This at least applies to single TRP case.

- FFS: How many combinations of CCs can be configured by RRC and relevant UE capability

Agreement

When a TCI-state ID is activated for a CORESET by a MAC CE for a set of CCs/BWPs at least for the same band, where the applicable list of CCs is indicated by RRC signalling, the TCI-state ID is applied for the CORESET(s) with the same CORESET ID for all the BWPs in the indicated CCs.

- Further signaling details are up to RAN2.

- Whether to support the inter-band CA for this feature will be decided in RAN1#99.

- Whether to indicate the applicable list of bands for the feature of single MAC-CE to activate the same PDCCH TCI state IDs for multiple CCs/BWPs is up to capability discussion.

  - FFS on the UE capability signaling details

- Note: This at least applies to single TRP case.

Agreement

For the purpose of simultaneous TCI state activation across multiple CCs/BWPs,

- Up to 2 lists of CCs can be configured by RRC per UE, and the applied list is determined by the indicated CC in the MAC CE.

  - UE expect no overlapped CC in multiple RRC-configured lists of CCs.

  - Send LS to RAN2 to inform above

Working assumption:

When a Spatial Relation Info is activated for a SP/AP SRS resource by a MAC CE for a set of CCs/BWPs at least for the same band, where the applicable list of CCs is indicated by RRC signalling, the Spatial Relation Info is applied for the SP/AP SRS resource(s) with the same SRS resource ID for all the BWPs in the indicated CCs.

- Further signaling details are up to RAN2.

- Whether to support the inter-band CA for this feature will be decided in RAN1#99.

- Whether to indicate the applicable list of bands for the feature of single MAC-CE to activate the same SRS resource IDs for multiple CCs/BWPs is up to capability discussion.

- FFS on the UE capability signaling details

- Note: This at least applies to single TRP case.

- FFS on the power control details (without RAN2 impact)

Working assumption:

For the purpose of simultaneous Spatial Relation update across multiple CCs/BWPs,

- Up to 2 lists of CCs can be configured by RRC per UE, and the applied list is determined by the indicated CC in the MAC CE.

  - UE expect no overlapped CC in multiple RRC-configured lists of CCs.

  - The lists are independent from those for simultaneous TCI state activation

  - Send LS to RAN2 to inform above

2. Actions:

To RAN2 group:

ACTION: 	RAN1 respectfully asks RAN2 to take the above agreements and working assumptions into account for future RAN2 work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #99 	18 - 22 November 2019    Reno, USA

TSG-RAN WG1 Meeting #100 	24 – 28 February 2020    Athens, Greece