3GPP TSG-RAN WG4 Meeting #93                                                                 R4-1915358

Reno, USA, 18 – 22 Nov, 2019

Title:	Draft reply LS on BCS reporting for EN-DC BC

Response to:	R2-1914226

Release:	Release 15

Work Item:	NR_newRAT-Core

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact Person:

Name:	Liu Ye

E-mail Address:	leo.liuye@huawei.com

Attachments:	None

1. Overall Description:

RAN4 has discussed the EN-DC BCS questions raised by RAN2.

Method of a product set of channel bandwidths in inter-band including intra-band EN-DC combination for BCS reporting was adopted in RAN4. Product set means common set of channel bandwidths among different BCS reported by the UE for LTE or NR carriers in the inter-band EN-DC band combination. RAN4 confirms that RAN2 understanding is correct that at most the following types of BCS need to be signalled.

1)	BCS for LTE CA component

2)	BCS for NR CA component

3)	BCS for intra-band EN-DC component

For the specific examples, RAN4 would like to clarify how to report BCS according to the generic principle.

- Intra-band EN-DC without LTE CA or NR CA, e.g. DC_(n)41AA, DC_3A_n3A

Answer: For this kind of intra-band EN-DC combinations, only BCS for intra-band EN-DC should be reported.

- Intra-band EN-DC with LTE CA and/or NR CA inside, e.g. DC_(n)41CA, DC_41C_n41A

Answer: For this kind of intra-band EN-DC combinations, BCS for intra-band EN-DC should be reported, e.g. for DC_(n)41CA, only BCS for DC_(n)41CA should be reported.

- Intra-band EN-DC with additional inter-band LTE CA and/or NR CA, e.g. DC_2A-(n)71AA, DC_(n)41AA_n77C

Answer: For this kind of inter plus intra-band EN-DC combinations, BCS for intra-band EN-DC and BCS for inter-band LTE CA and/or BCS for inter-band NR CA should be reported, e.g. for DC_2A-(n)71AA, both BCS for DC_(n)71AA and BCS for CA_2A-71A should be reported. The UE shall support a product or common set of channel bandwidths for the LTE or NR carrier.

2. Actions:

To RAN2

ACTION: 	RAN4 kindly asks RAN2 to take the above information into account in the future work.

3. Date of Next TSG-RAN WG4 Meetings:

3GPP RAN4 #94		February 24 – 28, 2020		Athens, Greece

3GPP RAN4 #94bis		April 20 – 24, 2019    			China