# 3GPP TSG-RAN WG2 #108	R2-1916435

# Reno, Nevada, US, 18 – 22 November 2019

Title:	LS reply on PUR transmission for NB-IoT/eMTC

Response to:	R2-1914304/R1-1911399

Release:	Release 16

Work Item:	NB_IOTenh3-Core, LTE_eMTC5-Core

Source:	RAN2

To:	RAN1

Cc:	RAN4

Contact Person:

#### Name:	Tuomas Tirronen

E-mail Address:	tuomas.tirronen@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 thanks RAN1 for their LS on PUR transmission for NB-IoT/eMTC. In the LS RAN1 has provided the following questions to RAN2:

- What is the range of the non-zero gap considering the followings questions

  - What is the proportion (how often) of PUR transmissions that have an application layer response after a PUR transmission.

  - What is the time duration between a PUR transmission and the corresponding application layer response.

How frequently a PUR transmission has an application layer response or the time duration between a PUR transmission and the corresponding application layer response is not deterministic considering that application layer aspects are transparent to the eNB and PUR is not designed for a particular application in mind.

Therefore, RAN2 considers that the gap as intended by RAN1 would not be beneficial. Furthermore, RAN2 has agreed that UE is not required to continue monitoring PDCCH after receiving L1 ACK and therefore RAN2 thinks an indication in PUR L1 ACK should not be used in the way RAN1 has discussed, i.e. as an application layer response after a gap. RAN2 made the following agreements in RAN2#108:

Agreements - After L1 ACK the PUR procedure is terminated and UE is not required to continue monitoring PDCCH after this (as per previous agreements). - L1 ACK is not intended for application layer response

2. Actions:

To RAN1

ACTION:

RAN2 respectfully request RAN1 to take the above information into account.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #109	2020-02-24 to 2020-02-28	Athens, Greece

TSG-RAN WG2 Meeting #109bis	2020-04-20 to 2020-04-24	TBD, Japan