3GPP TSG RAN WG2#109-e                                                                     R2-2000107

Electronic meeting, 24th February - 6th March, 2020

Title:	Draft Reply LS on Information Needed for MRO in UE RLF Report

Response to:	R3-197668

Release:	Rel-16

Work Item:	          NR_SON_MDT-Core

Source:	RAN2

To:	RAN3

Cc:	-

Contact Person:

Name:	Fan Jiangsheng

E-mail Address:	Fanjiangsheng (at) catt (dot) cn

- Overall Description:

RAN2 thanks RAN3 for their reply LS on MRO, RACH and RLF report.

After further discussion, RAN2 reached more agreements on other information which is necessary for MRO in UE RLF Report as below:

- The TAI of the failed cell is introduced in RLF report. As the source cell could know how to route the RLF INDICATION message based on the information of neighbor cell relationship, so it’s not necessary to introduce the TAI of the source cell to route of MRO signaling messages. More addition, RAN2 thinks one bit indicator is needed to differentiate between intra-system HO and inter-system HO.

- For introducing re-connection attempt cell CGI in RLF report, RAN2 thinks for the inter-system case, there is not any prepared inter-RAT cell, and the re-connection attempt cell CGI would help the network to detect the root cause of the failure. But for intra-system case, if the UE cannot re-establish to a prepared cell, the UE will go to IDLE. And whether the UE will quickly attempt to connect to a cell using the RRC establishment procedure will depend on the UE implementation. Since the limited time for R16, RAN2 would like to ask RAN3 to postpone this feature to R17.

- As for the agreement “LTE RLF can be reported in NR. How to support this is FFS”, the intention of RAN2 is that reporting LTE RLF Report to NR could be supported, but reporting NR RLF report to LTE is not supported. The UE will use a container to carry LTE RLF report when reporting LTE RLF report to a NR cell.

- 2. Actions:

To RAN3:

ACTION: RAN2 respectfully asks RAN3 to take the above into account.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2 Meeting #109bis	    20th-24th   Apr 2020	    Sapporo, Japan

TSG-RAN2 Meeting #110              25th-29th    May 2020         Athen,     Greece