3GPP TSG-RAN WG2 Meeting #109	electronic								    R2-2000146

E-Meeting, 24th Feb – 6th Mar, 2020							    	 revision of R2-1914366

Source: 	vivo

Title:  	Issue on the Autonomous BWP Switching in NR-U

Agenda Item:	6.2.2.1

Document for:	Discussion and Decision

Introduction

According to the current MAC specification, the UE could autonomously switch its active BWP when there is no PRACH resource on the active BWP [1]. However, the current specification only allows the UE to switch to the initial BWP when there is no PRACH resource on the active BWP. In this contribution, we discuss the issue related to the UE autonomous BWP switching.

Discussion

Autonomous BWP switching

According to the current NR specification [1] as quoted below, if an active BWP is not configured with the PRACH resource, the UE would autonomously switch to the initial BWP. The reason for not configuring the PRACH resource on an active BWP is that the PRACH resource which is only temporarily used for the RACH procedure would require lots of PRBs reserved for each active BWP. And if the UE is configured with 4 BWP(s), then each active BWP would require the reserved PRACH resources.

Upon initiation of the Random Access procedure on a Serving Cell, after the selection of carrier for performing Random Access procedure as specified in clause 5.1.1, the MAC entity shall for the selected carrier of this Serving Cell: 1> if PRACH occasions are not configured for the active UL BWP: 2> switch the active UL BWP to BWP indicated by initialUplinkBWP; 2> if the Serving Cell is a SpCell: 3> switch the active DL BWP to BWP indicated by initialDownlinkBWP.

For NR-U, if lots of UE(s) autonomously switch to the initial BWP for the RACH procedure and the initial BWP is temporarily congested, then the autonomous switching of the BWP would cause the target BWP even more congested. To achieve a better balance on the PRACH resource allocation and the avoidance of congesting the initial BWP, we consider that if the UE has the RSSI/CO measurement result of the initial BWP, the UE should decide whether to autonomously switch to the target BWP based on the RSSI/CO measurement result of the target BWP, and the UE should be allowed to switch to any BWP with available PRACH resource. Given that the serving cell measurement object would most likely be configured in the initial BWP.

Proposal 1: If PRACH resource is not configured in the active BWP, the UE is allowed to switch to any BWP configured with PRACH resource.

Proposal 2: The UE selects a BWP which is not congested when the UE switches its BWP due to PRACH resource not configured in the active BWP.

Conclusions

Based on the analysis given above, we have the following proposals：

Proposal 1: If PRACH resource is not configured in the active BWP, the UE is allowed to switch to any BWP configured with PRACH resource.

Proposal 2: The UE selects a BWP which is not congested when the UE switches its BWP due to PRACH resource not configured in the active BWP.

Reference

[1] 3GPP TS 38.321, “NR; Medium Access Control (MAC) protocol specification”