3GPP TSG-RAN WG2 Meeting #109-e	R2-200484

Electronic meeting, 24th February - 6th March, 2020

Agenda item:       6.1.5.3

Source: 		Qualcomm Incorporated, Samsung

Title: 	IAB access barring

Document for:	Discussion

# Introduction

RAN2 agreed in last meeting [1]:

SI broadcast, barring, initial access - Both support of IAB node(s) and the cell status for IAB node(s) is combined in a single IE, i.e. if the IE is present, the cell supports IABs and the cell is also considered as a candidate for IABs; if the IE is absent, the cell does not support IAB and/or the cell is barred for IAB. - This IE can be provided per PLMN. - The case that UEs are barred but IAB nodes are allowed to access shall be supported. FFS if this is supported by MIB: CellBarred (i.e. IAB MT ignores the MIB cellBarred when set) or SIB1: CellReservations (i.e. IAB MT ignores SIB cell reservations, or has an access identity that allow access) - No new Establishment Cause values in RRC Connection Setup are defined. - No new Re-establishment Cause values are defined.

This paper discusses how to support network access for IAB-nodes while UEs are barred.

# 2	Discussion

The restriction of network access to IAB-nodes may be necessary under the following conditions:

- The parent node should only support backhaul service.

- The IAB-node should use SA mode while UEs should use ENDC.

Further circumstances are possible.

It should be possible to independently bar IAB-nodes from network access.

Cell barring for UEs can be indicated via MIB. To accommodate the above conditions, cell barring could be restricted to UEs. IAB-nodes can be independently barred from network access via the IAB-support indicator agreed in last meeting.

Proposal 1: MIB-based cell barring should not apply to IAB-nodes. IAB-nodes network access should be controlled via the IAB-indicator in SIB1.

# 3	Conclusion

This paper discussed how to support network access for IAB-nodes while UEs are barred. The following proposal has been made:

Proposal 1: MIB-based cell barring should not apply to IAB-nodes. IAB-nodes network access should be controlled via the IAB-indicator in SIB1.

.

# 4	References

[1]	Chairman Report, 3GPP TSG RAN WG2, Meeting #107, Reno, USA, November 18th -22nd, 2019