3GPP TSG-RAN WG2 Meeting #109 electronic	R2-2000526

24th Feb – 6th Mar 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | xxxx | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of TS 37.340 on the support of MR-DC for IAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-02-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN2 and RAN3 have made the following agreements on the support of MR-DC for IAB. Agreements in RAN2#105bis meeting: - R2 assumes that the NR DC framework (e.g. MCG SCG related procedures) is used to configure dual radio links used as IAB bh links with two parent nodes. Agreements in RAN2#106 meeting: - In NR-DC framework for IAB nodes PDCP is not supported for BH RLC channels, so any PDCP related functions like “split bearer” is not supported. For routing etc BAP is used. - In Rel-16, the d’ option is supported. - For IAB node using EN-DC, from BAP and backhaul RLC channels point of view, this is a single link deployment (BAP route only by NR link). Agreements in RAN2#107 meeting: - Also the d’ can be supported by DC, by assigning the roles of MN and SN to the IAB nodes serving the outer leaf access IAB node. Agreements in RAN2#107bis meeting: - When NR DC is configured for the IAB-node, 2.1 RLF is detected separately for the MCG-link and for the SCG-link, and 2.2 existing UE procedures are used for MCG-link and SCG-link failure handling. - The following is agreed as working assumption: BH RLF recovery for DC case reuses UE’s MCG and SCG failure recovery procedures specified in Rel-16. - For DC case, the IAB-node considers the radio link is failed and uses RRC existing or Rel-16 mechanism (e.g. MCG or SCG failure report, RRC reestablishment) if “recovery failure” notification is received from parent nodes on MCG-link or/and SCG-link. - R2 assumes that RLF notification “recovery failure” would be triggered when RRC reestablishment has failed. - BAP layer is used to transmit BH RLF notification(s). Agreements in RAN2#108 meeting: - SRB2 is used for transport of all F1AP messages in EN-DC. - Extend LTE DL Information Transfer and UL Information Transfer RRC procedures for F1AP transport since they already use SRB2. - Container that carries F1AP message is carried directly in LTE RRC, i.e. there is no additional NR RRC container, assumes protocol stack of “option 1b”. Agreements in RAN3#106 meeting: - TP for NR-IAB BL CR for TS38.401 on NSA IAB Integration Procedure: Agreed in R3-197784. Those IAB specific features should be captured in the TS 37.340 for the MR-DC scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 1x: Add the IAB specific features supported in MR-DC Impact analysis Impacted 5G architecture options: EN-DC, NR-DC Impacted functionality: Stage 2 Inter-operability: - If the UE is implemented according to this CR but the network is not, there is no inter-operability issue foreseen. - If the network is implemented according to this CR but the UE is not, there is no inter-operability issue foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IAB is not supported in the MR-DC scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3, 1x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------1st Change -----------------------------------------------------

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

Child node: IAB-DU’s next hop neighbor node; the child node can be IAB-node or UE.

Downstream: Direction toward child node or UE in IAB-topology.

En-gNB: node providing NR user plane and control plane protocol terminations towards the UE, and acting as Secondary Node in EN-DC.

IAB-donor: gNB that provides network access to UEs via a network of backhaul and access links.

IAB-DU: IAB-node function that terminates the Uu interface to the child node using the procedures and behaviors specified for gNB-DU.

IAB-MT: IAB-node function that terminates the Uu interface to the parent node using the procedures and behaviors specified for UEs unless stated otherwise.

IAB-node: RAN node that supports NR access links to UEs and NR backhaul links to parent nodes and child nodes. The IAB-node does not support backhauling via LTE.

Master Cell Group:	in MR-DC, a group of serving cells associated with the Master Node, comprising of the SpCell (PCell) and optionally one or more SCells.

Master node: in MR-DC, the radio access node that provides the control plane connection to the core network. It may be a Master eNB (in EN-DC), a Master ng-eNB (in NGEN-DC) or a Master gNB (in NR-DC and NE-DC).

MCG bearer: in MR-DC, a radio bearer with an RLC bearer (or two RLC bearers, in case of CA packet duplication) only in the MCG.

MN terminated bearer: in MR-DC, a radio bearer for which PDCP is located in the MN.

MCG SRB: in MR-DC, a direct SRB between the MN and the UE.

Multi-Radio Dual Connectivity: Dual Connectivity between E-UTRA and NR nodes, or between two NR nodes.

Ng-eNB: as defined in TS 38.300 [3].

Parent node: IAB-MT’s next hop neighbor node; the parent node can be IAB-node or IAB-donor.

PCell: SpCell of a master cell group.

PSCell: SpCell of a secondary cell group.

RLC bearer: RLC and MAC logical channel configuration of a radio bearer in one cell group.

Secondary Cell Group: in MR-DC, a group of serving cells associated with the Secondary Node, comprising of the SpCell (PSCell) and optionally one or more SCells.

Secondary node: in MR-DC, the radio access node, with no control plane connection to the core network, providing additional resources to the UE. It may be an en-gNB (in EN-DC), a Secondary ng-eNB (in NE-DC) or a Secondary gNB (in NR-DC and NGEN-DC).

SCG bearer: in MR-DC, a radio bearer with an RLC bearer (or two RLC bearers, in case of CA packet duplication) only in the SCG.

SN terminated bearer: in MR-DC, a radio bearer for which PDCP is located in the SN.

SpCell: primary cell of a master or secondary cell group.

SRB3: in EN-DC, NGEN-DC and NR-DC, a direct SRB between the SN and the UE.

Split bearer: in MR-DC, a radio bearer with RLC bearers both in MCG and SCG.

Split PDU Session (or PDU Session split): a PDU Session whose QoS Flows are served by more than one SDAP entities in the NG-RAN.

Split SRB: in MR-DC, a SRB between the MN and the UE with RLC bearers both in MCG and SCG.

User plane resource configuration: in MR-DC with 5GC, encompasses radio network resources and radio access resources related to either one or more PDU sessions, one or more QoS flows, one or more DRBs, or any combination thereof.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

BH	Backhaul

DC	Intra-E-UTRA Dual Connectivity

EN-DC	E-UTRA-NR Dual Connectivity

IAB	Integrated Access and Backhaul

MCG	Master Cell Group

MN	Master Node

MR-DC	Multi-Radio Dual Connectivity

NE-DC	NR-E-UTRA Dual Connectivity

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NR-DC	NR-NR Dual Connectivity

SCG	Secondary Cell Group

SMTC	SS/PBCH block Measurement Timing Configuration

SN	Secondary Node

------------------------------------------------2st Change -----------------------------------------------------

# 13			Other aspects

## 13.1	Interference avoidance for in-device coexistence

In-Device Coexistence (IDC) solution as described in TS 36.300 [2] is extended to address EN-DC operation. Only FDM solution, where the list of NR carriers suffering from IDC problems is signalled in IDC indication, is supported in this version of the specifications. The requirement on RRM/RLM/CSI measurements in different phases of IDC interference defined in TS 36.300 [2] is applicable except that for NR serving cell, the requirements in TS 38.133 [8] and TS 38.101-1 [12], TS 38.101-2 [13], TS 38.101-3 [14] apply.

# 1x			Integrated Access and Backhaul

### 1x.1		Architecture

### 1x.1.1	E-UTRA-NR Dual Connectivity

The IAB-node can access the network using either SA-mode (option a) or EN-DC (option b). For the IAB-node using EN-DC, it also connects via E-UTRA to a MeNB, and the IAB-donor terminates X2 interface as SgNB.

From BAP and backhaul RLC channels point of view, BAP sub-layer routing only uses NR Uu link.

Figure 1x.1.1-1: EN-DC architecture; a) UE operates in EN-DC while IAB-node operates in SA with NGC, b) UE and IAB-node operates in EN-DC.

### 1x.1.2	NR-NR Dual Connectivity

The NR-DC framework (e.g. MCG and SCG related procedures) is used to configure dual radio links used as IAB backhaul links with two parent nodes, where there is single IAB-donor but two parent IAB-nodes.

Figure 1x.1.2-1: NR-DC architecture with single IAB-donor but two parent IAB-nodes

### 1x.2		IAB integration procedure for EN-DC

The IAB integration procedure for EN-DC is shown in Figure 1x.2-1.

Figure 1x.2-1: Signaling flow for IAB integration procedure in EN-DC

Phase 1-1:

IAB-MT part setup with E-UTRAN. In this phase, the IAB-MT part connects to the LTE network as a normal UE, by performing RRC connection setup procedure with an eNB, authentication with the EPC, IAB-node’s access traffic-related radio bearer configuration at the E-UTRAN side, and optionally, OAM connectivity establishment by using the IAB-MT’s PDN connection. The IAB node can select the IAB-supporting eNB based on an over-the-air indication from eNB (transmitted in SIB). To indicate its IAB capability, the IAB-MT includes the IAB-node indication in RRCConnectionSetupComplete message, to assist the eNB select the MME supporting IAB. The eNB then configures the IAB-MT part with an NR measurement configuration in order to perform discovery, measurement, and measurement reporting of candidate gNBs. To enable the eNB choose an SgNB which supports IAB function, the IAB capability of neighbor gNBs can be pre-configured in the eNB (e.g. by OAM).

Editor’s Note: Other ways to enable the eNB know the IAB capability of neighbour gNBs are not precluded.

Phase 1-2:

SgNB Addition. In this phase, the IAB-MT part connects to the parent DU and IAB-donor CU via the EN-DC SgNB addition procedure. The procedure defined in section 8.4.1 is reused. The eNB includes “IAB node indication” in SGNB ADDITION REQUEST message to inform the IAB-donor CU that the request is for an IAB-node. In addition, SRB3 can be setup for the IAB-MT, to transmit RRC message between the IAB-MT and the IAB-donor CU via the NR links directly.

Phase 2-1:

Backhaul RLC channel establishment. This phase is the same as the phase in the Standalone IAB integration procedure.

Phase 2-2:

Routing update. This phase is the same as the phase in the Standalone IAB integration procedure.

Phase 3:

IAB-DU part setup. This phase is the same as the phase in the Standalone IAB integration procedure.

### 1x.3		F1AP transport in EN-DC

In EN-DC, F1AP message between IAB-donor and IAB-node can be transmitted over LTE to enhance the robust, when the NR link is not available.

In this case, as shown below, container that carries F1AP message is carried directly in LTE RRC, i.e. there is no additional NR RRC container, and SRB2 is used for transport of all F1AP message over LTE Uu interface. DL Information Transfer and UL Information Transfer procedures in LTE RRC are extended for F1AP transport.

Figure 1x.3-1: Example protocol stack of F1AP transport in EN-DC