3GPP TSG-RAN WG2 Meeting #109e	 R2-2000545

# 24 Feb – 6 Mar 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | - | rev | - | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.331 for HARQ-Based RLF at TX UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2020-01-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To capture TP to 38.331 for HARQ-based RLF at TX UE, based on email discussion: [108#99][V2X] HARQ based TX side RLM/RLF |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture proposals in email discussion: [108#99][V2X] HARQ based TX side RLM/RLF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | HARQ-based RLF at TX UE is missing in stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.321 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st change

#### 5.X.9.3	Sidelink radio link failure related actions

The UE shall:

1>	upon indication from sidelink RLC entity that the maximum number of retransmissions for a specific destination has been reached or:

1>	upon indication from sidelink MAC entity that the maximum number of HARQ DTX for a specific destination has been reached:

2>	consider sidelink radio link failure to be detected for this destination;

2>	release the DRBs of this destination, in according to sub-clause 5.X.9.1.4;

2>	release the SRBs of this destination, in according to sub-clause 5.X.9.1.7;

2>	discard the NR sidelink communication related configuration of this destination;

2>	consider the PC5-RRC connection is released for the destination;

2>	indicate the release of the PC5-RRC connection to the upper layers  for this destination (i.e. PC5 is unavailable);

2>	if UE is in RRC_CONNECTED:

3>	perform the sidelink UE information for NR sidelink communication procedure, as specified in 5.X.3.3 or sub-clause 5.10.X in TS 36.331 [10];

Next change

#### –	SIBX

SIBX contains NR sidelink communication configuration.

SIBX information element

-- ASN1START

-- TAG-SIBX-START

SIBX-r16 ::=                            SEQUENCE {

sl-ConfigCommonNR-r16                  SL-ConfigCommonNR-r16,

lateNonCriticalExtension               OCTET STRING                          OPTIONAL,

...

}

SL-ConfigCommonNR-r16 ::=                 SEQUENCE {

sl-FreqInfoList-r16                       SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16      OPTIONAL,    -- Need R

sl-UE-SelectedConfig-r16                  SL-UE-SelectedConfig-r16                                               OPTIONAL,    -- Need R

sl-NR-AnchorCarrierFreqList-r16           SL-NR-AnchorCarrierFreqList-r16                                        OPTIONAL,    -- Need R

sl-EUTRA-AnchorCarrierFreqList-r16        SL-EUTRA-AnchorCarrierFreqList-r16                                     OPTIONAL,    -- Need R

sl-RadioBearerConfigList-r16              SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SL-RadioBearerConfig-r16       OPTIONAL,    -- Need R

sl-RLC-BearerConfigList-r16               SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16         OPTIONAL,    -- Need R

sl-MeasConfigCommon-r16                   SL-MeasConfigCommon-r16                                                OPTIONAL,    -- Need R

sl-CSI-Acquisition-r16                    ENUMERATED {enabled}                                                   OPTIONAL,    -- Need R

sl-ZoneConfig-r16                         SL-ZoneConfig-r16                                                      OPTIONAL,    -- Need R

sl-OffsetDFN-r16                          INTEGER (0..1000)                                                      OPTIONAL,    -- Need R

sl-maxNumConsecutiveDTX					ENUMERATED {t1, t2, t3, t4, t6, t8, t16, t32}	OPTIONAL,		-- Need R

t400                                      ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL,    -- Need R

...

}

SL-NR-AnchorCarrierFreqList-r16 ::=           SEQUENCE (SIZE (1..maxFreqSL-NR-r16)) OF ARFCN-ValueNR

SL-EUTRA-AnchorCarrierFreqList-r16 ::=        SEQUENCE (SIZE (1..maxFreqSL-EUTRA-r16)) OF ARFCN-ValueEUTRA

-- TAG-SIBX-STOP

-- ASN1STOP

SIBX field descriptions

sl-CSI-Acquisition This field indicates whether CSI reporting is enabled in sidelink unicast. If not set, SL CSI reporting is disabled.

sl-EUTRA-AnchorCarrierFreqList This field indicates the EUTRA anchor carrier frequency list, which can provide the NR sidelink communication configurations.

sl-FreqInfoList This field indicates the NR sidelink communication configuration on some carrier frequency (ies). In this release, only one entry can be configured in the list.

sl-MeasConfigCommon This field indicates the measurement configurations (e.g. RSRP) for NR sidelink communication.

sl-NR-AnchorCarrierFreqList This field indicates the NR anchor carrier frequency list, which can provide the NR sidelink communication configurations.

sl-OffsetDFN Indicates the timing offset for the UE to determine DFN timing when GNSS is used for timing reference. Value 0 corresponds to 0 milliseconds, value 1 corresponds to 0.001 milliseconds, value 2 corresponds to 0.002 milliseconds, and so on.

sl-RadioBearerConfigList This field indicates one or multiple sidelink radio bearer configurations.

sl-RLC-BearerConfigList This field indicates one or multiple sidelink RLC bearer configurations.

sl-maxNumConsecutiveDTX This field indicates the maximum number of consecutive HARQ DTX before triggering sidelink RLF. Value t1 corresponds to 1, value t2 corresponds to 2, and so on.

Next change

#### SL-ConfigDedicatedNR

The IE SL-ConfigDedicatedNR specifies the dedicated configuration information for NR sidelink communication.

SL-ConfigDedicatedNR information element

-- ASN1START

-- TAG-SL-CONFIGDEDICATEDNR-START

SL-ConfigDedicatedNR-r16 ::=                 SEQUENCE {

sl-ScheduledConfig-r16                       SetupRelease { SL-ScheduledConfig-r16 }                                    OPTIONAL,    -- Need M

sl-UE-SelectedConfig-r16                     SetupRelease { SL-UE-SelectedConfig-r16 }                                  OPTIONAL,    -- Need M

sl-FreqInfoToReleaseList-r16                 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF ARFCN-ValueNR                    OPTIONAL,    -- Need N

sl-FreqInfoToAddModList-r16                  SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfig-r16                OPTIONAL,    -- Need N

-- Editor’s Note: FFS on whether both mode-1 and mode-2 can be both configured.

sl-RadioBearerToReleaseList-r16              SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SLRB-Uu-ConfigIndex-r16            OPTIONAL,    -- Need N

sl-RadioBearerToAddModList-r16               SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SL-RadioBearerConfig-r16           OPTIONAL,    -- Need N

sl-RLC-BearerToReleaseList-r16               SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfigIndex-r16        OPTIONAL,    -- Need N

sl-RLC-BearerToAddModList-r16                SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16             OPTIONAL,    -- Need N

sl-MeasConfigInfoToReleaseList-r16           SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-DestinationIndex-r16         OPTIONAL,    -- Need N

sl-MeasConfigInfoToAddModList-r16            SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-MeasConfigInfo-r16           OPTIONAL,    -- Need M

sl-maxNumConsecutiveDTX						 ENUMERATED {t1, t2, t3, t4, t6, t8, t16, t32}									 		OPTIONAL,	 -- Need R

t400-r16                                     ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000}     OPTIONAL,    -- Need M

sl-CSI-Acquisition-r16                       ENUMERATED {enabled}                                                       OPTIONAL,    -- Need N

sl-SSB-PriorityNR-r16                        INTEGER (1..8)                                                             OPTIONAL,    -- Need N

sl-PUCCH-Config-r16                          PUCCH-Config                                                               OPTIONAL,    -- Need N

sl-PDCCH-Config-r16                          PDCCH-Config                                                               OPTIONAL,    -- Need N

networkControlledSyncTx-r16                  ENUMERATED {on, off}                                                       OPTIONAL,    -- Need N

...

}

SL-DestinationIndex-r16  ::=                     INTEGER (0..maxNrofSL-Dest-1-r16)

-- TAG-SL-CONFIGDEDICATEDNR-STOP

-- ASN1STOP

Next change

#### –	SL-PreconfigurationNR

The IE SL-PreconfigurationNR includes the sidelink pre-configured parameters used for NR sidelink communication.

SL-PreconfigurationNR information elements

-- ASN1START

-- TAG-SL-PRECONFIGURATIONNR-START

SL-PreconfigurationNR-r16 ::=             SEQUENCE {

sidelinkPreconfigNR-r16                  SidelinkPreconfigNR-r16,

...

}

SidelinkPreconfigNR-r16 ::=                    SEQUENCE {

sl-PreconfigFreqInfoList-r16                   SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16        OPTIONAL,    -- Need R

sl-PreconfigNR-AnchorCarrierFreqList-r16       SL-NR-AnchorCarrierFreqList-r16                                          OPTIONAL,    -- Need R

sl-PreconfigEUTRA-AnchorCarrierFreqList-r16    SL-EUTRA-AnchorCarrierFreqList-r16                                       OPTIONAL,    -- Need R

sl-RadioBearerPreConfigList-r16                SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SL-RadioBearerConfig-r16         OPTIONAL,    -- Need R

sl-RLC-BearerPreConfigList-r16                 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16           OPTIONAL,    -- Need R

sl-MeasPreConfig-r16                           SL-MeasConfigCommon-r16                                                  OPTIONAL,    -- Need R

sl-OffsetDFN-r16                               INTEGER (0..1000)                                                        OPTIONAL,    -- Need R

sl-maxNumConsecutiveDTX						   ENUMERATED { t1, t2, t3, t4, t6, t8, t16, t32 }								   OPTIONAL,	 -- Need R

t400-r16                                       ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000}   OPTIONAL,    -- Need R

sl-SSB-PriorityNR-r16                          INTEGER (1..8)                                                             OPTIONAL,    -- Need R

sl-PreconfigGeneral-r16                        SL-PreconfigGeneral-r16                                                  OPTIONAL,    -- Need R

sl-UE-SelectedPreConfig-r16                    SL-UE-SelectedConfig-r16                                                 OPTIONAL,    -- Need R

...

}

SL-PreconfigGeneral-r16 ::=                    SEQUENCE {

sl-TDD-Config-r16                              TDD-UL-DL-ConfigCommon                                                   OPTIONAL,    -- Need R

reservedBits-r16                               BIT STRING (SIZE (2))                                                    OPTIONAL,    -- Need R

...

}

END

-- TAG-SL-PRECONFIGURATIONNR-STOP

-- ASN1STOP

Change End