3GPP TSG-RAN WG2 Meeting #109-e                                              	　　　　　　　　    　　R2-2000661

Online, February 24th – March 6th 2020

Source: 	KDDI Corporation

Title:	Considerations on BAP entity release

Tdoc Type:	Discussion

Agenda Item:	6.1.3 BAP functionality

Document for:	Discussion and decision

Release:	Rel-16

Introduction

In this contribution we will share our views on the IAB node behaviour on BAP entity release. We also propose the text change related to BAP entity release in TS38.340.

Discussion

Figure 1 depicts how the backhaul channel is setup using RRC signaling and F1-AP signaling. For backhaul setup, the following for steps are assumed.

- The IAB-MT receives RRC Reconfiguration including BAP entity setup and BH RLC CH list to be setup according to TS38.331 [1][2]

- The IAB-DU initiates F1 setup procedure according to TS38.473 [3]

- The IAB-DU receives UE CONTEXT SETUP and BH ROUTING CONFIGURATION including BH RLC CH list and BAP routing information to be setup according to TS38.473 [3]

Figure 1:  TS38.331 and TS38.473 procedures for BH setup/release

For the backhaul release, the following steps are assumed.

- The IAB-DU receives UE CONTEXT SETUP and BH ROUTING CONFIGURATION including BH RLC CH list and BAP routing information to be removed. [3]

- The IAB-DU initiates F1 removal procedure according to TS38.473. [3]

- The IAB-MT receives RRC Reconfiguration including BAP entity release and BH RLC CH list to be released according to TS38.331. [1][2]

Considering the case where the BAP routing information removal and the F1 removal procedure cannot be executed because of some radio conditions including radio link failure, those removal procedures should be initiated at the timing when the BAP entity release is indicated.

- 8.2.8 F1 Removal - 8.2.8.1 General The purpose of the F1 Removal procedure is to remove the interface instance and all related resources between the gNB-DU and the gNB-CU in a controlled manner. If successful, this procedure erases any existing application level configuration data in the two nodes. NOTE: In case the signalling transport is shared among several F1-C interface instances, and the TNL association is still used by one or several F1-C interface instances, the initiating node should not initiate the removal of the TNL association. The procedure uses non-UE-associated signaling.

Excerpt 1:  F1 removal definition in TS38.473

We believe that the clear statements for the removal of the F1 association and the stored BH routing information is needed and we propose the following.

Proposal: 	RAN2 agree to have the following sentences for F1 removal and BH information removal in TS38.340.

- 5.1.2 BAP entity release When upper layers request a BAP entity release, the node shall: - remove all the BH routing information stored in the IAB-DU. - remove the interface instance and all related resources between the gNB-DU (the IAB-DU) and the gNB-CU in a controlled manner - erase any existing F1 application level configuration data in gNB-DU (the IAB-DU) and the gNB-CU. - release the BAP entity.

Conclusion

Proposal: 	RAN2 agree to have the following sentences for F1 removal and BH information removal in TS38.340.

- 5.1.2 BAP entity release When upper layers request a BAP entity release, the node shall: - remove all the BH routing information stored in the IAB-DU. - remove the interface instance and all related resources between the gNB-DU (the IAB-DU) and the gNB-CU in a controlled manner - erase any existing F1 application level configuration data in gNB-DU (the IAB-DU) and the gNB-CU. - release the BAP entity.

References

[1] R2-1916636		Draft38340-020_v5.0					Huawei

[2] R2-xxxxxxx		Running CR to 38.331 on Integrated Access and Backhaul for NR	Ericsson

[3] R3-200025		BL CR to 38.473: Support for IAB			Ericsson