3GPP TSG-RAN WG2 #109-e	 TDoc R2-2000945

# Electronic Meeting, 24th February – 6th March 2020

Agenda Item:	6.4.3.2

Source:	Ericsson

Title:	On PDCP re-establishment

Document for:	Discussion, Decision

# Introduction

In the last RAN2#108 meeting, the following agreements were made regarding PDCP:

Agreements on PDCP: 1: For SL groupcast and broadcast, PDCP out-of-order delivery is not supported. 2: ARP SDU type is not supported in NR sidelink. 3: PC5 Signaling protocol SDU type is not supported in NR sidelink. 4: The length of SDU type is 2bits. 5: For data PDU format, D/C field is not used for groupcast and broadcast. 6: The initial value of RX_DELIV in the receiving PDCP entity sets to 0 for unicast. The initial value of RX_DELIV in the receiving PDCP entity follows the LTE rule for groupcast and broadcast. 7: PDCP should support AS ciphering and integrity protection for SL data and PC5-RRC. 8: For SL groupcast and broadcast, only 18bits pdcp-SN-Size is used for the SL DRB. For SL unicast, 12bits and 18bits pdcp-SN-Size are used for the SL DRB. 9: For SL unicast, only 12bits pdcp-SN-Size is used for the SL SRB including SL SRBs which carrying PC5 Signalling and PC5 RRC message. FFS on how to handle initial PC5-S signaling for unicast link.

However, it has not been discussed if we support PDCP re-establishment and status report in Rel-16, which is discussed in this paper.

# Discussion

## PDCP re-establishment and status report

In NR Uu, during handover procedure, the existing PDCP will be re-established according to configuration form the new cell and PDCP status report will be sent to the new cell so that NW can understand what PDCP PDUs are in the list waiting for transmission. However, in SL, such scenario does not exist that a UE is switching from one UE to anther UE, thus there is no need to consider PDCP re-establishment and status report transmission.

NR sidelink does not consider PDCP re-establishment and status report.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	NR sidelink does not consider PDCP re-establishment and status report.

# References