3GPP TSG-RAN WG2 #109-e	Tdoc R2-2000980

Electronic meeting, 24 February – 6 March 2020	Resubmission of R2-1913117

Agenda Item:	7.1.9

Source:	Ericsson

Title:	Cell Reselection improvement for LTE-M Standalone cells

Document for:	Discussion, Decision

# 1	Introduction

The Rel-16 WI on “Additional MTC enhancements for LTE” [1] has the following objectives on stand-alone deployment for LTE-MTC:

Stand-alone deployment: - Enable the use of LTE control channel region for DL transmission (MPDCCH/PDSCH) to BL/CE UEs [RAN1, RAN2, RAN4] - This deployment mode should support legacy operation for legacy BL/CE UEs.

In RAN1#94bis it is clarified that the enhancements introduced by the WI objective on usage of the LTE DL control channel region for MPDCCH/PDSCH transmissions to LTE-MTC UEs do not only apply to LTE-MTC stand-alone deployments but also to the case when LTE-MTC is deployed within an NR carrier.

In the previous meetings concern has been raised for non-BL UEs camping in a standalone cell in enhanced coverage when not needed. A UE may end up camping in the standalone cell, due to higher ranking compared to the neighbouring cells, even though it would be possible for the UE to fulfil the S-criterion for normal coverage in a neighbour non-standalone cell. If the UE could camp in the non-standalone cell, it could use more advanced features such as broadcast/multicast in idle mode or in general transmissions with better energy efficiency and spectral efficiency which contributes to improve network capacity. In this contribution we propose a mechanism to improve the coverage mode selection in order to maximise UE and network performance.

# 2	Discussion

This scenario is described in Figure 1. In the graph the ranks used for the cell reselection algorithm are shown (Rs in green, Rn in blue) as a function of UE position. It is assumed the UE is camping in the standalone cell (left) while performing the cell reselection procedure.

In region 1 (left) the rank of the camped cell, i.e. standalone cell, is higher than the one for neighboring cell so the UE does not reselect the neighbor cell. Assuming that in region 1 the UE would be in enhanced coverage if it were to perform cell reselection to the neighbor cell, and thus current mechanism for ranking would be sufficient.

In region 2 (center) the rank of the camped cell, i.e. standalone cell, is still higher than the one for neighboring cell so the UE does not perform cell reselection. Nevertheless, the UE may benefit from performing cell reselection to the neighboring cell if it would be in normal coverage even though the UE is required to be camped in the standalone cell based on the ranking mechanism.

In region 3 (right) the rank of the neighboring cell becomes higher than the one of the camped cell, so the UE performs cell reselection.

Figure 1: Representation of a scenario where a non-BL UE may camp in a standalone cell with suboptimal performance

In some cases, the cell reselection procedure for BL UEs or UEs in CE let a non-BL UE to camp in a standalone cell even though it would be possible for such UE to camp in a neighbouring non-standalone cell in “normal” coverage.

We propose to modify the ranking criterion for the neighbour cell as follows (a detailed text proposal can be found in Section 2.1).

A new configurable offset QoffsetnonBL is introduced and signalled in SIB4 and SIB5. Other offsets are present in the calculation of the rank, but in order to not affect existing functions we prefer to use a new parameter instead of changing the value of existing ones.

This offset is added or subtracted to the legacy Rn if the UE is respectively in the standalone cell and it could camp in the neighbour non-standalone cell in normal coverage or if the UE is in normal coverage and the neighbour cell is standalone.

A new offset is introduced in the calculation of the neighbour cell rank when the UE is in a standalone cell and could camp in the neighbour non-standalone cell in normal coverage or vice-versa

Figure 2 shows the resulting ranks in the same scenario. With a proper configuration of QoffsetnonBL it is possible to completely avoid non-BL UEs working in suboptimal conditions, or it is possible to tune the probability of a non-BL UE moving to a more suitable cell. An offset equal to 0 is equivalent to use the legacy cell reselection procedure.

Figure 2: Representation of the aforementioned scenario with the new calculation of the rank when the UE is camping in the standalone cell (a) or when it is camping in a normal cell (b)

## 2.1 	Text Proposal (36.304)

CHANGE START

#### 5.2.4.6	Intra-frequency and equal priority inter-frequency Cell Reselection criteria

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

Rs= Qmeas,s+QHyst- Qoffsettemp+QoffsetSCPTM

If the UE is non-BL in CE and the criterion S for normal coverage is satisfied for a neighbouring cell, its cell-ranking criterion Rn is defined by:

Rn= Qmeas,n-Qoffset- Qoffsettemp+QoffsetSCPTM+QoffsetnonBL

If the UE is non-BL capable of CE in normal coverage and the criterion S for CE or CE1 is satisfied for a neighbouring cell, its cell-ranking criterion Rn is defined by:

Rn= Qmeas,n-Qoffset- Qoffsettemp+QoffsetSCPTM-QoffsetnonBL

Else, the neighbouring cell-ranking criterion Rn is defined by:

Rn= Qmeas,n-Qoffset- Qoffsettemp+QoffsetSCPTM

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Except for NB-IoT, equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. For NB-IoT equals to QoffsetDedicatedfrequency for any frequency other than the frequency of the dedicated frequency offset, if QoffsetDedicatedfrequency is valid, otherwise this equals to Qoffsetfrequency (if QoffsetDedicatedfrequency is valid Qoffsetfrequency is not used). |
| Qoffsettemp | Offset temporarily applied to a cell as specified in TS 36.331 [3] |
| QoffsetSCPTM | Offset temporarily applied to an SC-PTM frequency as specified below. The offset is applied to all cells on the SC-PTM frequency. If QoffsetSCPTM is valid, Qoffset for inter-frequency neighbour cells is not used. |
| QoffsetnonBL | Offset applied to a cell by non-BL UEs capable of CE when selecting to or from a cell which broadcasts SIB1-BR but not SIB1. |

If the NB-IoT UE or UE in enhanced coverage is capable of SC-PTM reception and is receiving or interested to receive an MBMS service and can only receive this MBMS service while camping on a frequency on which it is provided (SC-PTM frequency), the UE considers QoffsetSCPTM to be valid during the MBMS session TS 36.300 [2] as long as the following condition is fulfilled:

Either:

-	SIB15 (or SIB15-NB) of the serving cell indicates for that frequency one or more MBMS SAIs included in the MBMS User Service Description (USD) TS 26.346 [22] of this service; or

-	SIB15 (or SIB15-NB) is not broadcast in the serving cell and that frequency is included in the USD of this service.

NOTE:	UE should search for a higher ranked cell on another frequency for cell reselection as soon as possible after the UE stops using QoffsetSCPTM.

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2 (5.2.3.2a for NB-IoT), but may exclude all CSG cells that are known by the UE not to be CSG member cells.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell reselection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

When the UE uses infinite dBs for QoffsetSCPTM, the UE shall use QoffsetSCPTM zero and rank the cells on the SC-PTM frequency(ies) only first. If the UE cannot find a suitable cell on an SC-PTM frequency, the UE shall rank the cells on all frequencies.

#### 5.2.4.6a	Reselection for enhanced coverage

Ranking as defined in sub-clause 5.2.4.6 is applied for intra-frequency and inter-frequency cell reselection (irrespective of configured frequency priorities, if any) while the UE is in enhanced coverage.

#### 5.2.4.7	Cell reselection parameters in system information broadcasts

Cell reselection parameters are broadcast in system information and are read from the serving cell as follows:

cellReselectionPriority

This specifies the absolute priority for E-UTRAN frequency or NR frequency or UTRAN frequency or group of GERAN frequencies or band class of CDMA2000 HRPD or band class of CDMA2000 1xRTT.

cellReselectionSubPriority

This specifies the fractional priority value added to cellReselectionPriority for E-UTRAN frequency or NR frequency.

Poffset

This specifies the offset for 14 dBm power class for BL or NB-IoT UE.

Qoffsetauthorization

This specifies the offset for enhanced coverage authorization for NB-IoT.

Qoffsets,n

This specifies the offset between the two cells.

Qoffsetfrequency

Frequency specific offset for equal priority E-UTRAN frequencies.

QoffsetnonBL

This specifies the offset to be used for cell selection and re-selection by non-BL UEs when selecting to or from a cell which broadcasts SIB1-BR but not SIB1.

Qoffsetscptm

This specifies the offset to be used for cell re-selection for SC-PTM service reception for BL UE, UE in enhanced coverage and NB-IoT UE. The same offset is applicable to all frequencies providing MBMS services via SC-PTM.

Qoffsettemp

This specifies the additional offset to be used for cell selection and re-selection. It is temporarily used in case the T300 expires consecutively on the cell as specified in TS 36.331 [3].

Qhyst

This specifies the hysteresis value for ranking criteria.

Qqualmin

This specifies the minimum required quality level in the cell in dB.

Qqualmin_CE, Qqualmin_CE1

This specifies the coverage specific minimum required quality level in the cell in dB.

Qrxlevmin

This specifies the minimum required Rx level in the cell in dBm.

Qrxlevmin_CE, Qrxlevmin_CE1

This specifies the coverage specific minimum required Rx level in the cell in dBm.

RedistributionFactorFreq

This specifies the redistribution factor for a neighbour E-UTRAN frequency.

RedistributionFactorCell

This specifies the redistribution factor for a neighbour E-UTRAN cell.

RedistributionFactorServing

This specifies the redistribution factor for serving cell or serving frequency.

TreselectionRAT

This specifies the cell reselection timer value. For each target E-UTRA frequency and for each RAT (other than E-UTRA) a specific value for the cell reselection timer is defined, which is applicable when evaluating reselection within E-UTRAN or towards other RAT (i.e. TreselectionRAT for E-UTRAN is TreselectionEUTRA, for NR TreselectionNR, for UTRAN TreselectionUTRA for GERAN TreselectionGERA, for TreselectionCDMA_HRPD, and for TreselectionCDMA_1xRTT). For NB-IoT intra-frequency and inter-frequency specific values for the cell reselection timer are defined, which are applicable when evaluating reselection within NB-IoT.

NOTE:	TreselectionRAT is not sent on system information, but used in reselection rules by the UE for each RAT.

TreselectionEUTRA_ CE

This specifies the cell reselection timer value TreselectionRAT for E-UTRAN when a neighbour cell is evaluated for camping in enhanced coverage. The parameter can be set per E-UTRAN frequency.

TreselectionEUTRA

This specifies the cell reselection timer value TreselectionRAT for E-UTRAN. The parameter can be set per E-UTRAN frequency TS 36.331 [3].

TreselectionNR

This specifies the cell reselection timer value TreselectionRAT for NR.

TreselectionNB-IoT_Intra

This specifies the intra-frequency cell reselection timer value TreselectionRAT for NB-IoT.TreselectionNB-IoT_Inter

This specifies the inter-frequency cell reselection timer value TreselectionRAT for NB-IoT.

TreselectionUTRA

This specifies the cell reselection timer value TreselectionRAT for UTRAN.

TreselectionGERA

This specifies the cell reselection timer value TreselectionRAT for GERAN.

TreselectionCDMA_HRPD

This specifies the cell reselection timer value TreselectionRAT for CDMA HRPD.

TreselectionCDMA_1xRTT

This specifies the cell reselection timer value TreselectionRAT for CDMA 1xRTT.

ThreshX, HighP

This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a higher priority RAT/ frequency than the current serving frequency. Each frequency of E-UTRAN, NR and UTRAN, each group of GERAN frequencies, each band class of CDMA2000 HRPD and CDMA2000 1xRTT might have a specific threshold.

ThreshX, HighQ

This specifies the Squal threshold (in dB) used by the UE when reselecting towards a higher priority RAT/ frequency than the current serving frequency. Each frequency of E-UTRAN, NR and UTRAN FDD might have a specific threshold.

ThreshX, LowP

This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a lower priority RAT/ frequency than the current serving frequency. Each frequency of E-UTRAN, NR and UTRAN, each group of GERAN frequencies, each band class of CDMA2000 HRPD and CDMA2000 1xRTT might have a specific threshold.

ThreshX, LowQ

This specifies the Squal threshold (in dB) used by the UE when reselecting towards a lower priority RAT/ frequency than the current serving frequency. Each frequency of E-UTRAN, NR and UTRAN FDD might have a specific threshold.

ThreshServing, LowP

This specifies the Srxlev threshold (in dB) used by the UE on the serving cell when reselecting towards a lower priority RAT/ frequency.

ThreshServing, LowQ

This specifies the Squal threshold (in dB) used by the UE on the serving cell when reselecting towards a lower priority RAT/ frequency.

SIntraSearchP

This specifies the Srxlev threshold (in dB) for intra-frequency measurements.

SIntraSearchQ

This specifies the Squal threshold (in dB) for intra-frequency measurements.

SnonIntraSearchP

This specifies the Srxlev threshold (in dB) for E-UTRAN inter-frequency and inter-RAT measurements.

SnonIntraSearchQ

This specifies the Squal threshold (in dB) for E-UTRAN inter-frequency and inter-RAT measurements.

SSearchDeltaP

This specifies the Srxlev delta threshold (in dB) during relaxed monitoring.

CHANGE END

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In some cases, the cell reselection procedure for BL UEs or UEs in CE let a non-BL UE to camp in a standalone cell even though it would be possible for such UE to camp in a neighbouring non-standalone cell in “normal” coverage.

Based on the discussion in the previous sections we propose the following:

Proposal 1	A new offset is introduced in the calculation of the neighbour cell rank when the UE is in a standalone cell and could camp in the neighbour non-standalone cell in normal coverage or vice-versa

# References

RP-192875, “Revised WID: Additional MTC enhancements for LTE”, Ericsson, RAN#86, Sitges, Spain, December 2019

R2-1916424, RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC, Reno, USA, November 2019

R1-1913594, RAN1 agreements for Rel-16 Additional MTC Enhancements for LTE, Reno, USA, November 2019

R2-1906441, Non-BL UE operation in standalone deployment, Intel, Reno, Nevada, US, May 2019

TS 36.331, Radio Resource Control (RRC) Protocol specification v15.4.0, December 2018

TS 36.304, User Equipment (UE) procedures in idle mode v15.5.0, January 2020