3GPP TSG-RAN WG1 Meeting #109-e	R2-2001862

Electronic meeting, February 24th – March 6th, 2020

Agenda Item:	7.1.5

Source:	Ericsson (Rapporteur)

Title:	Summary of contributions on scheduling multiple DL/UL transport blocks

Document for:	Discussion, Decision

# Introduction

This document provides a summary of proposals and topics discussed in [1] - [3] for Agenda Item 7.1.5.

The proposals are grouped per topic and suggestions for way forward as rapporteur proposals are added based on the company inputs.

# Discussion

## HARQ RTT timers for LTE-M

For HART RTT timers for LTE-M, the following proposal is submitted by Ericsson:

For multi-TB scheduling in LTE-M, the drx-InactivityTimer value is dynamically adjusted based on the number of TBs being scheduled by the DCI. [3] (Ericsson)

This is the remaining case with no agreement yet and there are no alternative views proposed, therefore suggestion is to agree the following:

For LTE-M the length of HARQ RTT timer is set to 7+k*N for bundled HARQ ACK, where k is equal to the number of HARQ ACK bundles.

## HARQ RTT timers for NB-IoT

Related to HARQ RTT timers for NB-IoT, the following proposal is brought up by Blackberry:

Remove the Editor’s note in the next update of [3], and update the document to clarify that associated agreements are for the non-interleaved case.[1] (Blackberry)

- The reference [3] in above proposals refers to RAN2 agreements document (R2-1916424). As all different cases for HARQ RTT timer for both LTE-M and NB-IoT should be addressed after discussion/agreement of Proposal 1 above, the suggestion is to agree above, i.e.:

Remove Editor's note from RAN2 agreements document and clarify which agreements related to HARQ RTT timer length are for non-interleaved NB-IoT case.

## Unicast configuration

Related to unicast configuration for NB-IoT, Huawei has provided the following proposal:

For unicast, multiple TBs scheduling is enabled separately for uplink and downlink. [2]

ul-MultiTB-Config, dl-MultiTB-Config and dl-HARQ-ACK-Bundling are introduced in MultiTB-Config-NB. [2] (Huawei)

- Additionally, both Huawei in [2] and Ericsson in [3] have detailed text proposals related to above.

- The following are proposed as way forward and details for CRs are to be discussed over email:

For unicast, multiple TBs scheduling is enabled separately for uplink and downlink for both LTE-M and NB-IoT.

Discuss over email the details of unicast configuration for scheduling multiple TBs for 1) NB-IoT and 2) LTE-M.

## Multicast/SC-PtM configuration

The following proposals related to multicast configuration are proposed by Huawei and Ericsson:

Explicit enabling indication for multiple TBs scheduling in SC-PTM is not needed. [2] (Huawei)

multiTB-Gap is introduced in SC-MTCH-Info-NB-r14 to indicate the scheduling gap for each SC-MTCH configured with multiple TBs scheduling. [2] (Huawei)

Scheduling gaps for multi-TB scheduling with multicast are configured in SIB20 for LTE-M.[3] (Ericsson)

Scheduling gaps for multi-TB scheduling with multicast are configured in SIB20-NB for NB-IoT.[3] (Ericsson)

- Additionally, both Huawei and Ericsson documents include text proposals related to above proposals.

- As summary, neither company seems to indicate need for explicit indication for multi-TB scheduling in SC-PTM, thus it should be straightforward to agree:

Explicit enabling indication for multiple TBs scheduling in SC-PTM is not needed.

For scheduling gaps, one company seems to prefer configuration in SC-MTCH and other in SIB20(-NB). Therefore, the following should be discussed further together with the exact details of the configuration(s):

Discuss over email whether scheduling gap configuration is in SC-MTCH or in SIB20(-NB) for 1) NB-IoT and 2) LTE-M with details of configuration to be captured.

## Capabilities for NB-IoT

Huawei has proposed the following related to UE capability reporting for NB-IoT:

For NB-IoT, support of multiTB-UL-r16 and multiTB-DL-r16 is conditional on support of two HARQ processes. [2] (Huawei)

For NB-IoT, introduce separate capabilities multiTB-UL-Interleaving-r16, and multiTB-DL-Interleaving-r16 conditional to support of multiTB-UL-r16 and multiTB-DL-r16 respectively. [2] (Huawei)

For NB-IoT, introduce a new capability multiTB-HARQ-ACK-Bundling-r16, conditional to support of multiTB-DL-Interleaving-r16. [2] (Huawei)

Support of multiple TBs scheduling in multicast is optional without capability reporting. [2] (Huawei)

It should be noted capabilities for LTE-M are already captured in running CR for TS 36.306 [4].

There are no alternative proposals related to capabilities for NB-IoT. First three above reflect the information in UE feature list from RAN1 and not many controversies are expected, the last one has not been discussed yet in RAN2, thus:

For NB-IoT, support of multiTB-UL-r16 and multiTB-DL-r16 is conditional on support of two HARQ processes.

For NB-IoT, introduce separate capabilities multiTB-UL-Interleaving-r16, and multiTB-DL-Interleaving-r16 conditional to support of multiTB-UL-r16 and multiTB-DL-r16 respectively.

For NB-IoT, introduce a new capability multiTB-HARQ-ACK-Bundling-r16, conditional to support of multiTB-DL-Interleaving-r16.

Discuss whether multiple TBs scheduling in multicast is optional without capability reporting.

## Other proposals

Ericsson has provided the following proposal:

For multi-TB scheduling in LTE-M, the drx-InactivityTimer value is dynamically adjusted based on the number of TBs being scheduled by the DCI.[3] (Ericsson)

- This topic has been discussed online in previous meetings, and is likely to require more discussion:

Postpone discussion or discuss over email whether drx-InactivityTimer for LTE-M needs changes when scheduling multiple TBs.

# Summary and grouping of proposals

Based on the discussion in the previous sections we propose the following:

Proposal 1	For LTE-M the length of HARQ RTT timer is set to 7+k*N for bundled HARQ ACK, where k is equal to the number of HARQ ACK bundles.

Proposal 2	Remove Editor's note from RAN2 agreements document and clarify which agreements related to HARQ RTT timer length are for non-interleaved NB-IoT case.

Proposal 3	For unicast, multiple TBs scheduling is enabled separately for uplink and downlink for both LTE-M and NB-IoT.

Proposal 4	Discuss over email the details of unicast configuration for scheduling multiple TBs for 1) NB-IoT and 2) LTE-M.

Proposal 5	Explicit enabling indication for multiple TBs scheduling in SC-PTM is not needed.

Proposal 6	Discuss over email whether scheduling gap configuration is in SC-MTCH or in SIB20(-NB) for 1) NB-IoT and 2) LTE-M with details of configuration to be captured.

Proposal 7	For NB-IoT, support of multiTB-UL-r16 and multiTB-DL-r16 is conditional on support of two HARQ processes.

Proposal 8	For NB-IoT, introduce separate capabilities multiTB-UL-Interleaving-r16, and multiTB-DL-Interleaving-r16 conditional to support of multiTB-UL-r16 and multiTB-DL-r16 respectively.

Proposal 9	For NB-IoT, introduce a new capability multiTB-HARQ-ACK-Bundling-r16, conditional to support of multiTB-DL-Interleaving-r16.

Proposal 10	Discuss whether multiple TBs scheduling in multicast is optional without capability reporting.

Proposal 11	Postpone discussion or discuss over email whether drx-InactivityTimer for LTE-M needs changes when scheduling multiple TBs.

# References

R2-2000395, HARQ RTT Timers in Rel-16 NB-IoT, BlackBerry UK Limited, RAN2#109e, February 2020

R2-2000644, Signalling aspect of multiple TBs scheduling for NB-IoT, Huawei, HiSilicon, RAN2#109e, February 2020

R2-2000977, Scheduling enhancements for LTE-M and NB-IoT, Ericsson, RAN2#109e, February 2020

R2-1916562, Introduction of additional enhancements for eMTC (running CR for TS 36.306 for eMTC), Huawei, Reno, USA, November 2019