3GPP TSG-RAN WG2 Meeting #109 electronic                                                  R2-2002039

Elbonia, 24 Feb– 6 Mar 2020                                                                Revision R2-2001454

Source:	CATT

Title:	Discussion on MCG Failure Information Report

Agenda Item:	6.10.5

Document for:	Discussion and Decision

Introduction

Upon the initiation of the MCG fast recovery, UE will report the MCG failure information to the network which will include the measurement results to assist the network making the mobility control decision. This contribution discusses which measurement results should be included.

Discussion

In current DC/CA enhancement running 38.331 CR [1], the UE will include the NR and EUTRAN measurement results in the MCGFailureInformation if available.

The UE shall set the contents of the MCGFailureInformation message as follows: 1> include and set failureType in accordance with 5.7.y.3; 1> for each MeasObjectNR configured by a measConfig associated with the MCG, and for which measurement results are available: 2> include an entry in measResultFreqList; 2> if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to ssb: 3> set ssbFrequency in measResultFreqList to the value indicated by ssbFrequency as included in the MeasObjectNR; 2> if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to csi-rs: 3> set refFreqCSI-RS in measResultFreqList to the value indicated by refFreqCSI-RS as included in the associated measurement object; 2> if a serving cell is associated with the MeasObjectNR: 3> set measResultServingCell in measResultFreqList to include the available quantities of the concerned cell and in accordance with the performance requirements in TS 38.133 [14]; 2> set the measResultNeighCellList in measResultFreqList to include the best measured cells, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows; 3> ordering the cells with sorting as follows: 4> based on SS/PBCH block if SS/PBCH block measurement results are available and otherwise based on CSI-RS; 4> using RSRP if RSRP measurement results are available, otherwise using RSRQ if RSRQ measurement results are available, otherwise using SINR; 3> for each neighbour cell included: 4> include the optional fields that are available. 1> for each EUTRA frequency the UE is configured to measure by measConfig for which measurement results are available: 2> set the measResultFreqListEUTRA to include the best measured cells, ordered such that the best cell is listed first using RSRP to order if RSRP measurement results are available for cells on this frequency, otherwise using RSRQ to order if RSRQ measurement results are available for cells on this frequency, otherwise using SINR to order, and based on measurements collected up to the moment the UE detected the failure, and for each cell that is included, include the optional fields that are available; 1> if the UE is in NR-DC: 2> include and set measResultSCG in accordance with 5.7.3.4; 1> if the UE is in NE-DC: 2> include and set measResultSCG-EUTRA in accordance with TS 36.331 [10] clause 5.6.13.5;

It was agreed if the UE report MCG failure information, the network may response with RRC reconfiguration with sync or RRC release message. Therefore, the intra-RAT measurement results (NR) are useful for network. Considering the NR-DC/NE-DC change to eNB is supported, and the EUTRAN measurement results also can be included in the MCG failure information, it seems the network can also send the MobilityFromNRCommand as response to the MCGFailureInformation to move the UE from NR-DC/NE-DC to eNB in order to keep the UE connection. The similar problem also exists in EUTRAN, in current 36.331 running CR, the UE also report the NR measurements associated with the measConfig configured by MN and the (NG)EN-DC changed to gNB is also supported, it seems the network can also send the MobilityFromEUTRACommand as response to the MCGFailureInformation to move the UE from (NG)EN-DC to gNB. RAN 2 should discuss whether the inter-RAT handover can be supported for MCG fast recovery.

Proposal 1: RAN2 should discuss whether inter-RAT handover can be supported for MCG fast recovery.

If Proposal 1 is not agreeable, i.e. the inter-RAT handover is not supported for MCG fast recovery, the EUTRAN measurement result is useless for PCell change for NR MN.  However, EUTRAN measurement can still be used for PSCell change in NE-DC. Note that change of PCell with PSCell change is supported in current MR-DC handover scenarios. But for NR-DC in current 37.340, specification doesn’t support handover from NR-DC to NE-DC. This means that the EUTRAN measurement cannot be used for PCell change (as inter-RAT HO is currently not supported for MCG fast recovery) or PSCell change ( as  handover from NR-DC to NE-DC is not supported) in NR-DC. Therefore, EUTRAN measurement provided in MCGFailureInformation cannot be used.

Table B-1: Supported MR-DC handover scenarios.

| To (column) HO from (row) | E-UTRA with EPC | E-UTRA with 5GC | NR | GERAN or UTRAN | EN-DC | NGEN-DC | NE-DC | NR-DC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA with EPC | YES | YES | YES | YES | YES | NO | NO | NO |
| E-UTRA with 5GC | YES | YES | YES | NO | NO | YES | NO | NO |
| NR | YES | YES | YES | NO | NO | NO | YES | YES |
| GERAN or UTRAN | YES | NO | NO | YES | NO | NO | NO | NO |
| EN-DC | YES | YES | YES | YES | YES | NO | NO | NO |
| NGEN-DC | YES | YES | YES | NO | NO | YES | NO | NO |
| NE-DC | YES | YES | YES | NO | NO | NO | YES | NO |
| NR-DC | YES | YES | YES | NO | NO | NO | NO | YES |

If proposal 1 is not agreeable, for NR-DC the UE doesn’t need to include the EUTRAN measurement results in the MCGFailureInformation message for NR-DC as this will increase the overhead without any usefulness to network.

In 36.331, as only the EN-DC is supported, so the problem is not existed in 36.331.

Proposal 2: If proposal 1 is not agreeable, i.e. if the inter-RAT handover is not supported for MCG fast recovery, the UE shouldn’t include the EUTRAN measurement results in the MCGFailureInformation if NR-DC is configured.

Conclusion

In this contribution, we discuss the further configuration of CHO execution condition, and propose:

Proposal 1: RAN2 should discuss whether inter-RAT handover can be supported for MCG fast recovery.

Proposal 2: If proposal 1 is not agreeable, i.e. if the inter-RAT handover is not supported for MCG fast recovery, the UE shouldn’t include the EUTRAN measurement results in the MCGFailureInformation if NR-DC is configured.

Reference

- RAN2#108 email discussion # 33, running CR, R2-191xxxx - NR_DCCA Running CR for 38331 for CA_DC enh_after_merge;