3GPP TSG-RAN WG2 #109-e	R2-2002058

Online, February 24th– March 6th 2020

Agenda Item:	6.1.5.3

Source:	Ericsson

Title:	Summary of 6.1.5.3: SI Broadcast, cell Restrictions/Reservation and Barring, Initial Access, and Connection Setup

Document for:	Discussion, Decision

# 1	Introduction

This document summarizes the tdocs for the topic of system information broadcast, cell restrictions/reservation and barring, initial access, and connection setup, which are available in the agenda item 6.1.5.3 of RAN2#109-e. Note that the summary excludes tdocs (available in the agenda item 6.1.5.3) on other topics, such as RLF and F1AP related aspects.

The rapporteur provided summaries and ways forward only for those topics where several companies have contributed, while other topics are listed in section 2.3.

# 2	Discussion

## 2.1	Cell restrictions, reservations, and barring

This section summary has considered the documents [1-11].

In RAN2#108, the following conclusion and agreement was reached:

Both support of IAB node(s) and the cell status for IAB node(s) is combined in a single IE, i.e. if the IE is present, the cell supports IABs and the cell is also considered as a candidate for IABs; if the IE is absent, the cell does not support IAB and/or the cell is barred for IAB.

The case that UEs are barred but IAB nodes are allowed to access shall be supported.

FFS if this is supported by MIB: CellBarred (i.e. IAB MT ignores the MIB cellBarred when set) or SIB1: CellReservations (i.e. IAB MT ignores SIB cell reservations, or has an access identity that allow access)

In Rel-15, cell reservations are based on 3 different IEs which can provide different cell restriction options depending on how they are used and combined.

From the company’s proposals, few companies proposed the possibilities of using/combining all the 3 IEs with the new IE “iab-support” to support differentiation between UEs and IABs. Most of the company’s focus on how to restrict UEs and/or IABs using the “cellBarred” IE in MIB and “iab-Support” IE in SIB1.

The table below gives an overview of the proposals available in [1-11].

|  | Does IAB-MT follow… |  |  |
| --- | --- | --- | --- |
|  | cellBarred | cellReservedForOtherUse | cellReservedForOperatorUse |
| QC | No | N/A | N/A |
| ZTE, Sanechips | No | N/A | N/A |
| Huawei | No | N/A | N/A |
| Ericsson | Yes | No | No |
| Sony | Yes | N/A | Yes |
| CATT | No | N/A | N/A |
| Nokia | No | N/A | N/A |
| LG-E | Yes | No | N/A |
| Futurewei | No | N/A | N/A |
| Vivo | No | Yes | No |
| Samsung | No | N/A | N/A |

“N/A” means that no information is available in the company contribution.

In general, all these 4 IEs (cellBarred, cellReservedForOtherUse, cellReservedForOperatorUse, iab-Support) should be studied and discussed together. Given that this is not the case, the diversity of the proposals, and that there are not many proposals about the IEs “cellReservedForOtherUse” and “cellReservedForOperatorUse”, we suggest two different way forward.

Way forward 1:

IAB-MTs ignore the IEs cellBarred, cellReservedForOtherUse, and cellReservedForOperatorUse.

Way forward 2:

Further discuss all the possible/allowable combinations for the IEs (cellBarred, cellReservedForOtherUse, cellReservedForOperatorUse, and iab-Support), and come with an agreeable proposal for how to differentiate between UEs and IABs.

## 2.2	Unified Access Control

Two contributions [6,7] discuss unified access control (UAC). This topic was also discussed in the email discussion [107bis#40] and a summary of the discussion was reported in [12]. The proposal suggested by the rapporteur was not agreed upon.

Since this is still an open issue, the rapporteur suggests the following two ways forward:

Way forward 1 (as suggested in [12]):

IAB-MTs are under UAC.

Way forward 2:

Discuss UAC but focus on the merits of UAC for IABs.

## 2.3	Other areas

The topics listed in this section are raised by only one company, and since there is not enough input, no summary or proposal is provided.

- PWS information [13].

Topics in “2.3 other areas” are not discussed in RAN2#109-e.

# 4	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	IAB-MTs ignore the IEs cellBarred, cellReservedForOtherUse, and cellReservedForOperatorUse.

Proposal 2	Further discuss all the possible/allowable combinations for the IEs (cellBarred, cellReservedForOtherUse, cellReservedForOperatorUse, and iab-Support), and come with an agreeable proposal for how to differentiate between UEs and IABs.

Proposal 3	IAB-MTs are under UAC.

Proposal 4	Discuss UAC but focus on the merits of UAC for IABs.

Proposal 5	Topics in “2.3 other areas” are not discussed in RAN2#109-e.

# 5	References

R2-2000484, IAB access barring. Qualcomm Incorporated

R2-2000512, Consideration on IAB node access control.	ZTE, Sanechips

R2-2000523, Leftover issue for cell barring. Huawei, HiSilicon

R2-2000751, On cell Reservations in MIB and SIB1. Ericsson

R2-2000835, IAB Cell Barring. Sony

R2-2000892, Views on Cell Barring Mechanism for IAB. CATT

R2-2001058, Remaining aspects of IAB initial access. Nokia, Nokia Shanghai Bell

R2-2001523, Access control in IAB networks	. LG Electronics France

R2-2001605, Differential barring for IAB nodes and UEs. Futurewei Technologies

R2-2000277, Cell baring indication for IAB support. Vivo

R2-2001016, Remaining details for IAB-MT access. Samsung

R2-1915478	Summary for Email Discussion [107bis#40] [IAB] SI Broadast, barring, Initial access, Connection setup (Ericsson).

R2-2000824, PWS information handling in IAB. Sony