3GPP TSG RAN WG2#109-e	R2-2000033

Electronic meeting, 24th February - 6th March, 2020

3GPP TSG-RAN WG3 Meeting #106	R3-197794

Reno (NV), USA, 18 – 22 November, 2019

Title:	LS on DL-AOD procedure

Response to:

Release:	Release 16

Work Item:	NR_POS-Core

Source:	RAN3

To:	RAN2

Cc:

Contact Person:

#### Name:	Philippe Reininger

E-mail Address:	philippe.reininger@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

In the DL-AoD positioning technique, the UE position is estimated based on multiple Angles of Departure (AoD) of DL PRS from RAN node to UE, along with knowledge of the geographical coordinates of the measured RAN nodes. The UE measurements in DL-AoD should be combined with the RAN node information in order to derive AOD the UE location at LMF.

From architecture perspective; there are two options for the measurements reporting and AOD calculation:

Option 1: LMF calculates the AOD based on the UE measurements and RAN node assistance information. LMF then further calculates UE location based on the AODs.

Option 2: The RAN nodes calculate the AoD with the RSRP information based on gNB configuration and send the AoD information back to LMF, who then calculates the UE position.

RAN3 has discussed and prefers to support option 1 for DL-AOD positioning method in the stage 2 specifications.

2. Actions:

To RAN2: 	RAN3 kindly requests RAN2 to take the above answers into account in their work.

3. Dates of Next TSG-RAN WG3 Meetings:

3GPP TSG RAN WG3#107	24 – 28 February, 2020	Athens, Greece

3GPP TSG RAN WG3#108	20 – 24 April, 2020	TBD, Japan