3GPP TSG RAN WG2#109-e	R2-2000097

Electronic meeting, 24th February - 6th March, 2020

3GPP TSG-RAN WG1 Meeting #99	R1-1913464

Reno, USA, 18th – 22nd November 2019

Title:	Reply LS on SL RLM/RLF

Response to:

Release:	Release 16

Work Item:	5G_V2X_NRSL

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

#### Name:	Moon-il Lee

E-mail Address:	moonil.lee@interdigital.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN1 would like to thank RAN2 for the LS related to sidelink RLM/RLF (R2-1908466). RAN1 has made further agreements in the context of sidelink RLM/RLF as following:

Agreements: For sidelink RLM/RLF at Tx UE, the usage of HARQ feedback status is feasible from RAN1 perspective when sidelink HARQ is enabled - There is no IS/OOS indication to upper layer from physical layer for sidelink RLM at Tx UE - It is RAN1 understanding that HARQ feedback status (i.e., ACK, NACK) is available in upper layer without additional RLM indication from physical layer - DTX is reported to upper layer for sidelink RLM if RAN2 agree to use it - This doesn’t require RAN1 specification impact

2. Actions:

To RAN2:

ACTION: RAN1 respectfully asks RAN2 to take the above agreements into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting#100	24 – 28 February 2020	Athens, Greece

TSG RAN WG1 Meeting#100bis	20 – 24 April 2020	Busan, South Korea