3GPP TSG RAN WG2#109-e	R2-2002136

Electronic meeting, 24th February - 6th March, 2020

# Liaison Statement

| Liaison Statement Title: | Mandatory User Plane Integrity for 5G |
| --- | --- |
| Security Classification: | Non-confidential |

| Source Meeting Information |  |  |
| --- | --- | --- |
| Meeting Number | Meeting Date | Meeting Location |
| FSAG#79 | 23 Mar 2020 | Conference Call |

| Document Details |  |  |
| --- | --- | --- |
| Document Number: | Creation Date: | Document Author: |
| FSAG Doc 79_002 | 21st Feb 2020 | James Skuse, GSMA |
| Originating GSMA Source: | Deadline for response: | Liaison Statement Contact |
| FSAG | None | GSMALiaisons@gsma.com |

| Action Required by Recipient |  |
| --- | --- |
| External Recipients: | 3GPP SA, 3GPP RAN, 3GPP CT, 3GPP RAN2, 3GPP SA3, 3GPP CT1, 3GPP SA2 |
| Internal Recipients: |  |

# Background

GSMA has been made aware through its “Coordinated Vulnerability Disclosure Programme” about new research on layer-2 attack scenarios against LTE. The research has been published at [1] and will be presented on the security conference NDSS [2] end of February 2020.

# Item for Consideration

## Abstract from the Research Paper

The key findings of the research are copied from the research paper's abstract below:

"Long Term Evolution (LTE/4G) establishes mutual authentication with a provably secure Authentication and Key Agreement (AKA) protocol on layer three of the network stack. Permanent integrity protection of the control plane safeguards the traffic against manipulations. However, missing integrity protection of the user plane still allows an adversary to manipulate and redirect IP packets, as recently demonstrated.

In this work, we introduce a novel cross-layer attack that exploits the existing vulnerability on layer two and extends it with an attack mechanism on layer three. […] allows an active attacker to impersonate a user towards the network and vice versa; we name these attacks IMP4GT (IMPersonation attacks in 4G neTworks). In contrast to a simple redirection attack as demonstrated in prior work, our attack dramatically extends the possible attack scenarios and thus emphasizes the need for user-plane integrity protection in mobile communication standards."

## Observations from GSMA

It is essential for security of 5G cellular networks that the 3GPP standards mandate support of user plane integrity protection at the full data rate of a UE.

The new IMP4GT attacks exploit the same underlying issue as the earlier work described in [3], which was reported to 3GPP in 2018 [4]. GSMA gave 3GPP advance notice of the IMP4GT research in May 2019 [7].

The researchers executed the IMP4GT attacks in an LTE environment, but indicate that the same attack would in principle also work against 5G (UEs using NR connected to the 5GC) unless User Plane integrity protection is in place.

It is GSMA's understanding of the current work in 3GPP SA3 that a 5G “standalone” SA network with NR radio connected to a 5G Core Network would support User Plane integrity protection, and thus could be immune against IMP4GT and similar attacks. The only aspect that remains unclear is the current implementation status of UEs, because [5] and [6] only mandate that UEs support a data rate of 64 kbps with integrity protection, while user plane integrity protection with full data rate is an optional UE feature.

GSMA can only confirm the researcher's statement in the abstract:

"We emphasize the requirement for a mandatory and full-rate integrity protection for all 5G data connections to prevent IMP4GT"

Even though the new attacks may have limited impact in real-world deployments, there is the need to strengthen the security of 5G networks and UEs for the future. It is essential to not create a large legacy of 5G networks and UEs that are vulnerable to such attacks. At this point in time no wide scale commercial penetration of « standalone » SA supporting terminals is in the market, hence it is now the right point in time to mandate this UE functionality.

# Action

GSMA politely requests SA/RAN/CT to ask the affected working groups to agree the necessary CRs in the appropriate release to have mandatory support of full-rate user plane integrity protection for all UEs supporting NR connected to the 5GC.

## Deadline

GSMA would appreciate a response following the plenary meetings in March 2020.

# Contacts

In case of any further questions and/or feedback to this Liaison Statement, please contact James Moran [jmoran@gsma.com] or James Skuse [jskuse@gsma.com].

# References

[1]	https://imp4gt-attacks.net/

[2]	https://www.ndss-symposium.org/ndss2020/accepted-papers/

[3]	https://alter-attack.net/

[4]	R2-1806381/S3-181429  LS from GSMA CVDGT to SA3, RAN2, cc RAN

[5]	3GPP TS 38.300 clause 13.1

[6]	3GPP TS 24.501 clause 9.11.4.7 Integrity protection maximum data rate

[7]	R2-1908606/S3-192266  LS from GSMA CVDGT to SA3, RAN2, cc RAN